3GPP TSG-RAN2 Meeting #129	R2-2501096

Athens, Greece, 17 - 21 February 2025

Agenda Item:	8.0 General

Source: 	Ericsson

Title:  	Time Synchronization for MBS

Document for:	Discussion and Decision

# Introduction

The time synchronization for MBS is discussed further in this contribution.

# Background

When SIB19 was introduced in LTE then UTC time for MBMS was mentioned as one of the use cases:

Broadcasting “reference time” from LTE network is useful for various use cases, e.g.

- GPS time for GNSS

- Coordinated Universal Time (UTC) for MBMS

- Local time provisioning

In LTE it is optional for the UE, without UE capability signalling, to support the timeInfoUTC in SIB16. The timeInfoUTC was introduced in Rel-11 and it provides a counter of the UTC seconds in 10 millisecond granularity since the start of the Gregorian calendar.

The timeReferenceInfo in SIB16 was introduced in Rel-15 for Ultra Reliable Low Latency Communication to support a granularity up to 0.25 us. This feature is also optional, but requires UE capability signalling, i.e. via unicast signalling the UE receives updates on the reference time and uncertainty:

SystemInformationBlockType16-r11 ::=		SEQUENCE {

timeInfo-r11							SEQUENCE {

timeInfoUTC-r11						INTEGER (0..549755813887),

dayLightSavingTime-r11				BIT STRING (SIZE (2))		OPTIONAL,	-- Need OR

leapSeconds-r11						INTEGER (-127..128)			OPTIONAL,	-- Need OR

localTimeOffset-r11					INTEGER (-63..64)			OPTIONAL	-- Need OR

}																	OPTIONAL,	-- Need OR

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

...,

[[	timeReferenceInfo-r15				TimeReferenceInfo-r15	OPTIONAL	-- Need OR

]]

}

TimeReferenceInfo-r15 ::=		SEQUENCE {

time-r15							ReferenceTime-r15,

uncertainty-r15						INTEGER (0..12)				OPTIONAL,	-- Need OR

timeInfoType-r15					ENUMERATED {localClock}		OPTIONAL,	-- Need OR

referenceSFN-r15					INTEGER (0..1023)			OPTIONAL	-- Cond TimeRef

}

ReferenceTime-r15 ::=			SEQUENCE {

refDays-r15							INTEGER (0..72999),

refSeconds-r15						INTEGER (0..86399),

refMilliSeconds-r15					INTEGER (0..999),

refQuarterMicroSeconds-r15			INTEGER (0..3999)

}

timeInfoUTC Coordinated Universal Time corresponding to the SFN boundary at or immediately after the ending boundary of the SI-window in which SystemInformationBlockType16 is transmitted. In an NTN cell, the indicated time is referenced at the uplink time synchronization reference point (RP), i.e., UE should take into account the propagation delay between UE and RP when determining the UTC time at the UE. The field counts the number of UTC seconds in 10 ms units since 00:00:00 on Gregorian calendar date 1 January, 1900 (midnight between Sunday, December 31, 1899 and Monday, January 1, 1900). NOTE 1. This field is excluded when estimating changes in system information, i.e. changes of timeInfoUTC should neither result in system information change notifications nor in a modification of systemInfoValueTag in SIB1.

Upon receiving SystemInformationBlockType16 with timeReferenceInfo, the UE may perform the related actions as specified in clause 5.6.1.3.

5.6.1.3	Reception of the DLInformationTransfer by the UE

Upon receiving DLInformationTransfer message, the UE shall:

…

1>	if timeReferenceInfo is included:

2>	calculate the time reference based on the included time, timeInfoType and referenceSFN in timeReferenceInfo;

2>	calculate the inaccuracy of the time reference based on the uncertainty and other implementation-related inaccuracies, if uncertainty is included in timeReferenceInfo;

2>	inform upper layers of the time reference and, if uncertainty is included in timeReferenceInfo, of the inaccuracy of the time reference.

In NR, similar as in LTE, it is optional for the UE to support timeInfoUTC in SIB9 providing a granularity of 10 millisecond, without UE capability signalling. For Time Sensitive Communications (TSC) services the UE may indicate to support referenceTimeProvision-r16 in UE capabilities for 10 nanoseconds granularity.

# Discussion

In LTE the timing requirements for MBMS are captured in section 4.6 in TS 26.346 (SA4):

A number of MBMS metadata fragments and File Delivery Table (FDT) contain NTP encoded time values. NTP uses UTC as reference time and is independent from time zones. In order to process the time information from the BM-SC correctly, the MBMS UEs shall be time synchronized with the BM-SC with a tolerance of +/- 1 second. The BM-SC shall offer an SNTP [84] time server. The MBMS UEs should use SNTP to synchronize the time with the BM-SC. It is expected that the MBMS UE periodically requests SNTP time synchronization in order to keep the +/- 1 second tolerance. However, the MBMS UE should use the SNTP time synchronization service only as necessary to keep +/- 1 second accuracy, and should at most use the SNTP time synchronization once every 24 hours to avoid scalability issues.

For MBMS in LTE the following requirements apply:

- The MBMS UE has to be time synchronized with a tolerance of +\- 1 second e.g. to interpret the start and stop times in UTC time provided for a session in the USD.

- The network provides an SNTP server to enable this synchronization.

For MBS in NR these requirements are missing in TS 26.517 (SA4):

In both cases the time is specified as the absolute date and UTC time. The duration may be determined by subtracting the start time from the stop time.

SA4 asked SA2 and RAN2 which network entity could provide the MBS time synchronization [1]. SA4 mentions that the MBSF and MBSTF are possible entities See Multicast/Broadcast Service (Transport) Function (MBSF, MBSTF) in TS 26.502 (SA4).

SA2 already discussed the SA4 LS and replied that SIB9 and/or SNTP server could be used to provide UTC time information to the UE [2]:

In NR, SIB9 contains information related to GPS time and Coordinated Universal Time (UTC), as specified in TS 38.331 e.g. the timeInfo parameter that provides 10ms accuracy. The UE may get time information from SIB9 to synchronize the UE clock, so that the UE can be time synchronized to a UTC time source. To apply to MBS for the purpose indicated by SA4 in the incoming LS, it requires MBSF and MBSTF are time synchronized to the same UTC time source (or wall clock time) as NG-RAN nodes.

However, as SIB9 is optional, SA2 suggests also deploying for example an SNTP server in service layer, to fulfil the time synchronization requirement of keeping +/- 1 second accuracy between UE and the network. SA2 understands the development of such solution is under the remit of SA4. It is also up to SA4 to determine whether to integrate related functionality in the MBSF, MBSTF, or another SA4 defined network function.

The MBSF/MBSTF entities need to be time synchronized to the same UTC time source. SA2 indicates that it is up to SA4 decide which network entities to integrate this functionality.

From RAN2 perspective the following observations can be made:

SNTP is supported in the higher layers and transparent to RAN, i.e. outside RAN2 scope.

The use of SNTP requires unicast signalling and the UE should at most use the SNTP time synchronization once every 24 hours to avoid scalability issues.

It is optional for the UE to support the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) without UE capability signalling. This feature does not have scalability constraints.

It is optional for the UE to support reference time with granularity up to 10 ns in SIB9 and DLInformationTransfer message (i.e. referenceTimeInfo-r16 IE) and referenceTimeInfoPreference-r16 signalling in UAI for TSC services with UE capability signalling (referenceTimeProvision-r16). This feature has scalability constraints.

Based on the observations above, it is proposed to reply to SA4 and SA2:

The use of SNTP for time synchronization in MBS is outside RAN2 scope.

RAN2 confirms that the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) can be used for time synchronization in MBS. It is optional for the UE to support timeInfoUTC in SIB9 without UE capability signalling.

The use of referenceTimeInfoPreference in SIB9 for time synchronization is feasible, but this feature provides higher granularity than required and is more complex to support compared to the standard UTC time in SIB9. Furthermore this feature has scalability constraints:

The use of SNTP for time synchronization in MBS is outside RAN2 scope

RAN2 confirms that the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) can be used for time synchronization in MBS. It is optional for the UE to support timeInfoUTC in SIB9 without UE capability signalling.

A draft LS for discussion and agreement is provided in the Annex.

# Summary

RAN2 is kindly asked to discuss the reply LS to SA4 about time synchronization with MBS:

Observation 1	SNTP is supported in the higher layers and transparent to RAN, i.e. outside RAN2 scope.

Observation 2	The use of SNTP requires unicast signalling and the UE should at most use the SNTP time synchronization once every 24 hours to avoid scalability issues.

Observation 3	It is optional for the UE to support the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) without UE capability signalling. This feature does not have scalability constraints.

Observation 4	It is optional for the UE to support reference time with granularity up to 10 ns in SIB9 and DLInformationTransfer message (i.e. referenceTimeInfo-r16 IE) and referenceTimeInfoPreference-r16 signalling in UAI for TSC services with UE capability signalling (referenceTimeProvision-r16). This feature has scalability constraints.

Proposal 1	The use of SNTP for time synchronization in MBS is outside RAN2 scope

Proposal 2	RAN2 confirms that the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) can be used for time synchronization in MBS. It is optional for the UE to support timeInfoUTC in SIB9 without UE capability signalling.

# References

- S4-242169 LS on Time Synchronization for MBS, FS_AMD (Advanced Media Delivery), QC, LS out, To: SA2, RAN2, Rel-19

- S2-2501327, Reply LS on Time Synchronization for MBS, FS_AMD, Ericsson, LS out, To: SA4, RAN2

# Annex

3GPP TSG-RAN2 Meeting #129	R2-25xxxxx

Athens, Greece, 17 - 21 February 2025

Title:	DRAFT Reply LS on Time Synchronization for MBS

Response to:	LS (S4-242169/R2-25xxxxx) LS on Time Synchronization for MBS

LS (S2-2501327/R2-25xxxxx) Reply LS on Time Synchronization for MBS

Release:	Rel-19

Work Item:	FS_AMD

Source:	Ericsson [TSG RAN WG2]

To:	3GPP SA WG4, 3GPP SA WG2

Cc:

Contact Person:

#### Name:	Martin van der Zee

- E-mail Address:	martin.van.der.zee@ericsson.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

RAN2 thanks SA4 and SA2 for the (reply) LS on Time Synchronization for MBS.

RAN2 would like to provide the following feedback:

The use of SNTP for time synchronization in MBS is outside RAN2 scope.

RAN2 confirms that the UTC time with 10 ms granularity in SIB9 (i.e. timeInfo IE including timeInfoUTC) can be used for time synchronization in MBS. It is optional for the UE to support timeInfoUTC in SIB9 without UE capability signalling.

2. Actions:

To SA4, SA2

ACTION: 	RAN2 kindly asks SA4 and SA2 to take the information above into account.

3. Date of Next TSG-RAN2 Meetings:

TSG RAN2 Meeting #129-bis	7 – 11 April 2025		China

TSG RAN2 Meeting #130		19 – 23 May 2025		Malta