3GPP TSG-RAN WG2 Meeting #129		R2-2501098

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Discussion on change of si-SchedulingInfo for SIB6, SIB7 and SIB8

Agenda Item:	5.1.3.1

Document for:	Discussion and Decision

- Introduction

According to the following highlighted part in TS38.331 [1], the change of SIB6/7/8 will not result in system information change notification. However, whether the change of scheduling information for SIB6/7/8 in SIB1 will lead to system information change notification is not clear.

6.5 Short Message Short Messages can be transmitted on PDCCH using P-RNTI with or without associated Paging message using Short Message field in DCI format 1_0 (see TS 38.212 [17], clause 7.3.1.2.1). Table 6.5-1 defines Short Messages. Bit 1 is the most significant bit. Table 6.5-1: Short Messages Bit; Short Message. 1; systemInfoModification If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs.. 2; etwsAndCmasIndication If set to 1: indication of an ETWS primary notification and/or an ETWS secondary notification and/or a CMAS notification.. 3; stopPagingMonitoring This bit can be used for only operation with shared spectrum channel access and if nrofPDCCH-MonitoringOccasionPerSSB-InPO is present. If set to 1: indication that the UE may stop monitoring PDCCH occasion(s) for paging in this Paging Occasion as specified in TS 38.304 [20], clause 7.1.. 4; systemInfoModification-eDRX If set to 1: indication of a BCCH modification other than SIB6, SIB7, SIB8 and posSIBs. This indication applies only to UEs using IDLE eDRX cycle longer than the BCCH modification period.. 5 – 8; Not used in this release of the specification, and shall be ignored by UE if received..

In this contribution, we will discuss the issue related to the change of the scheduling information of SIB6/7/8.

- Discussion

In TS38.331 [1], the cases not resulting in the system information change notification have been specified, including change of hyperSFN, timeInfoUTC, movingReferenceLocation, posSI-BroadcastStatus, ephemerisInfo, epochTime, ntn-UlSyncValidityDuration, ta-Common, ta-CommonDrift, ta-CommonDriftVariant, time in ReferenceTimeInfo, si-RequestResources, si-RequestResourcesRepetitionNum2/4/8, and si-BroadcastStatus. For example, the clarification of changing of si-RequestResources is copied as below.

si-RequestResources If there is only one entry in the list, the configuration is used for all SI messages for which si-BroadcastStatus is set to notBroadcasting. Otherwise the 1st entry in the list corresponds to the first SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting, 2nd entry in the list corresponds to the second SI message in schedulingInfoList for which si-BroadcastStatus is set to notBroadcasting and so on. Change of si-RequestResources should not result in system information change notification.

The intention is to avoid frequent/unnecessary system information change notifications, and to save power consumption of UE that does not need to acquire the updated SI.

Observation 1: In TS 38.331, the cases that should not result in system information change notification are clarified.

Upon receiving a short message, the UE behavior is extracted as follows [1].

If the UE receives a Short Message, the UE shall: 1> if the UE is ETWS capable or CMAS capable, the etwsAndCmasIndication bit of Short Message is set, and the UE is provided with searchSpaceSIB1 and searchSpaceOtherSystemInformation on the active BWP or the initial BWP: 2> immediately re-acquire the SIB1; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB6: 3> acquire SIB6, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is ETWS capable and si-SchedulingInfo includes scheduling information for SIB7: 3> acquire SIB7, as specified in clause 5.2.2.3.2, immediately; 2> if the UE is CMAS capable and si-SchedulingInfo includes scheduling information for SIB8: 3> acquire SIB8, as specified in clause 5.2.2.3.2, immediately; NOTE: In case SIB1, SIB6, SIB7, or SIB8 overlap with a measurement gap it is left to UE implementation how to immediately acquire SIB1, SIB6, SIB7, or SIB8. 1> if the UE does not operate an IDLE eDRX cycle longer than the modification period and the systemInfoModification bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next modification period; 1> if the UE operates an IDLE eDRX cycle longer than the modification period and the systemInfoModification-eDRX bit of Short Message is set: 2> apply the SI acquisition procedure as defined in clause 5.2.2.3 from the start of the next eDRX acquisition period boundary.

When there is a PWS notification, the network sends the short message with etwsAndCmasIndication set to 1. Besides, if the network provides the updated scheduling information of SIB6/7/8, the systemInfoModification should also be set to 1. Based on the above procedure, the following issues may occur:

- For the UE not supporting ETWS/CMAS, it acquires MIB/SIB1/OSI from the start of the next modification period or the next eDRX acquisition period boundary due to the system information change notification. However, only the scheduling information of SIB6/7/8 in SIB1 is changed. The power of the UE not supporting ETWS/CMAS is wasted.

- For the UE supporting ETWS/CMAS, it will acquire SIB1 twice. The first time is due to the PWS notification and UE immediately acquires SIB1 upon receiving the short message in the modification period. The second time is due to the system information notification and UE acquires SIB1 in the next modification period. Considering only the scheduling information of SIB6/7/8 in SIB1 is changed, no need the second time to acquire SIB1, which also costs more power.

Observation 2: In the existing spec, the change of the scheduling information of SIB6/7/8 in SIB1 will result in system information change notification, which leads to unnecessary SIB1 acquisition.

To address the above issue, we think the change of the scheduling information of SIB6/7/8 in SIB1 should not result in the system information change notification. We also provide the corresponding CR in [2] [3] [4] [5].

Proposal 1: RAN2 to agree that the change of the scheduling information of SIB6/7/8 in SIB1 should not result in system information change notification.

- Conclusion

In this contribution, we discuss the issue related to the change of the scheduling information of SIB6/7/8. Based on the discussion, we have the following observations and proposals.

Observation 1: In TS 38.331, the cases that should not result in system information change notification are clarified.

Observation 2: In the existing spec, the change of the scheduling information of SIB6/7/8 in SIB1 will result in system information change notification, which leads to unnecessary SIB1 acquisition.

Proposal 1: RAN2 to agree that the change of the scheduling information of SIB6/7/8 in SIB1 should not result in system information change notification.

- References

- TS 38.331

- R2-2501099, Clarification on change of si-SchedulingInfo for SIB6, SIB7 and SIB8 (R15)

- R2-2501100, Clarification on change of si-SchedulingInfo for SIB6, SIB7 and SIB8 (R16)

- R2-2501101, Clarification on change of si-SchedulingInfo for SIB6, SIB7 and SIB8 (R17)

- R2-2501102, Clarification on change of si-SchedulingInfo for SIB6, SIB7 and SIB8 (R18)