3GPP TSG-RAN WG2 Meeting #129	R2-2501107

Athens, Greece, Feb. 17th – 21st, 2025

Title: 	Discussion on leftovers from Rel-18

Source: 	Huawei, HiSilicon

Agenda item:	8.10.5.1

Document for:	Discussion/Decision

- Introduction

Rel-19 WID includes the following leftover topics from Rel-18 to be further discussed in RAN2:

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

- RACH optimization for SDT

- MHI Enhancement for SCG Deactivation/Activation

- MRO for MR-DC SCG failure

In this paper, we mainly analyse possible RAN2 impacts for SDT related RACH optimization and MHI enhancement for SCG Deactivation/Activation.

- Discussion

Discussion on RA report solution for SDT

RAN2#128 made the following agreements:

Agreements - The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report. - The following failure causes can be logged for failed SDT in RA report: ⁻ upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing; ⁻ upon random access problem indication is received from MCG MAC while SDT procedure is ongoing; ⁻ upon T319a expires; ⁻ upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing; ⁻ FFS: unsuccessfully completed upon cell re-selection.

Per current specification, The UE terminates the SDT transmission and goes to RRC Idle when cell re-selection occurs during the on-going SDT transmission. Even though the SDT transmission terminates, we do not consider it as the SDT failure case. Because the radio link quality deteriorates due to UE mobility and the failure risk of SDT transmission increases dramatically, it is natural to prioritize the cell re-selection process over any other process. There is nothing for network to improve in the cell re-selection case. Therefore, there is no need to log the cell re-selection as failure cause.

Proposal 1: RAN2 do not include the cell-reselection as failure cause for SDT transmission.

Furthermore, since RAN2 does not rule out the case when UE fall backs from the CG-SDT to RA-SDT, we analyse the necessity of other SDT related information as below:

- CG-SDT related information if the UE is configured with CG-SDT but fails to initiate the CG-SDT

- indication regarding that the UE failed to initiate RA-SDT or CG-SDT

When UL data arrives in the UE in RRC inactive, the UE could first check whether the CG-SDT is configured and thus the conditions for CG-SDT are fulfilled, and then check if the RA-SDT is fulfilled if CG-SDT is not configured or the conditions for CG-SDT are not fulfilled. In our point of view, the network may need to know why CG-SDT is not initiated when the CG is reserved and configured for the UE. The UE also compares the DL RSRP and buffered data volume to CG-SDT condition parameter to derive the fulfilment status. Regarding the fallback case from CG-SDT to RA-SDT, it may help the network to optimize the CG-SDT related parameters and improve the utilization probability of CG resources. Since the DL RSRP and buffered data are already agreed to be reported by UE and the UE-specific CG-SDT config is known from the UE context, we understand that an indication of whether CG-SDT is configured in RA report, when RA-SDT is initiated, enables the network to know that it fails to initiate CG-SDT.

Proposal 2: The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

Discussion on MHI enhancement for SCG deactivation/activation

RAN3 LS [1] gives the feedback that the network-based solution is supported. Compared to the network-based solution, the UE-based solution has the advantage to log the SCG deactivation/activation state in a more reliable manner, when UE RRC state transitions from RRC connected to RRC Idle and back to RRC connected. Because the network-based solution lost all the PSCell information in UE’s RRC Idle state and could not link this UE’s mobility information in previous RRC connected state to the UE which comes back to RRC connected again. It is an interruption of UE mobility information logging. Therefore, we propose:

Proposal 3: RAN2 support the UE-based solution for MHI enhancement for SCG deactivation/activation.

- Conclusion

Proposal 1: RAN2 do not include the cell-reselection as failure cause for SDT transmission.

Proposal 2: The UE indicates in the RA-Report if the CG-SDT initiation was evaluated.

Proposal 3: RAN2 support the UE-based solution for MHI enhancement for SCG deactivation/activation.

- Reference

[1]	R3-247906, Reply LS on MHI enhancement solution for SCG deactivation/activation, RAN3