3GPP TSG-RAN WG2 Meeting #129   								   R2-2501109

Athens, Greece, Feb. 17th - 21st, 2025

Agenda Item:	8.1.2.3

Source:	Huawei, HiSilicon

Title:	Discussion on LCM for positioning use case

Document for:	Discussion and decision

Introduction

At the RAN#102 meeting, a new R19 WI on AI for air interface [1] was approved and further revised in the RAN#106 meeting [2]. One objective is specification of support for positioning accuracy enhancements. At RAN2#128 meeting, RAN2 agreed on the basic procedure for LCM for case 1, and some FFSes are left for further discussion.

1 For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS 2 As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

In this paper, we mainly discuss possible RAN2 impacts for UE-sided and NW-sided model for positioning. In addition, two incoming LSes [5][6] will be treated at RAN2#129. For [6], no RAN2 actions are observed, and for [5], there are some actions for RAN2. So this paper is also to address the LS [5].

Discussion

UE-sided model for positioning

Based on the latest RAN1 and RAN2 progress (shown in section 5), we are to discuss possible RAN2 impacts further for Case 1, such as UE capability reporting, functionality management, monitoring, positioning methods.

Discussion on UE functionality reporting for positioning

In the last meeting, RAN2 agreed that when the applicability of the functionality changes from applicable to non-applicable or vice versa, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Currently, whether/when unsolicited LPP ProvideCapabilities message is sent to the NW is up to UE implementation. However, the applicable functionality change information is important for the NW. The network should not rely on location estimates provided by the UE using a non-applicable functionality and that is why RAN2 previously agreed that in such case an error cause should be sent by the:

As a baseline, If the AIML based positioning method becomes non-applicable when LMF requests UE location estimation, UE cannot perform the AIML based positioning, and reply with LPP Providelocationinformation message with error cause. FFS if other fallback options are considered

Such an error situation should be avoided as much as possible, that is why as soon as the functionality becomes non-applicable, the UE shall report this to the network as soon as possible.

Proposal 1: The UE should send an updated LPP ProvideCapabilities  message to the NW as soon as possible when the applicability status of the functionality changes.

On the FFS if fallback is needed, RAN2 agreed that when the applicability changes, UE sends an unsolicited LPP ProvideCapabilities message to LMF. Based on this the UE should be reconfigured and LMF will normally not ask the UE for a location until this reconfiguration is done. With that in mind, the issue seems a corner case, so replying with an error cause seems sufficient.

Furthermore, we should follow the principles agreed for BM use case as a baseline and for BM we agreed:

When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Similarly, for positioning, the functionality management should be under NW control, so if the NW has received the error cause from the UE, it is up to NW decision, e.g. the NW can decide to disable AI/ML based positioning and instruct the UE to use legacy positioning method.

Proposal 2: There is no need for any fallback option (i.e. the NW can reconfigure the UE after an updated LPP ProvideCapabilities message or a failure message is received).

Discussion on monitoring

For the UE-sided model positioning, in the RAN1 116b meeting, the following agreements have been achieved.

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF.

We can observe that for the UE-sided model positioning case, the monitoring related information exchange is between UE and LMF. Meanwhile, the functionality management triggers and commands are also between UE and LMF. The UE and LMF communicate via LPP signaling. The main procedures should include the following.

- Report configurations from LMF to UE. According to the RAN1 116b meeting, in option A, it is the UE to calculate the monitoring metrics while in option B, it is the LMF to perform the calculation based on UE reported inference result. Whichever option is chosen, the LMF should configure the UE about how to acquire calculation assistance data (option A) or what inference results to report (option B).

- Report from UE to LMF. Based on the configuration from LMF, the UE reports to LMF via LPP signaling. The UE may be configured to report with trigger conditions or periodic. After receiving reports from UE, the LMF may further send another configuration to the UE, to modify the previous configuration or cancel the UE reporting.

For now, we see that RAN1 is still discussing options and they may do down-selection of some options. So we think RAN2 can wait for more RAN1 progress on monitoring.

Proposal 3: For positioning Case 1, RAN1 will discuss details of options and they may do down-selection. So RAN2 can wait for more RAN1 progress on monitoring.

Discussion on functionality management

At RAN2#127bis meeting, the following agreement was made, and there was an FFS.

RAN2#127bis Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress).

In the post-meeting email discussion [4], the following summary proposal was made.

Proposal 5: UE receives the needed assistance data for calculating UE location for AI/ML in step3 (ProvideAssistanceData) and UE receives the instruction to perform the interference in step 5 (RequestLocationInformation). The content of Assistance Data and the content of request location information is based upon RAN1 parameter list.

In our understanding, in step 3 the UE can have all the information to perform inference, and it needs more RAN1 progress on the content. In step 5, LMF may do functionality management to the UE, e.g. UE may be requested to provide its location information or fall back to legacy positioning method.

For other messages (e.g. LPP provide assistance data message), we do not see a motivation to use them to do functionality management.

Proposal 4: It is assumed that the inference configuration is provided in Step 3, and it needs more RAN1 progress on the content.

Proposal 5: Only the LPP request location information message can be used to do functionality management to the UE, e.g. UE may be requested to provide its location information or fall back to legacy positioning method.

Discussion on the incoming LS [5] from SA2

In the LS [5], Case 2b is mentioned. According to the latest WID [2], the definition is listed as below:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]:

  - Direct AI/ML positioning:

    - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning

Due to the LS [5], the actions are listed as below:

To RAN 1 and RAN 2: For case 2b, SA2 is working on some procedures in corresponding R19 specifications, but has also captured the following Editor’s Notes on the interaction between the LMF and UE which depend on RAN progress: Editor's note: The input data used for AI/ML based positioning will be decided by RAN WGs. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs. Q1: SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

From the RAN1 LS [5], we understand that SA2 would like to know RAN1/RAN2 progress on data collection aspects for inference and training.

For Case 2b, we checked the previous RAN1 agreements, and  we have listed the relevant ones in section 6. For now, we observe that RAN1 has not made much progress on the data collection requirements, and RAN2 has not discussed LCM procedures for case 2b. In our understanding, for inference aspect or training aspect for case 2b, if the LMF needs some inputs from the UE, the existing LPP Provide Location Information (LMF-initiated or UE-initiated) can be used as the baseline, and details may wait for more RAN1 progress.

In summary, we think RAN2 could first check the latest RAN1 progress on the data collection requirements, and then RAN2 may discuss LCM procedures for inference and training aspects. After that, RAN2 may consider to reply LS to SA2.

Proposal 6: For inference and training aspects for case 2b, the existing LPP Provide Location Information can be used as the baseline for collecting data from UE to LMF.

Conclusions

In this paper, we discuss UE functionality reporting for positioning, monitoring, and functionality management. We have the following proposals:

Proposal 1: The UE should send an updated LPP ProvideCapabilities  message to the NW as soon as possible when the applicability status of the functionality changes.

Proposal 2: There is no need for any fallback option (i.e. the NW can reconfigure the UE after an updated LPP ProvideCapabilities message or a failure message is received).

Proposal 3: For positioning Case 1, RAN1 will discuss details of options and they may do down-selection. So RAN2 can wait for more RAN1 progress on monitoring.

Proposal 4: It is assumed that the inference configuration is provided in Step 3, and it needs more RAN1 progress on the content.

Proposal 5: Only the LPP request location information message can be used to do functionality management to the UE, e.g. UE may be requested to provide its location information or fall back to legacy positioning method.

Proposal 6: For inference and training aspects for case 2b, the existing LPP Provide Location Information can be used as the baseline for collecting data from UE to LMF.

References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- RP-243244, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm

- TR 38.843 v18.0.0

- [POST128][026][AIML] LCM Procedure for Positioning Case1 (Ericsson)

- S2-2501133, LS on LMF-based AI/ML Positioning for Case 2b, Source: SA2, To: RAN1, RAN2, Cc: RAN3

- S2-2501341, LS on LMF-based AI/ML Positioning for case 3b, Source: SA2, To: RAN3, RAN1, Cc: RAN2

Relevant RAN1/RAN2 progress on LCM for Positioning use case

In the objective, two types of UE-sided model exist for positioning accuracy enhancements, as highlighted with yellow marker [1].

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases

In the RAN2 #125b meeting, there are the following agreements for the UE-sided model.

Agreements: 1. Which AI/ML-enabled Features/FGs and functionalities are supported should be standardized. The details wait for RAN1’s progress. “supported” means that the UE is capable of supporting the functionality and doesn’t mean necessarily that the UE has the model available. FFS what functionality refers to. 2. Supported AI/ML-enabled Features/FGs and supported functionalities are included in UE capability. Agreements for positioning and beam management - Support proactive reporting of UE-sided applicable functionality, e.g., the UE reports its applicable AI/ML functionalities via UAI message/LPP message. - Support reactive reporting of UE-sided applicable functionality. The NW configures AI/ML functionalities via RRC/LPP message. FFS what the configuration contains. FFS how to report applicable functionality and what is applicable functionality 3 FFS how the two approaches will be specified and whether we can combine them into one procedure. FFS how to report applicable functionality, what is applicable functionality, how the UE determines which function is applicable or not (if it is needed) Agreements: 1 For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). 2 “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19

In the RAN2 #126 meeting, the following agreements are achieved:

Agreements

- RAN2 to await RAN1 progress to determine need for any specification work on AI/ML positioning Case 3a and Case 3b. (for NW-sided model)

- The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities.  FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality.

- wait for RAN1 for associate ID discussion

- At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation.  FFS further details on signaling enhancements

RAN2#127bis meeting minutes:

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message. 2: Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). 3: Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). 4: FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details. 5: RAN2 will decide whether AI positioning will be a new method after further details from RAN1 are received.

RAN1#118bis meeting minutes:

Agreement

For training data collection of AI/ML based positioning, the quality indicator of timing information in Part A when reported is:

- When applicable, the existing IE for timing quality, i.e., NR-TimingQuality in 37.355 and IE “Timing Measurement Quality” in 38.455;

  - FFS: details on how to associate quality indicator to timing information

Conclusion

For training data collection of Case 1, in terms of DL PRS configuration for collecting training data, both options are feasible by using legacy mechanisms:

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

Note: the UE can be a PRU and/or a Non-PRU UE.

Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB.

Agreement

From RAN1 perspective, for model inference of AI/ML positioning Case 3b, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) Channel measurement;

- (Optional) Quality of the channel measurement;

  - FFS: details of the quality

- (Mandatory) Time stamp of the channel measurement.

Agreement

From RAN1 perspective, when timing information is reported for Rel-19 AI/ML positioning Case 3a, at least the following are mandatorily or optionally supported in a measurement report from gNB to LMF:

- (Mandatory) timing information;

- (Optional) Quality of the timing information;

  - Existing IE “Timing Measurement Quality” can be reused.

- (Mandatory) Time stamp.

FFS: LOS/NLOS indicator.

Note: The final decision of “mandatory” or “optional” presence of each field is up to RAN3.

Agreement

For AI/ML positioning Case 1, regarding the assistance data provided from LMF to UE, for ensuring consistency between training and inference,

- for each of the existing assistance data IE of UE-based DL-TDOA and/or UE-based DL-AoD, study whether it should be: (a) explicitly indicated, (b) implicitly indicated and/or (c) other;

- Companies can provide inputs on further enhancements of existing assistance data, including new information

- Note: this does not mean that training and inference phases are mentioned in assistance data.

Table. Existing assistance data (supported up to Rel-18) that may be transferred from LMF to UE in UE-based DL-TDOA [1] or UE-based DL-AoD [2], as applicable.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 2 | Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 3 | DL-PRS configuration of candidate NR TRPs |  |  |
| 4 | Indication of which DL-PRS Resource Sets across DL-PRS positioning frequency layers are linked for DL-PRS bandwidth aggregation |  |  |
| 5 | SSB information of the TRPs (the time/frequency occupancy of SSBs) |  |  |
| 6 | Spatial direction information (e.g. azimuth, elevation etc.) of the DL-PRS Resources of the TRPs served by the gNB |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |
| 8 | Fine Timing relative to the serving (reference) TRP of candidate NR TRPs |  |  |
| 9 | PRS-only TP indication |  |  |
| 10 | The association information of DL-PRS resources with TRP Tx TEG ID |  |  |
| 11 | LOS/NLOS indicators |  |  |
| 12 | On-Demand DL-PRS-Configurations, possibly together with information on which configurations are available for DL-PRS bandwidth aggregation |  |  |
| 13 | Validity Area of the Assistance Data |  |  |
| 14 | PRU measurements together with the location information of the PRU |  |  |
| 15 | Data facilitating the integrity results determination of the calculated location |  |  |
| 16 | TRP beam/antenna information (including azimuth angle, zenith angle and relative power between PRS resources per angle per TRP) |  |  |
| 17 | Expected Angle Assistance information |  |  |
| 18 | PRS priority list |  |  |

[1] Table 8.12.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

[2] Table 8.11.2.1.0-1 in 38.305, Use equipment (UE) positioning in NG-RAN (Release 18), v18.3.0

Relevant RAN1 progress on LCM for Positioning use case 2b

RAN1#118 meeting agreements:

Agreement

For AI/ML positioning Case 2b and 3b, regarding the power information for determining the model input,

- For downlink power measurement, use DL PRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of DL PRS-RSRPP, use the existing measurement report mapping table for PRS-RSRPP in 38.133 as a starting point.

- For uplink power measurement, use UL SRS-RSRPP defined in TS 38.215 as a starting point.

  - For measurement report of UL SRS-RSRPP, use the existing measurement report mapping table for SRS-RSRPP in 38.133 as a starting point.

RAN1#116bis meeting agreements:

Working Assumption

For training data generation of AI/ML based positioning Case 2a and 2b, the channel measurement and its related data (e.g., time stamp) are generated by PRU and/or non-PRU UE.

Working Assumption

For training data generation of AI/ML based positioning Case 2b, the label and its related data (e.g., time stamp) can be generated by:

- PRU

- Non-PRU UE with estimated location

- LMF

Note: transfer of label and its related data is out of RAN1 scope.

RAN1#116 meeting agreements:

Agreement

For AI/ML based positioning case 2b, at least the following types of time domain channel measurements are supported for UE reporting to LMF:

- timing information;

- paired timing information and power information.

RAN1#114bis meeting agreements:

RAN1 approved the outgoing LS to RAN2 (R1-2310681/R2-2311720)

*****************************

For positioning

| LCM purpose | Case | Data content | Typical data size (per data sample) | Typical latency requirement | Notes |
| --- | --- | --- | --- | --- | --- |
| Training | All Cases | Measurements (corresponding to model input): timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | Relaxed |  |
|  | Direct AI/ML positioning | Label: Location coordinates as model output | 56 to 144 bits See Note 3 | Relaxed |  |
|  | AI/ML assisted positioning | Label: Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | Relaxed |  |
| Inference | 1 | Location coordinates as model output | 56 to 144 bits See Note 3 | See Note 5 |  |
|  | 2a, 3a | Intermediate positioning measurement (timing info, LOS/NLOS indicator) as model output See Note 2 | 10s bits to 100s bits per PRS/SRS resource See Note 3 | See Note 5 |  |
|  | 2b, 3b | Measurements (corresponding to model input): Timing, power, and/or phase info See Note 2 | Size depends on number of PRS/SRS resources, measurement type (timing, power, and/or phase info) and report format: ~100 bits to 1000s bits per PRS/SRS resource See Note 3 | See Note 5 |  |
| Monitoring | All Cases | See Note 8 | See Note 8 | Near-real-time | See Note 6 and 7 |

Note 1: The necessity and feasibility of difference cases (Case1 to Case3b) needs further discussion/conclusion.

Note 2: For measurements as model input, no agreement on measurement types (i.e., time, power, and/or phase) in RAN1 for all cases (i.e., Case1 to Case3b). Measurement types (including their necessity) and sizes/dimension needs to be further discussed. Candidate measurement types discussed/evaluated for model input include CIR (contains timing, power and phase information), PDP (contains timing and power information), DP (contains timing information). For labels (i.e., model output) of AI/ML assisted positioning (Case2a, Case3a), RAN1 identified an initial listing of candidates that provide performance benefits (i.e., timing info, LOS/NLOS indicator). RSRP/RSRPP is for further discussion.

Note 3: The measurement size of one data sample = (measurement data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model input). The label size of one data sample = (label data size of one PRS/SRS resource)*(number of PRS/SRS resources needed for model output). The quantization and bit representation of time, power, and phase information (including their necessity) still need to be further discussed.  Existing specification allows reporting of up to 64 PRS/SRS resources per frequency layer for one positioning fix. For evaluations, most companies considered up to 18 TRPs. It should be noted that AI/ML positioning is not restricted to work only with maximum of 18 TRPs.

- Example of calculation on a potential lower bound on measurement size per PRS/SRS resource:

  - A potential lower bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing only for 9 measurements per PRS/SRS resource): 16 + 9*8 = 88 bits. The total lower bound can be 88*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix. This is based on the assumption of timing info as 16 bits for first arrival and 9 bits for relative timing.

- Example of calculation of a potential upper bound on measurement size per PRS/SRS resource:

  - A potential upper bound on measurement size per PRS/SRS resource can be calculated as follows (assuming timing, power, and phase for 256 measurements per PRS/SRS resource and assuming 8 bit representation of each real number): 2*(8*256) = 4096 bits. The total upper bound can be 4096*N bits, where N is number of PRS/SRS resources used as model input for obtaining a positioning fix.

- For location coordinates (corresponding to model output)

  - The bit representation of location coordinates depends on the type of shape, resolution, and uncertainty used to indicate the location (e.g., ellipsoid point, ellipsoid point with uncertainty circle, high accuracy ellipsoid with uncertainty ellipsoid, etc.) as listed in TS 23.032. The range of bit representation for location coordinates can be 7 bytes to 18 bytes (i.e., 56 to 144 bits). The location information report in existing specifications may contain additional information besides location coordinates (e.g., velocity, location error, integrity info, etc.)

- For intermediate positioning measurement (corresponding to model output):

  - The quantization and bit representation of time, [RSRP/RSRPP], and LOS/NLOS information (including their necessity) as model output still need to be discussed in an appropriate working group. As a reference to existing timing representation in Rel17 [TS 37.355], an example on the label size can be of 21 bits per PRS/SRS resource while assuming model output produces one timing of 21 bits per PRS/SRS resource. The label size can be 21*N bits, where N is number of PRS/SRS resources for which intermediate positioning measurement has been generated. If LOS/NLOS indicator (1 bit per PRS/SRS resource assuming hard value for LOS/NLOS indicator) is included, the label size becomes 22*N bits.

Note 4: No agreement on reporting types (i.e., periodicity, event-triggered/on-demand, etc.).

Note 5: There are no agreements on the reporting latency.

Note 6: RAN1 agreed on an initial listing of entities that can derive the monitoring metric for AI/ML positioning for different cases (Case1 to Case3b):

-1: At least UE derives monitoring metric

-2a: At least UE derives monitoring metric

- LMF (if monitoring based on ground truth)

-3a: At least gNB/TRP derives monitoring metric

- LMF (if monitoring based on ground truth)

-2b and 3b: At least LMF derives monitoring metric

Note 7: No agreement yet on a monitoring decision entity or their mapping to other entities (e.g., entity running the inference, entity deriving the monitoring metric, etc.).

Note 8: RAN1 has studied several types of related statistics where potential request/report of Monitoring related statistics and its necessity are for further discussion.

*****************************