3GPP TSG-RAN WG2 Meeting #129					    R2-2501111

Athens, Greece, Feb. 17th - 21st, 2025

Title: 	Discussion on signalling feasibility of dataset and parameter sharing for CSI compression

Source: 	Huawei, HiSilicon

Agenda item:	8.1.5

Document for:	Discussion/Decision

- Introduction

For two-sided model, RAN1 has sent a LS [1] to RAN2, and the action is listed as below:

To: RAN2 CC: SA2/SA3/SA5 Action: RAN1 respectfully asks RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options - Dataset sharing consisting of {(Target CSI, CSI feedback)} - Encoder parameter sharing - Encoder parameter sharing + dataset sharing consisting of {target CSI}

In this contribution, we provide our analysis on feasibility of RAN1 identified solutions on two-sided model.

- Discussion

Background

For [1], it mainly mentions the following:

- NW can transfer dataset/parameter from NW to UE or UE-side (e.g. OTT server). Dataset/parameter can refer to Option 4-1, 3a-1 with/without target CSI. RAN1 also provides some analysis on data size for different objects

  - Option 4-1: may be 225 MB

  - Option 3a-1 without target CSI: may range from 36 KB to 52 MB

  - Option 3a-1 with target CSI: may be 161.6 MB (N2=600K)

- Transmission requirement: relaxed latency (e.g., days/weeks) and infrequent

- Generalization: RAN1 evaluations show good generalization ability, so RAN2 may assume that the number of dataset and/or parameter sharing instances will be limited to a small number

- RAN1 thinks that the standardized signaling may be:

  - over-the-air approach

  - other approaches

- Action to RAN2:

  - analyze the feasibility of standardized signaling (over-the-air and/or other approaches)

Discussion on Over-the-air approach

For over-the-air approach, the direction is:

- the dataset/parameter can be transferred from NW to UE first, and then UE transfers the dataset/parameter to OTT server. After offline engineering at OTT server (e.g. model (re-)training), OTT server may transfer model related information to UE. See figure 1.

Figure 1: over-the-air approach (to OTT server)

For CSI compression, the dataset/parameters are related to CSI information as shown in the RAN1 agreement where NW sends the CSI generation part to UE side in Option 3a-1 while for Option 4-1, the NW sends {target CSI, CSI feedback} to the UE side. Hence it is reasonable for the NW-side to be the NG-RAN.

Observation 1: Since NG-RAN generates CSI generation part in Option 3a-1 and {target CSI,  CSI feedback) for Option 4-1,  NG-RAN initiates the transfer of the dataset/parameters to the UE.

For solution analysis, we suggest re-using the model transfer/delivery options defined in TR 38.843 [2]:

-	Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling.

-	Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling.

-	Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling.

-	Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data.

-	Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data.

-	Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP).

-	Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE.

Once NG-RAN has generated dataset/parameter, it may either directly send them to UE via Uu interface, or send them to CN/OAM. For the later way, CN/OAM can further transfer them to OTT server, i.e. other approaches. If CN/OAM transfer them to NG-RAN, and then to UE, the transmission path would be: NG-RAN -> CN/OAM -> NG-RAN -> UE -> OTT server, and we do not think it has benefits over other approaches (detailed analysis can be found in section 2.3). So the above solution 2a/3a/2b/3b/4b are not considered for analyzing the over-the-air approach. For the above solution 4a, it is about the model transfer between OTT server and UE, and it is not related to this over-the-air approach. In summary, we think the above solution 1a and 1b can be considered for the over-the-air approach.

Then, there are two possible options for transfer of dataset/parameter in Uu interface:

Option 1a: NG-RAN can transfer dataset/parameter to UE via RRC signalling

Option 1b: NG-RAN can transfer dataset/parameter to UE via UP data

In figure 1, the information transfer between UE and OTT server can be left to implementation, i.e. UE-sided data collection option 1a. Otherwise, if UE-sided data collection solutions 2 and 3 are considered here, the whole procedure will be UE->NW->UE->NW->OTT server, and it makes no sense.

Proposal 1: There are two options for transfer of dataset/parameter in Uu interface:

- Option 1a: NG-RAN can transfer dataset/parameter to UE via RRC signalling

- Option 1b: NG-RAN can transfer dataset/parameter to UE via UP data

Proposal 2: For over-the-air approach, it is assumed that NG-RAN generates the dataset/parameter.

In the following sections, we study in details the Option 1a and 1b to transfer the dataset/parameters for RAN1 Options 3a-1 and 4-1.

Option 1a: dataset/parameter transfer using RRC signalling

For Solution 1a for model transfer/delivery defined in [2], the analysis from TR 38.843 is copied as below and we think it can be used as a starting point for analyzing Option 1a of transfer of dataset/parameters.

Table 7.3.1.4-1 Analysis of current status and gaps, and potential 
RAN specification impact for Solution 1a

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | Model size >45kBytes is not supported based on existing number of RRC segments | Extension of the number of RRC segments is required to support models larger than 45kBytes |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Transmission is restarted upon mobility | - Requires service continuity support for SRBs with segmentations. - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A3. Network controllability on model transfer/delivery and management at gNB | Management and interaction between UE and gNB is not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Procedure latency depends on model size and SRB priority | Impact on SRB in DL, e.g., a new SRB with configurable priority, etc. |

As we mentioned in section 2.1, the dataset/parameter may be small or large. If the size is less than 45kBytes, dataset/parameter transfer using RRC signalling is feasible. If the size is larger than 45KBytes, there may be some challenges.

For example, if the size of dataset/parameter is 50MB, it means at least 5556 PDU SDU (as defined in TS 38.323, the maximum supported size of a PDCP SDU is 9000 bytes). To support the transfer of such a large amount of information, RAN2 may discuss the following enhancements:

(A) If NG-RAN is to do the segmentation of information in RRC layer, extension of the number of RRC segments is required to support dataset/parameter larger than 45kBytes

(B) If NG-RAN is to do the segmentation of dataset/parameter above RRC layer (e.g. a new AS protocol layer), there may be few impacts to RRC

In the RAN1 LS, Option 3a-1 and 4 require offline engineering and hence OTT server is needed. The dataset/parameter can be transferred to UEs via two methods:

Method A): "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE transfers the information to OTT server

Method B): "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs can transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

In summary, there is a combination of solutions:

Table 1: Combination of solutions for Option 1a

|  | New AS protocol layer to do segmentation | RRC layer to do segmentation |
| --- | --- | --- |
| P2P | There may be a large amount of RRC messages for one UE, e.g. one RRC message carries a segment from the upper layer. Transparent to RRC layer. | There may be a large amount of RRC message segmentations (e.g. the DLDedicatedMessageSegment message) for one UE. Non-transparent to RRC layer. |
| P2M | There may be a small amount of information for one UE, e.g. one RRC message. Transparent to RRC layer. | There may be a small amount of information for one UE, e.g. one RRC message. Non-transparent to RRC layer. |

For P2M transmission, the data toward one UE is not large, so the impact at UE side is small. However, if a new protocol layer is to do segmentation, there may be some RAN2 impacts.

For P2P transmission with a large amount of data, according to the analysis in section 2.1, the transfer of dataset/parameter is "relaxed latency (e.g., days/weeks) and infrequent", so we think P2P transmission is feasible due to the following two reasons:

(a) the network does not require to transfer a large amount of data to the UE in a given short period of time

(b) the network can select a proper time and a proper UE for the transmission, e.g. when the network is in lightly loaded and the UE has good signal quality. In this case, data transmission is not a big problem

For P2P transmission, it may bring some challenges of UE power consumption and memory in order to receive the dataset/parameter from NW. In comparison, such impacts do not exist for P2M transmission as the data volume of dataset/parameter for P2M is expected to be quite small. So we suggest RAN2 to study UE impacts for candidate options, such as power consumption and memory, and the analysis of UE impacts can be applicable to all over-the-air approaches.

Observation 2: For Option 1a, if the size is less than 45KBytes, transfer of dataset/parameter using RRC signalling is feasible.

Proposal 3: For Option 1a, Table 7.3.1.4-1 in TR 38.843 can be used as the starting point for analysis.

Proposal 4: For Option 1a, if the size is larger than 45KBytes, the following enhancements can be considered:

- NG-RAN can do the segmentation of dataset/parameter above RRC layer. This is transparent to RRC layer and a new AS protocol layer may have some RAN2 impacts

- NG-RAN can do the segmentation of dataset/parameter in RRC layer. This is non-transparent to RRC layer, and this may have some RAN2 impacts

Proposal 5: Regarding how NG-RAN transfers dataset/parameter to UE, the following two ways can be considered:

- "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE can directly use them or the UE transfers the information to OTT server

- "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

Proposal 6: For over-the-air approach, UE impacts can be listed as one discussion area, such as power consumption, memory.

As we mentioned above, if the size of dataset/parameter is larger than 45KBytes, it may need to divided to several segments (either segments in the layer above RRC or RRC segments), and then NW can transfer these segments to UE. In our understanding, one or more segments may form a subset, so the NW may send the subsets to the UE. It is expected that all segments should be transferred to UE side successfully, however, it may happen that some segments may not be received by the UE. In this case, even if the transfer of dataset/parameter is triggered by NW, it is beneficial for UE to request the delivery of some missing segments/subsets to the network.

An alternative is that the NW can inform UEs of segments/subsets information in advance, such as total number of segments/subsets and identifiers. Then, UEs can request specific segments/subsets from the NW.

Proposal 7: For Option 1a, the NW can trasnfer segments/subsets of dataset/parameter to UE, and then the UE can request missing segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 8: For Option 1a, the NW can inform UEs of segments/subsets information to UE in advance, such as total number of segments/subsets and identifiers. Then UEs can request specific segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Option 1b: dataset/parameter transfer using UP data

For Solution 1b for model transfer/delivery defined in [2], the analysis from TR 38.843 is copied as below and we think it can be used as a starting point for analyzing Option 1b of the transfer of dataset/parameter.

Table 7.3.1.4-3 Analysis of current status and gaps, and potential 
RAN specification impact for Solutions 1b

| Discussion Area | Current status and Gaps | Potential RAN specification impact |
| --- | --- | --- |
| A1. Large, no upper limit model/model parameter size | - No model size limitation - PDU session termination at gNB is not supported | Requires PDU session termination at gNB if needed |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Model transfer continuity if PDU session terminated at gNB is not studied | - Identify a solution to support service continuity support between gNBs when PDU session is terminated at gNB if needed - Xn/NGAP enhancement(s) for model transfer/delivery continuity |
| A3. Network controllability on model transfer/delivery and management at gNB | Management and interaction between UE and gNB appear to be feasible but not supported | Requires management and interaction between UE and gNB (e.g., model identification, model transfer completion indication, etc.) when model management at gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | - Procedure latency depends on model size, QoS requirement and DRB priority - QoS management at gNB if PDU session is terminated at gNB is not supported | Identify a solution to support QoS management at gNB for model transfer when PDU session is terminated at gNB if needed |

For Option 1b, how it works is unclear. During Rel-18 study, one possible solution is that the gNB may establish a local DRB for the transfer of dataset/parameter. However, this solution has couple of issues, such as QoS management, radio bearer mapping, and so on. Besides, RAN2 may need to co-ordinate with other WGs, e.g. terminology, QoS management, charging. Thus, we do not think Option 1b should be listed as an alternative solution for over-the-air approach.

Proposal 8: Option 1b (i.e. dataset/parameter transfer using UP data) is not listed as an alternative solution for over-the-air approach.

Discussion on other approaches

For other approaches, the principle is that NW can transfer dataset/parameter to OTT server without impacting Uu interface. And then, OTT server may do offline engineering and then transfer model related information to UE, or OTT server can directly transfer the dataset/parameter to UE. Figure 2 shows the basic work flow.

Figure 2: work flow of other approaches

It is assumed that NG-RAN can generate dataset/parameter, and then there may be two ways:

(1) the transfer of dataset/parameter is done via core network, e.g. NG-RAN -> CN NF -> OTT server

(2) the transfer of dataset/parameter is done via OAM, e.g. NG-RAN -> OAM -> OTT server

For (1), how CN NF(s) are to be involved and potential specification impacts can be up to SA2 and perhaps other WGs (e.g. SA3).

For (2), how OAM is to be involved and potential specification impacts can be up to SA5 and perhaps other WGs (e.g. SA3).

As we analyzed in section 2.2., the information transfer between UE and OTT server can be left to implementation.

Observation 3: From RAN2 point of view, other approaches may be:

- the transfer of dataset/parameter is done via core network, e.g. NG-RAN -> CN NF -> OTT server. At least SA2/RAN3 should check details and feasibility

- the transfer of dataset/parameter is done via OAM, e.g. NG-RAN -> OAM -> OTT server. At least SA5 should check details and feasibility

- The information transfer between UE and OTT server can be left to implementation

Proposal 10: RAN2 can send a LS including the above observation 3 to SA2, and then they could check the feasibility in Rel-19.

- Conclusion

In this contribution, we mainly discuss signalling feasibility of dataset and parameter sharing for CSI compression based on the RAN1 LS [1], and we have the following observations and proposals:

For over-the-air approach

Observation 1: Since NG-RAN generates CSI generation part in Option 3a-1 and {target CSI,  CSI feedback) for Option 4-1,  NG-RAN initiates the transfer of the dataset/parameters to the UE.

Observation 2: For Option 1a, if the size is less than 45KBytes, transfer of dataset/parameter using RRC signalling is feasible.

Proposal 1: There are two options for transfer of dataset/parameter in Uu interface:

- Option 1a: NG-RAN can transfer dataset/parameter to UE via RRC signalling

- Option 1b: NG-RAN can transfer dataset/parameter to UE via UP data

Proposal 2: For over-the-air approach, it is assumed that NG-RAN generates the dataset/parameter.

Proposal 3: For Option 1a, Table 7.3.1.4-1 in TR 38.843 can be used as the starting point for analysis.

Proposal 4: For Option 1a, if the size is larger than 45KBytes, the following enhancements can be considered:

- NG-RAN can do the segmentation of dataset/parameter above RRC layer. This is transparent to RRC layer and a new AS protocol layer may have some RAN2 impacts

- NG-RAN can do the segmentation of dataset/parameter in RRC layer. This is non-transparent to RRC layer, and this may have some RAN2 impacts

Proposal 5: Regarding how NG-RAN transfers dataset/parameter to UE, the following two ways can be considered:

- "Point to point (P2P)". NG-RAN can transfer all dataset/parameter to one UE, and then the UE can directly use them or the UE transfers the information to OTT server

- "Point to multipoint (P2M)". NG-RAN can transfer dataset/parameter to some UEs, and each UE may get part of the information. And then these UEs transfer the information to OTT server. At OTT server, all received information can be combined into a whole one

Proposal 6: For over-the-air approach, UE impacts can be listed as one discussion area, such as power consumption, memory.

Proposal 7: For Option 1a, the NW can trasnfer segments/subsets of dataset/parameter to UE, and then the UE can request missing segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 8: For Option 1a, the NW can inform UEs of segments/subsets information to UE in advance, such as total number of segments/subsets and identifiers. Then UEs can request specific segments/subsets from the NW to assist the transfer of dataset/parameter. Here one or more segments may form a subset.

Proposal 9: Option 1b (i.e. dataset/parameter transfer using UP data) is not listed as an alternative solution for over-the-air approach.

For other approaches

Observation 3: From RAN2 point of view, other approaches may be:

- the transfer of dataset/parameter is done via core network, e.g. NG-RAN -> CN NF -> OTT server. At least SA2/RAN3 should check details and feasibility

- the transfer of dataset/parameter is done via OAM, e.g. NG-RAN -> OAM -> OTT server. At least SA5 should check details and feasibility

- The information transfer between UE and OTT server can be left to implementation

Proposal 10: RAN2 can send a LS including the above observation 3 to SA2, and then they could check the feasibility in Rel-19.

- References

R1-2410922, LS on signalling feasibility of dataset and parameter sharing, Source: RAN1, To: RAN2, Cc: SA2, SA3, SA5

TR 38.843 v18.0.0