3GPP TSG-RAN WG2 Meeting #129		R2-2501119

Athens, Greece, 17th – 21st February, 2025

Source:	vivo

Title:		Discussion on CP and SRAP impact for baseline procedure

Agenda Item:		8.13.3

Document for:	Discussion and Decision

# Introduction

In the last RAN2#128 meeting [1], the following agreements were reached on control plane procedures.

Agreements:

In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED.  Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED.  FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE.  FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE.

The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted.

In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED.

In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection.

The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted.

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network.  FFS how to split the PDB over each of the individual remaining hops.

Agreement:

In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s).  FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE).

Agreement:

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment.

All agreements in this WI apply to both these cases unless otherwise specified.

Meanwhile, the following agreements were reached on SRAP impact.

Agreements:

Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.  This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs.

Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

After the RAN2#128 meeting, an email [POST128][401] was kicked off to develop solution details on multi-hop relay control plane [2].

In this contribution, we will further discuss some remaining issues on CP and SRAP impact, by taking the above agreements and the email progress into account.

# Discussion

## System information delivery

Regarding the system information delivery procedure, the most controversial part is mainly related to the intermediate Relay UE’s behaviors. The FFS issues are listed as below according to the summary of [POST128][401][Relay] Control plane baseline solution (InterDigital):

Proposal 9: When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the cell on Uu (when the intermediate relay UE is in coverage). Proposal 10: The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): - when there is a change in the SI required by the intermediate UE - when the intermediate UE enters RRC_IDLE/RRC_INACTIVE - when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request) - upon reception of new/changed required SI received from a remote UE/child relay UE - FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay. Proposal 11: The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node: - Upon reception of SI received from a parent node (intermediate relay or last relay) containing SI requested by a child node (intermediate relay or remote UE) - Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE) - Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE) - Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay) - Upon receiving updated SIB1 from the network (as in Rel17) - Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE) FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states. FFS whether to consolidate some of the conditions in stage 3 specification text.

For the FFS issue from “Proposal 9”in [POST128][401], it is noticeable that even for legacy Rel-17 Remote UE, it can obtain SI from direct Uu interface by UE implementation. See highlighted text in green as blow:

TS 38.331-i30 ##### 5.2.2.2.1 SIB validity A L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected L2 U2N Relay UE as defined in clause 5.8.9.8.2 and receive the SIB(s) from the L2 U2N Relay UE as defined in clause 5.8.9.9.3. A L2 U2N Remote UE in RRC_CONNECTED receives SIB1 and other SIB(s) in RRCReconfiguration message and performs on-demand SI request if required, as defined in clause 5.2.2.3.5 and 5.2.2.3.6. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED (when MP is not configured) is not required to obtain SI over Uu interface, but it may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 by UE implementation.

For an intermediate Relay UE which is in coverage, the situation can be similar to the legacy Rel-17 Remote UE behavior. For example, when the intermediate Relay UE has Uu RSRP measurement results and finds it can be enough good to complete the SI acquisition procedure, it may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 by UE implementation.

Proposal 1	The intermediate Relay UE in coverage may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 in RRC specification by UE implementation, as legacy Rel-17 Remote UE behaviour.

For the FFS issue from “Proposal 10”in [POST128][401], there are two cases when there is a change in the ability of the intermediate Relay UE to receive SIB broadcast on Uu.

- Case 1: The intermediate Relay UE moves from in-converge (IC) to out-of-coverage (OOC)

- Case 2: The intermediate Relay UE moves from out-of-coverage (OOC) to in-converge (IC)

For Case 1, it is the intermediate Relay UE implementation whether it decides to perform the SI acquisition procedure over Uu interface when it is IC. Therefore, when it was previously performing the SI acquisition procedure over Uu interface, the intermediate Relay UE has no choice but must send the SI request in PC5-RRC to the parent relay to initiate SI forwarding by the parent relay when it moves to OOC. When it was previously not performing the SI acquisition procedure over Uu interface, then nothing needs to be done by the intermediate Relay UE since the SI request has already been sent to the parent relay.

For Case 2, similar to Case 1, when it decides to perform the SI acquisition procedure over Uu interface when it moves to IC from OOC, it surely will send SI request in PC5-RRC to the parent relay to cancel SI forwarding by the parent relay. When it decides not to perform the SI acquisition procedure over Uu interface, then nothing needs to be done by the intermediate Relay UE.

Therebefore, we have separate proposal for each Case when there is a change in the ability of the intermediate Relay UE to receive SIB broadcast on Uu.

Proposal 2	When the intermediate relay UE moves from IC to OOC, it sends SI request in PC5-RRC to the parent relay to initiate SI forwarding by the parent relay only if it was previously performing the SI acquisition procedure over Uu interface.

Proposal 3	When the intermediate relay UE moves from OOC to IC, it sends SI request in PC5-RRC to the parent relay to cancel SI forwarding by the parent relay only if it has sent SI request to its parent relay before and determined to perform the SI acquisition procedure over Uu interface afterwards.

For the FFS issue from “Proposal 11”in [POST128][401], we don’t think it needs to be limited to RRC_CONNECTED relay UE. At least when the in-coverage intermediate relay UE in RRC_IDLE or RRC_INACTIVE, it has the ability to receive SIB broadcast on Uu. Some stage 3 specification text can be added on top of the above green text described for legacy Rel-17 U2N Remote UE in clause 5.2.2.2.1.

Proposal 4	RAN2 confirms that the intermediate relay UE can send SI acquired (from the network) over Uu interface to a child node in any RRC state, i.e., no limitation to RRC_CONNECTED. This can be clarified in RRC specification clause 5.2.2.2.1.

## Paging delivery

There is issue on whether an intermediate Relay UE can monitor the paging messages for itself and its downstream UEs according to the summary of POST128][401][Relay] Control plane baseline solution (InterDigital):

Proposal 21: An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself. FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED. Proposal 22: When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly on Uu (when the the intermediate relay UE is in coverage).

With respect to “Proposal 21” and “Proposal 22” in POST[128][401], there are the following two issues which are not sufficiently discussed yet:

- Question 1: whether an intermediate Relay UE in RRC_IDLE/INACTIVE can monitor the paging message directly on Uu for itself and its downstream Remote UEs.

- Question 2: whether/how an intermediate Relay UE in RRC_CONNECTED state can monitor the paging message for its downstream Remote UEs.

In Rel-19 multiple-hop U2N network, a UE is qualified to be a last Relay UE only when the Uu radio quality of the UE is good enough. Moreover, a UE who is not qualified to be a last Relay UE may act as an intermediate Relay UE. For an intermediate Relay UE, its Uu radio quality is not as perfect as the last Relay UE but it does not mean that the intermediate Relay UE is always out of Uu coverage, and it is also possible that an intermediate Relay UE is in coverage but not serving as last relay UE, based on threshold configuration.

When an intermediate Relay UE is out of Uu coverage, the intermediat Relay UE should further forward the paging information received from a Remote UE to its last Relay UE to request the last Relay UE to monitor the paging message for this Remote UE on-behalf and forward the paging message received from the last Relay UE to this Remote UE later on, i.e. in this case, the baseline solution (i.e. the last Relay UE to monitor the paging message) has to be used.

When an intermediate Relay UE is in coverage, the intermediate U2N Relay UE can monitor the paging message for itself and the downstream UEs, i.e., the intermediate U2N Relay UE can keep the received paging information and monitor the paging message for the downstream UE on-behalf. In this case, the paging procedure delay for the Intermediate Relay and its downstream U2N Remote UEs can be shorter.

When an intermediate Relay UE is in RRC_CONNECTED state, there are different options for the intermediate relay UE to receive paging information:

Option 1: via dedicated RRC signaling

Option 2: monitor paging on PC5-RRC from parent relay UE

Option 1 can utilize the dedicated signaling but it may need extra signaling interaction because now the paging of the intermediate relay UE’s downstream IDLE/INACTIVE U2N Remote UEs are sent by dedicated signaling, while for option 2, the UE behavior is not changed, because if the intermediate relay UE enters RRC_CONNECTED, it means that all its upstream relay UEs also enter RRC CONNECTED, so the paging message is sent to last relay UE by dedicated RRC signaling and will be transferred on PC5-RRC to intermediate relay UE. We prefer to take option 2 as the baseline, because it is simpler and no extra signaling is needed.

Also, in this case, the paging information of its downstream U2N Remote UEs should not be released. Considering the following 2 cases:

Case 1: An intermediate Relay UE in RRC_CONNECTED state monitors the paging message for its downstream Remote UEs and the paging information of the downstream Remote UEs are released.

In this case, if the intermediate Relay UE transits to RRC_IDLE/INACTIVE state from RRC_CONNECTED state later on and has to rely on the last Relay UE to monitor the paging message for its downstream Remote UEs again, the intermediate Relay UE has to forward paging information of its downstream Remote UEs to the last Relay UE again. There can be an interruption period of paging message monitoring for its downstream Remote UEs during this period, depending on the RRC state transition duration and the duration for the paging information forwarding procedure. If the intermediate Relay UE frequently transits between RRC_IDLE/INACTIVE state and RRC_CONNECTED state, there is frequent interruptions for monitoring paging messages for the downstream Remote UEs.

Case 2: An intermediate Relay UE in RRC_CONNECTED state monitors the paging message for its downstream Remote UEs and the paging information of the downstream Remote UEs are still reserved.

If the paging information of a Remote UE is not released at the last Relay UE, the gNB can send the paging message to the last Relay and then to intermediate relay UE as before. There is no interruption for paging message monitoring for any Remote UE of an Intermediate Relay UE. Therefore, we propose:

Proposal 5	Upon reception of the paging information from a Remote UE, an intermediate Relay UE can perform one of the following actions:

- Further forward the paging information of the Remote UE to the last Relay UE, and forward the received paging message from the last Relay UE to the Remote UE accordingly afterwards, if it is out of coverage or if it is in RRC_CONNECTED state.

- Monitor paging message for the Remote UE and forward the paging message received over Uu interface to the Remote UE by itself, e.g., if it is in-coverage.

Proposal 6 	When an intermediate Relay UE enters RRC_CONNECTED state and monitors the paging message for its downstream U2N Remote UEs on-behalf, the paging information of its downstream U2N Remote UEs is not released.

A Remote UE in RRC_IDLE/RRC_INACTIVE connected with an intermediate Relay UE via PC5 link may leave the L2 multiple-hop Relay network due to mobility. In this case, if the intermediate Relay UE has sent the paging information of this U2N Remote UE to a last Relay UE before, the intermediate U2N Relay UE should notify the last U2N Relay UE to release the paging information of this Remote UE. This can help the last Relay UE to stop paging message monitoring for this Remote UE, which is beneficial for power saving of the last Relay UE.

Proposal 7	The intermediate Relay UE should send a notification to the last Relay UE which is responsible to monitor the paging information for the Remote UE on-behalf when the Remote UE left so that the last Relay UE can stop paging message monitoring for the Remote UE.

## SRAP impact

Following issues are further analyzed from Uu radio bearer configuration and SRAP configuration perspective.

## Issue 1: FFS whether each Relay UE except the Last Relay UE can establish SRB only (w/o DRB)

If a Relay UE except the Last Relay UE (i.e., an intermediate Relay UE) is just performing the relaying role for at least one remote UE but doesn’t have its own service, it is natural that the DRB configuration for this UE in not necessary. Moreover, the current RRC specification has already supported that for IAB only SRB1 and SRB2 are setup (w/o DRB), see highlighted blue text as shown in below Figure 1. IAB is also used for relaying purpose, we believe that it is feasible to extend a similar principle to multi-hop relay scenario. In other words, an intermediate Relay UE w/o its own service can establish SRB only (w/o DRB) during its RRC reconfiguration procedure if it is serving for at least one remote UE.

Figure 1. Pre-condition for SRB and DRB setup in current RRC specification

Proposal 8	RAN2 agree that an intermediate Relay UE w/o its own service can establish SRB1 and SRB2 only (i.e., w/o DRB) during its RRC reconfiguration procedure if it is serving at least one Remote UE.

## Issue 2: Uu radio bearer mapping configuration

RAN2#128 has agree that “Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel.”. The below Figure 2 further shows how an end-to-end Uu radio bearer is mapped to/from the serving gNB from each UE perspective (different color represents different UE’s Uu radio bearer).

Figure 2. Example of different UE’s Uu radio bearer mapping with different color

In accordance with the above illustration, it’s straightforward to have a bulk of proposals for each UE’s Uu radio bearer mapping configuration as follows:

Proposal 9	From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

  - Local identity of the Remote UE

  - The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

Proposal 10	From an intermediate Relay UE perspective (including the first Relay UE), the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

- For its own Uu RRC singnalling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to Egress PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC singnalling, if any

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to Egress PC5 RLC channel mapping

Proposal 11	From the last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5/Uu RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC singnalling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to Egress PC5/Uu RLC channel mapping

## Issue 3: UL/DL differentiation

For SRAP PDU format, RAN2#128 has agree that “Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.” In addition to that, for Uu SRB/DRB identity to Egress PC5 RLC channel mapping (see our Proposal 10), there is still an unresolved issue that how for an intermediate Relay UE to identify if the received SRAP PDU over a PC5 RLC channel is for upstream (UL) forwarding or downstream (DL) forwarding. The possible options are considered as below:

- Option 1: enhance the SRAP header format i.e., a “Direction” bit to discern UL/DL

- Option 2: separate Egress PC5 RLC channel configurations for UL/DL

For Option 2, it requires large number of RLC channel IDs, wherein currently available ID values may not be enough. And also, ASN.1 impact is foreseen for a new ID length extension. While for Option 1, it is simpler for UE forwarding behavior, which is also similar to IAB relaying mechanism. And thus, Option 1 is preferred.

Proposal 12	RAN2 to down-select Option 1 as following two Options to identify if the received SRAP PDU is for UL or DL forwarding at an intermediate Relay UE.

  - Option 1: enhance the SRAP header format i.e., a “Direction” bit to discern UL/DL

  - Option 2: separate Egress PC5 RLC channel configurations for UL/DL

## Issue 4: SRAP rerouting

Local rerouting is supported for IAB network to ensure robust data transmission. For multiple-hop U2N relay, the backhaul route is rather instable compared to multiple-hop IAB network as IAB node is supposed to be fixedly deployed while the U2N Relay UE(s) could be probably mobile. To ensure the robust data transmission, flexible data rerouting could be considered for SRAP design of multiple-hop U2N Relay, in order to conquer the backhaul route broken, see below Figure 3 as an example for rerouting. One simple improvement could be to introduce UL/DL indication in the SRAP header by reusing the R bit in R18 SRAP header. Based on the UL/DL indication in the SRAP header, an

y U2N relay which received an SRAP PDU with unknown local UE ID can determine upstream/downstream direction to forward the SRAP PDU instead of simply dropping the SRAP PDU.

Figure 3. Example of Rerouting

As multiple-hop U2N Relay could be very instable due to the mobility of U2N relay UEs, such rerouting scheme could be more important than IAB network. To facilitate the data rerouting in multiple-hop U2N Relay network, we propose:

Proposal 13	RAN2 to discuss whether SRAP rerouting needs to be supported for multiple-hop U2N Relay.

## Backward compatibility issue

L2 multiple-hop U2N relay can extend the network coverage even more than the R17/18 L2 U2N Relay network. By the time of L2 multiple-hop U2N relay deployment, legacy U2N Remote UE may have already considerable penetration in the market. Similar to the case that an advanced NR gNB is usually required to serve legacy NR UEs in the market by the operators, an L2 multiple-hop U2N relay network may also be probably required to serve legacy U2N Remote UEs so that the legacy U2N Remote UEs (see Figure 4 below) can also benefit from the coverage extension by the L2 multiple-hop U2N relay network.

Figure 4. Topology to serve legacy L2 U2N remote UE in L2 U2N multi-hop relay network

Proposal 14	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is supported, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# Conclusion

This contribution mainly discussed the potential CP and SRAP impact. The contribution concludes with:

System information delivery

Proposal 1	The intermediate Relay UE in coverage may decide to perform the SI acquisition procedure over Uu interface as defined in clause 5.2.2.3 in RRC specification by UE implementation, as legacy Rel-17 Remote UE behaviour.

Proposal 2	When the intermediate relay UE moves from IC to OOC, it sends SI request in PC5-RRC to the parent relay to initiate SI forwarding by the parent relay only if it was previously performing the SI acquisition procedure over Uu interface.

Proposal 3	When the intermediate relay UE moves from OOC to IC, it sends SI request in PC5-RRC to the parent relay to cancel SI forwarding by the parent relay only if it has sent SI request to its parent relay before and determined to perform the SI acquisition procedure over Uu interface afterwards.

Proposal 4	RAN2 confirms that the intermediate relay UE can send SI acquired (from the network) over Uu interface to a child node in any RRC state, i.e., no limitation to RRC_CONNECTED. This can be clarified in RRC specification clause 5.2.2.2.1.

Paging delivery

Proposal 5	Upon reception of the paging information from a Remote UE, an intermediate Relay UE can perform one of the following actions:

- Further forward the paging information of the Remote UE to the last Relay UE, and forward the received paging message from the last Relay UE to the Remote UE accordingly afterwards, if it is out of coverage or if it is in RRC_CONNECTED state.

- Monitor paging message for the Remote UE and forward the paging message received over Uu interface to the Remote UE by itself, e.g., if it is in-coverage.

Proposal 6 	When an intermediate Relay UE enters RRC_CONNECTED state and monitors the paging message for its downstream U2N Remote UEs on-behalf, the paging information of its downstream U2N Remote UEs is not released.

Proposal 7	The intermediate Relay UE should send a notification to the last Relay UE which is responsible to monitor the paging information for the Remote UE on-behalf when the Remote UE left so that the last Relay UE can stop paging message monitoring for the Remote UE.

SRAP impact

Proposal 8	RAN2 agree that an intermediate Relay UE w/o its own service can establish SRB1 and SRB2 only (i.e., w/o DRB) during its RRC reconfiguration procedure if it is serving at least one Remote UE.

Proposal 9	From a Remote UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

  - Local identity of the Remote UE

  - The Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

Proposal 10	From an intermediate Relay UE perspective (including the first Relay UE), the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5 RLC channel mapping

- For its own Uu RRC singnalling

  - Local identity of this Relay UE

  - This Relay UE’s SRB identity to Egress PC5 RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC singnalling, if any

  - Local identity of each downstream intermediate Relay UE

  - Each downstream intermediate Relay UE’s SRB identity to Egress PC5 RLC channel mapping

Proposal 11	From the last Relay UE perspective, the dedicated Uu radio bearer mapping configuration at least includes the following information:

- For relaying at least one Remote UE’s Uu RRC signaling and data

  - Local identity of each Remote UE

  - Each Remote UE’s Uu SRB/DRB identity to Egress PC5/Uu RLC channel mapping

- For relaying its downstream intermediate Relay UE’s Uu RRC singnalling

  - Local identity of each intermediate Relay UE

  - Each intermediate Relay UE’s SRB identity to Egress PC5/Uu RLC channel mapping

Proposal 12	RAN2 to down-select Option 1 as following two Options to identify if the received SRAP PDU is for UL or DL forwarding at an intermediate Relay UE.

  - Option 1: enhance the SRAP header format i.e., a “Direction” bit to discern UL/DL

  - Option 2: separate Egress PC5 RLC channel configurations for UL/DL

Proposal 13	RAN2 to discuss whether SRAP rerouting needs to be supported for multiple-hop U2N Relay.

Backward compatibility issue

Proposal 14	RAN2 to discuss whether backward-compatibility of multi-hop L2 U2N relay network is supported, i.e. whether the multi-hop L2 U2N relay network should also be able to serve R17/R18 L2 U2N Remote UEs.

# References

- RAN2#128 Meeting Chairman Notes.

- R2-2500496, Report of [POST128][401][Relay] Control plane baseline solution, InterDigital.