3GPP TSG-RAN WG2 Meeting #129		R2-2501120

Athens, Greece, 17th – 21st February, 2025

Source:	vivo

Title:		Discussion on Service continuity for multi-hop relay

Agenda Item:		8.13.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #128 meeting, the scenario A/B of service continuity for multi-hop relay were discussed and some agreements were reached as follows[1]:

Agreements:

For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused.

For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching.  Spec impact will be minimised.

For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure.  This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE.

Agreements for baseline control plane model:

The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching.

Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure.

The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching:

-	Event Y2

-	Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported.

Agreements:

Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300.

Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300.

This contribution will further discuss the scenario A/B about the steps based on the baseline procedure in R2-2410589 to finalize the procedure design.

- Discussion

  - Intra-gNB multi-hop indirect to direct path switching

According to agreements, the baseline procedure of intra-gNB multi-hop indirect to direct path switching procedure is as follows:

Figure 1.  Intra-gNB multi-hop indirect to direct path switching procedure

In above step-1, we would like to further clarify the measurement behavior on intermediate relay UE side. Now there are more than one hops, i.e.:

- Link 1: the link between remote UE and first relay UE

- Link 2: the link between first relay UE and intermediate relay UE

- Link 3: the link between intermediate relay UE and last relay UE

For link 1, the legacy configuration and measurement report signalling procedures can also be reused. But for link 2, in order to make the gNB known of the quality of link 2, the first relay UE should evaluate the relay link measurement and report it to gNB. What is included in the measurement report can also reuse the legacy SL relay mechanism, i.e. to include the measured intermediate Relay UE’s source L2 ID, serving cell ID and measurement quantity result. For link 3, the situation is similar. Meanwhile the intermediate relay UE may also measure link 2, which may not be restricted, anyway the gNB would finally know the reported measurement result is for which link because the measured relay information would be included.

The different aspects compared to legacy i2d case is that:

- The first relay UE and the intermediate relay UE does not have to perform Uu link measurement, as the gNB only needs the Uu measurement result for remote UE to switch it to direct Uu link.

Therefore, we propose:

Proposal 1: For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 2: For multi-hop i2d, first relay UE/ Intermediate relay UE evaluates adjacent relay link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Then for step 6, the gNB may also reconfigure the link 2 and link 3, e.g. to release PC5 Relay RLC channel configuration for relaying. However, it is not clear when the connection between first/intermediate relay UE and the intermediate/last relay UE needs to be released, e.g. only when there is no sidelink traffic anymore between them. we suggest this can be left to first/intermediate relay UE’s implementation when to release it.

Proposal 3: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 4: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

After all above discussions, we think Intra-gNB multi-hop indirect to direct path switching procedure is clear and can work well.

  - Intra-gNB multi-hop indirect to single-hop indirect path switching

According to agreements, the baseline procedure of  intra-gNB multi-hop indirect to single-hop indirect path switching is as follows[1].

Figure 2.  Intra-gNB multi-hop indirect to single-hop indirect path switching procedure

In step 1, the remote UE should also report one or multiple candidate last Relay UE(s) (or called candidate single-hop relay, to be more accurate), because after all the UE would be connected to gNB via a single hop relay at last. The reported information can be the same as legacy as well.

Proposal 5: For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

The relay link should also be reported to gNB. As we discussed for multi-hop i2d, for link 2 and link 3, in order to make the gNB known of the quality, the first/ intermediate relay UE should evaluate the relay link measurement and report it to gNB.

Therefore, we propose:

Proposal 6: For multi-hop i2i, first/ intermediate relay UE evaluates adjacent relay links, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Similarly, as we discussed in multi-hop i2d, after remote UE is switched to single-hop relay, the connection between source first/ intermediate relay UE and the source intermediate/ last relay UE may also need to be released, e.g. if there is no sidelink traffic anymore between them. But we understand this can be left to first relay/ intermediate relay UE’s implementation.

Proposal 7: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 8: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

- Conclusion

In this contribution, we discussed the path switch scenarios for first priority. We have the following Observations and Proposals:

For multi-hop i2d:

Proposal 1: For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 2: For multi-hop i2d, first relay UE/ Intermediate relay UE evaluates adjacent relay link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 3: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 4: For multi-hop i2d, after remote UE is switched to direct link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

For multi-hop i2i:

Proposal 5: For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Proposal 6: For multi-hop i2i, first/ intermediate relay UE evaluates adjacent relay links, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 7: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between remote UE and first relay UE, is up to remote UE implementation.

Proposal 8: For multi-hop i2i, after remote UE is switched to single-hop relay link, when to release the PC5 unicast link between first/ intermediate relay UE and intermediate relay UE/last relay UE, is up to first/ intermediate Relay UE implementation.

- References

- RAN2 #128 chair notes;