3GPP TSG-RAN WG2 Meeting #129	R2-2501136

Athens, Greece, Feb. 17th – 21st, 2025

Source:				ETRI

Title:					DSR enhancements for UL scheduling

Agenda Item:		8.7.4.2	DSR enhancements

Document for:		Discussion and Decision

1. Introduction

During the study of DSR enhancements by the RAN2#127bis meeting, agreements on DSR enhancements have been made as the followings:

Agreements on DSR enhancements - We do not change the definition of delay-critical data - For the sake of RAN2 discussions, we use the following terms: triggering threshold, reporting threshold(s) - Companies should analyse the impact of setting the triggering threshold to value lower than largest reporting threshold on DSR procedure, e.g. triggering, cancellation etc. - For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#127bis the following agreements have been made:

Agreements on DSR enhancements Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In this contribution, we provide discussions on the DSR enhancements to support scheduling enhancements for delay-critical data and a form of new DSR MAC CE in Rel-19 by leveraging the definition of DSR MAC CE in Rel-18.

2. Discussion

Multiple reporting thresholds and RRC configuration

The DSR MAC CE triggering mechanism relies on a single triggering threshold. When the smallest remaining delay time of delay-critical data falls below this threshold, a DSR MAC CE is initiated. If no DSR MAC CE is pending, a new one is transmitted.

It’s important that UE reports buffer status information to a scheduler to receive UL grants. The delay status in the buffer is more valuable when it is used to prioritize resource allocation for delay-critical data transmission. As time passes, the remaining delay time of the data decreases and it requires high priority for resource allocation. By utilizing information on the latency distribution of data in the buffer during scheduling, it is possible to anticipate resource requirements over time and provide UL grants. This could reduce the number of DSR transmissions and signalling overhead, thereby improving radio resource utilization.

The information about the delay time distribution of both delay-critical and non-delay-critical data is valuable for the scheduler. Time-critical data changes its classification over time and non-delay critical data changes to delay-critical data. When designing multiple reporting time thresholds, there is no inherent limitation to restricting report content to delay-critical data only. Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the delay remaining times of data within these ranges are reported.

Proposal 1) Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the shortest remaining times of data within these ranges are reported.

Proposal 2) The number of reporting time thresholds is configured by the RRC message for each LCG.

Proposal 3) New Rel-19 DSR-config  is introduced in radio resource control IE for each LCG.

Updating the DSR MAC CE

Legacy DSR MAC CE consists of delay information presence, BT, and single pair of remaining time and buffer size per LCG. These fields reused in enhanced DSR MAC CE as many as possible. The legacy DSR MAC CE is extended to include multiple pairs of remaining time and buffer size using the reserved field, R. In the position of 1-bit R field a new N field is defined as 1-bit indicator to indicate the start of a reporting pair for new LCG. Otherwise, the N fields indicate a reporting pair belonging to the previous LCG.

Observation 1) The legacy DSR MAC CE includes a reserved R field that can be utilized to introduce new indicator.

In the last meeting, it was agreed to include reporting information for each part by multiple reporting thresholds. Reporting pairs of remaining time and buffer size generated by reporting thresholds set for each LCG are included and reported in the DSR MAC CE. The base station utilizes this information in the scheduling process to efficiently allocate radio resources.

Proposal 4) The Rel-19 DSR MAC CE includes multiple reporting pairs of remaining time and buffer size per LCG.

Proposal 5) The different number of reporting pair of remaining time and buffer size is allowed per LCG.

In the last meeting, there was an agreement on a one-bit indicator. That indicator may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. We understand that there are various interpretations and applications of the agreement. From an extension perspective, we apply the indicator in a way that implements multiple pair entries. The indicator expresses the continued and non-continued states of multiple entries belonging to an LCG. If the indicator is continued, it indicates that a pair entry belongs to the same LCG. If the indicator is non-continued, it indicates that a pair entry belongs to a new LCG. Let's name the indicator field as the N (new) field if it emphasizes a new LCG state, and let's name the indicator field as the E (extension) field if it emphasizes the continued state.

Proposal 6) A Extension bit (e.g. redefining R bit in R-18 DSR) per a reporting pair of remaining time and buffer size is defined to indicate the presence of the next pair of the same LCG. (i.e. to determine if each subsequent pair entry belongs to the current or next LCG)

The figure below illustrates a modified DSR MAC CE structure that accommodates multiple reporting pairs within each Logical Channel Group (LCG) by introducing an E field, which is redefined R bit in Rel-18 DSR. Each reporting pair is uniquely identified by its index K. BTm_K, Remaining Time m_K, and Buffer Size m_K are included in the Kth reporting pair. The E field serves as an indicator of whether a new LCG is for the next pair. A value of E(1) signifies a current LCG, while 0 denotes a new LCG. While the depicted example contains K reporting pairs per LCG, the flexibility of the E field allows for varying the number of reports on a per-LCG basis.

Proposal 7) The R bit in legacy DSR MAC CE is redefined as the E(xtension) bit in new DSR MAC CE.

An example of enhanced DSR MAC CE

Mixture of Rel-18 DSR configuration and Rel-19 DSR configuration

In Release-18 DSR configuration, a remainingTimeThreshold is defined for each LCG. When the remaining time of delay-critical data falls below this threshold, a Release-18 DSR MAC is transmitted. Release-19 DSR configuration introduces a single triggering threshold and multiple reporting thresholds per LCG. If the triggering threshold is met, a Release-19 DSR MAC Control Element (CE) is transmitted. This CE contains multiple pairs of shortest remaining time and buffer size.

The Release-19 DSR MAC CE can be formatted to accommodate Release-18 DSR MAC CE. An "R" field in the Release-18 DSR MAC CE is used to indicate multiple reporting entries. Release-18 DSR MAC CE is transmitted with Release-18 DSR configuration, while Release-19 DSR MAC CE is transmitted with Release-19 DSR configuration.

A UE can be configured with both Release-18 and Release-19 DSR configurations. If the Release-19 DSR MAC CE contains only a single reporting entry, it can be formatted as a Release-18 DSR MAC CE. Otherwise, the full Release-19 DSR MAC CE can be utilized.

Proposal 8) The UE may be configured with a mixture of Rel-18 DSR configuration and Rel-19 DSR configuration by the RRC message.

Proposal 9) If the UE is configured with Rel-19 DSR configuration, the Rel-19 DSR MAC CE is used.

Proposal 10) If at least one LCG in the DSR MAC CE contains two or more reporting pairs of remaining time and buffer size, the Rel-19 DSR MAC CE is transmitted.

Proposal 11) the calculation of buffer size depends on the DSR configuration per LCG

3. Conclusion

In the discussion we made the following observations:

Observation 1) The legacy DSR MAC CE includes a reserved R field that can be utilized to introduce new indicator.

We propose the followings:

Proposal 1) Multiple reporting time thresholds are configured by the RRC message for each LCG to define remaining time ranges between time thresholds, and the shortest remaining times of data within these ranges are reported.

Proposal 2) The number of reporting time thresholds is configured by the RRC message for each LCG.

Proposal 3) New Rel-19 DSR-config  is introduced in radio resource control IE for each LCG.

Proposal 4) The Rel-19 DSR MAC CE includes multiple reporting pairs of remaining time and buffer size per LCG.

Proposal 5) The different number of reporting pair of remaining time and buffer size is allowed per LCG.

Proposal 6) A Extension bit (e.g. redefining R bit in R-18 DSR) per a reporting pair of remaining time and buffer size is defined to indicate the presence of the next pair of the same LCG. (i.e. to determine if each subsequent pair entry belongs to the current or next LCG)

Proposal 7) The R bit in legacy DSR MAC CE is redefined as the E(xtension) bit in new DSR MAC CE.

Proposal 8) The UE may be configured with a mixture of Rel-18 DSR configuration and Rel-19 DSR configuration by the RRC message.

Proposal 9) If the UE is configured with Rel-19 DSR configuration, the Rel-19 DSR MAC CE is used.

Proposal 10) If at least one LCG in the DSR MAC CE contains two or more reporting pairs of remaining time and buffer size, the Rel-19 DSR MAC CE is transmitted.

Proposal 11) the calculation of buffer size depends on the DSR configuration per LCG

4. References

R2-2409101, “Discussion on DSR enhancements,” ETRI.

R2-2410843, “Further Discussion on DSR enhancements,” ETRI.