3GPP TSG-RAN WG2 Meeting #129	R2-2501138

# Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.2.4 (Ambient_IoT_solutions)

Source:	LG Electronics Inc.

Title: 	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1.	Introduction

This document discusses ambient IoT (A-IoT) data transmission. In this contribution, we discuss mainly retransmission for data transmission failure handling, segmentation, and AS ID. The related work item objectives are as follows.

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

2.	Discussion

Data (re)transmission for failure handling

The handling of data transmission failure is studied during SI phase of A-IoT and described in TR38.769 (Study on solutions for Ambient IoT in NR). It is understood that the subsequent R2D data transmission after D2R data transmission does not need to be always sent. In case of D2R data transmission failure, the A-IoT device follows the reader subsequent R2D instruction, if any. For instance, the reader can repeat the R2D upper layer "command" to trigger the A-IoT device to re-send the same D2R upper layer "response" (i.e., the A-IoT device just follows the received R2D to transmit following D2R).

For data retransmission to handle D2R data transmission failure, the above-mentioned method seems to be working well. Considering that HARQ, RLC ARQ/AM, and RLC-like/ARQ-like retransmission are studied but concluded not to be supported, it is supported for the reader to re-send the R2D upper layer message (e.g., the command) if the corresponding D2R data is not received.

Proposal 1. Confirm that, if the reader does not receive the expected D2R message, the reader is allowed to re-send the R2D message which triggered the D2R message to trigger the A-IoT device to re-send the same D2R message.

R2D data transmission failure does not need to be handled separately from D2R data transmission failure handling based on the following two reasons.

First, it is assumed that the corresponding D2R data transmission follows R2D data transmission. If the R2D data transmission is a ‘read’ command, the D2R data transmission for the response (i.e., read data) will happen. Even for a ‘write’ command, the result for the ‘write’ command may be transmitted in the D2R direction. Therefore, the reader may assume R2D data transmission failure if the corresponding D2R data is not received. However, it does not only mean R2D data transmission failure, but also it may mean D2R data transmission failure because it is also possible that the corresponding D2R data is not received due to D2R data transmission failure even if the R2D data transmission is successful. Based on the observations, if the corresponding D2R data transmission follows R2D data transmission, R2D data transmission failure can be covered by D2R data transmission failure handling.

Secondly, device-originated by device-terminated trigger (DO-DTT) and device-terminated (DT) data transmission are considered for Rel-19 A-IoT. A reader controls transmission between the reader and A-IoT devices, and an A-IoT device does not autonomously send any data including ACK/NACK feedback indication for R2D data transmission without trigger by the reader. In addition, if an A-IoT fails to receive an R2D data transmission, it also misses a trigger for D2R transmission which is included in the R2D data transmission. In other words, if an R2D data transmission fails and retransmission is required, the reader needs to be aware of the R2D data transmission failure. But, the A-IoT device does not send any indication without the reader’s trigger. Moreover, if the device fails to receive the R2D message which includes the trigger information, it does not send anything, either. From the observations, the reader can determine R2D data transmission failure based on the corresponding D2R transmission failure. Therefore, the failure handling can be covered by D2R data transmission failure handling.

Observation 1. R2D data transmission failure does not need to be handled separately. R2D data transmission failure leads to D2R data transmission failure. R2D data transmission failure can be covered by D2R data transmission failure handling.

Segmentation for D2R

According to WID, segmentation is supported at least in D2R. During the SI phase, it is agreed that segmentation indication will be discussed in WI phase and if segmentation is supported it is beneficial for the reader to be able to trigger a re-transmission of a segment the previous meeting (R2-128 meeting). However, how to trigger/handle a re-transmission of a segment is not concluded yet. Also, it is agreed that sequence number and segment number are not supported for a simple segmentation operation for A-IoT segmentation functionality in R2-127bis meeting.

Assuming that sequence numbers and segment numbers are not there, the segments should be transmitted in order. Otherwise, it is not guaranteed that the reassembly is performed properly. To guarantee in-order delivery of segments belonging to the same message, a stop and wait protocol should be considered. The reader performs the next new D2R scheduling for the next segment after successfully receiving the segment which is sent from the A-IoT device according to the current D2R scheduling.

Proposal 2. If segment number is not supported, a stop and wait protocol is used for segment transmissions to guarantee in-order delivery.

For D2R segmentation, the reader needs to know whether the data is segmented or not for the D2R scheduling for segment transmission. With the stop and wait protocol, 1-bit segmentation indication seems to be working well. The 1-bit segmentation indication may be interpreted as whether there is the next segment which needs to be scheduled or not.

If the D2R message size is known to the reader, the segmentation indication is not necessary because the reader can determine whether the A-IoT device has the remaining part of the message based on comparing the size of the received data and the D2R message size. However, it depends on other WG’s discussion that the D2R message size information is always informed to the reader.

Proposal 3. 1-bit segmentation indication is considered for D2R segmentation if a segmentation indication is required for A-IoT segmentation functionality.

AS ID

It is assumed that AS ID is used at least for the purpose of D2R scheduling and R2D reception, and results from SI phase discussion are described in TR38.769 (Study on solutions for Ambient IoT in NR). An AS ID should be shorter than the full upper layer device ID, and should be unique in the area covered by the reader when it is used. In that sense, random ID can be used as AS ID. If random access is successfully completed, the random ID used for the random access is unique within the access occasion. Thus, the message transmissions related to the A-IoT service can be scheduled using the AS ID (i.e., random ID) within the access occasion without risk of AS ID collision.

In addition, considering that the A-IoT device may be equipped with a small energy storage and it may store AS ID only for a limited duration, we think, the AS ID needs to be re-assigned every trigger (i.e., paging message) for A-IoT service.

Proposal 4. Support that random ID is reused as AS ID.

According to WID, multiplexing/multiple access in R2D is by only TDMA. But, FDMA and TDMA can be used in D2R. Using D2R FDMA, Msg1 in random access can be transmitted in different frequencies from different A-IoT devices at the same time. If multiple Msg1 transmitted in the same frequency have the same random ID, it is a collision. However, even if Msg1 transmitted in different frequencies have the same random ID, it is not a collision. The random ID can be used as AS ID for D2R scheduling. On the other hand, Only TDMA is used for R2D. If random ID is used as AS ID for R2D reception without consideration about frequency, there may be confusion between A-IoT devices which choose different frequency, but select the same random ID. For example, an R2D transmission to a specific A-IoT device can be received by another A-IoT devices. To resolve this issue, another restriction may be considered. One possible restriction is that random IDs are required to be different across frequencies for contention resolution. However, it will prevent access on different frequencies from work separately. The related operation on the reader side will be more complicated and the performance of random access using FDMA may be degraded. The approach is not preferred.

In the case that D2R FDMA is used, another way to resolve the above-mentioned issue is to use frequency information along with AS ID (i.e., random ID) for R2D reception. Although additional bits to indicate the frequency where the random access is successfully completed are required, random ID can be used as AS ID for R2D reception with no impact on Msg1 contention resolution. This can be considered a way to resolve AS ID collision for R2D reception when D2R FDMA is used.

Proposal 5. Discuss using D2R frequency information along with AS ID for R2D reception if FDMA is used only in D2R.

3.	Conclusion

This document discusses retransmission for data transmission failure handling, segmentation, and AS ID.

Proposal 1. Confirm that, if the reader does not receive the expected D2R message, the reader is allowed to re-send the R2D message which triggered the D2R message to trigger the A-IoT device to re-send the same D2R message.

Observation 1. R2D data transmission failure does not need to be handled separately. R2D data transmission failure leads to D2R data transmission failure. R2D data transmission failure can be covered by D2R data transmission failure handling.

Proposal 2. If segment number is not supported, a stop and wait protocol is used for segment transmissions to guarantee in-order delivery.

Proposal 3. 1-bit segmentation indication is considered for D2R segmentation if a segmentation indication is required for A-IoT segmentation functionality.

Proposal 4. Support that random ID is reused as AS ID.

Proposal 5. Discuss using D2R frequency information along with AS ID for R2D reception if FDMA is used only in D2R.