3GPP TSG-RAN WG2 Meeting #129	R2-2501141

Athens, Greece, Feb 17 – 21, 2025

Agenda item:	8.7.6

Source:	Samsung

Title:	Discussion on UL rate control for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the last RAN2 meeting, we agreed that:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this paper, we provide our view on the UL congestion/rate control signalling.

Discussion

QoS Flow Level Bit Rate Signalling

In the last meeting, we have agreed to support QoS flow level rate indication from gNB to UE. However, there are two options that have been considered for this purpose, i.e., 1) MAC CE based approach, and 2) RRC/MAC CE combined approach.

Regarding MAC CE based approach, how it works is clear, i.e., a new MAC CE with QoS flow level rate indication is to be introduced. However, the motivation of the consideration of the RRC/MAC CE combined approach, which may be much more complex, is caused by the concern that, in CU/DU split case, DU is not capable of managing QoS flow level rate control. However, we think the concern is not valid in current 5G system, since DU is already provided with QoS flow level QoS parameters (e.g., by UE CONTEXT SETUP REQUEST from CU) including the GFBR/MFBR reflecting the capability of throttling, and since all the QoS flows mapped to a single DRB will be treated indifference in AS, DU can infer the average bandwidth provided to each QoS flow, based on which, the congestion level and the desirable source rate for each QoS flow can be inferred.

Besides, there is quite a lot relevant information for DU to take into account, e.g., ARP priority/pre-emption capability/pre-emption vulnerability, and priority level in 5G QoS characteristics are all reflecting the relative importance among different QoS flows, and hence, DU with such information can firstly consider the relatively less important/more vulnerable QoS flow for the rate throttling.

Observation 1: In CU/DU split case, DU is already provided with essential information for QoS flow level rate control.

Moreover, RRC/MAC CE combined approach introduces more residual issues, e.g., how to let UE know each respective portion of bit rate for each QoS flow if multiple QoS flows are mapped to a common DRB, with only MAC CE indicating DRB level bit rate. Also, it is still challenging for RRC/MAC CE approach on how the DU can decide target DRBs to be throttled and the corresponding bit rates, respectively, without knowing the QoS flow level characteristics.

Observation 2: The RRC/MAC CE combined approach entails more uncertainties than MAC CE based approach for NW and UE to implement QoS flow level rate control.

Proposal 1: RAN2 to prioritize MAC CE based approach over RRC/MAC CE combined approach, in implementing QoS flow level rate control, unless critical issues are exposed by other WGs.

MAC CE for Rate Query/Preference Indication

From the responsibility point of view, it is gNB who is supposed to be responsible for enforcing the QoS requirement for each QoS flow, based on the corresponding QoS profile. While, the application residing in UE can have its own preference, e.g., based on its own optimization policy. In order to let gNB make final decision considering both QoS enforcement and application preference, it is desirable to construct a closed loop system embracing a UL MAC CE for UE rate query/preference indication.

Observation 3: A closed-loop system is desirable for UE to provide its preference, and for gNB to adapt the final decision considering both QoS enforcement and UE preference.

Proposal 2: RAN2 to consider UL MAC CE for UE to query the preferred bit rate per QoS flow.

Recommended Bit Rate Values Enhancement for XR

SA4 replies that the target rate ranges from 100 kbps to 40 Mbps, with at least 10% granularity.

Considering the legacy recommended bit rate MAC CE supports up to 1.6 Gbps bit rate indication, the legacy bit rate table with bitRateMultiplier already covers the whole range replied by SA4. Also, for the granularity, in the legacy table, the quantization error of the legacy table is in average ~10%.

Observation 4: Current legacy bit rate table with multipliers already covers the range of the XR bit rates replied by SA4 with acceptable quantization error.

Proposal 3: RAN2 to consider reusing the legacy bit rate table for the XR bit rate indication.

Note that if the upper bound of the XR bit rates (as replied by SA4) is 40 Mbps, with legacy table, currently the smallest multiplier, i.e., 40, has to be used for indicating it, while, the expressable range then becomes 0 to 320 Mbps. In this case, the number of codepoints within the bit rate range of interest (i.e., below 40 Mbps) is only 36, resulting in the waste of almost half of the codepoints. Therefore, it would be better to introduce smaller multiplier(s), e.g., 5 (800 kbps x 5 = 40 Mbps), to exploit as many codepoints as possible for serving the target XR bit rate range.

Observation 5: Current multipliers, i.e., 40, 70, 100, and 200, available for the legacy bit rate table are too large to serve the target XR bit rate, resulting in the waste of the codepoints.

Proposal 4: RAN2 to introduce smaller multiplier(s) for legacy bit rate table to serve XR bit rate indication.

Conclusion

Based on the discussion above, we have the following observations and proposals:

Observation 1: In CU/DU split case, DU is already provided with essential information for QoS flow level rate control.

Observation 2: The RRC/MAC CE combined approach entails more uncertainties than MAC CE based approach for NW and UE to implement QoS flow level rate control.

Proposal 1: RAN2 to prioritize MAC CE based approach over RRC/MAC CE combined approach, in implementing QoS flow level rate control, unless critical issues are exposed by other WGs.

Observation 3: A closed-loop system is desirable for UE to provide its preference, and for gNB to adapt the final decision considering both QoS enforcement and UE preference.

Proposal 2: RAN2 to consider UL MAC CE for UE to query the preferred bit rate per QoS flow.

Observation 4: Current legacy bit rate table with multipliers already covers the range of the XR bit rates replied by SA4 with acceptable quantization error.

Proposal 3: RAN2 to consider reusing the legacy bit rate table for the XR bit rate indication.

Observation 5: Current multipliers, i.e., 40, 70, 100, and 200, available for the legacy bit rate table are too large to serve the target XR bit rate, resulting in the waste of the codepoints.

Proposal 4: RAN2 to introduce smaller multiplier(s) for legacy bit rate table to serve XR bit rate indication.