3GPP TSG-RAN WG2 Meeting #129	R2-2501144

Athens, Greece, Feb 17 – 21, 2025

Agenda item:	8.7.3

Source:	Samsung

Title:	Discussion on RRM measurement gaps/restrictions enhancements for Rel-19 XR

Document for:	Discussion & Decision

Introduction

In the past RAN2 meetings, we have reached the agreements as follows.

Agreements for RRM measurement gap skipping - RAN2 assumes that at least some impact on DSR from MG skipping can be avoided by NW implementation. FFS whether there is an impact which would require some specification changes/enhancements. - No need to have delay-aware LCP enhancements specific for MG skipping, i.e. MG skipping and delay-aware LCP are designed as independent features - RAN2 can further evaluate whether there is any impact on DRX from MG skipping. For the moment, the issue is unclear. - RAN2 will focus its work on supporting the solution chosen by RAN1/RAN4 - RAN2 can discuss whether there is a need to additionally have other solution (e.g. RRC-based) which can be decided after RAN1/RAN4 evaluation and knowing more details of DCI-based solution - No MG-specific enhancements is needed on DSR operation. - RAN2 assumes that UE follows DRX pattern as currently, even when MG is indicated as skipped - No MG-specific enhancements is needed for DRX operation.

In this paper, we discuss the essential aspects of the operations regarding transmission/reception in gaps/restrictions.

Discussion

RAN2 Impact

RAN1 has agreed with the working assumption that a one-bit indication in DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset.

From the MAC perspective, TS 38.321 specified that during an activated measurement gap, the MAC entity is not allowed to perform transmission/reception/PDCCH monitoring, on the serving cell(s) in the corresponding frequency range.  Since we have one more state, i.e., not skipped, of an activated measurement gap occasion to be applied for, we should reflect the condition accordingly in the TS 38.321 for the conditions of the UE operations conducted previously during an activated measurement gap.

Proposal 1: RAN2 to reflect the newly introduced state, i.e., not skipped, for an activated measurement gap in TS 38.321, as the additional condition of the UE operations conducted previously during an activated measurement gap.

On the other hand, new UE capability signalling should be introduced to indicate the support of the measurement gap occasion skipping based on DCI indication.

Proposal 2: RAN2 to introduce a new UE capability signalling, indicating the support of the DCI-based measurement gap occasion skipping.

Need of Semi-Static Solution

Besides the DCI-based dynamic solution, a question worth discussing is, whether or not the semi-static solution is essential for serving delay-critical UL data.

Since XR traffic in UL, e.g., video, audio, pose and control, etc., is periodic with some extent of jitter (as the UAI extension introduced in Rel-18 XR), RRC-based semi-static solution, i.e., RRC configures which measurement gap occasions can be skipped, can save signalling overhead compared to the dynamic solution, and can facilitate UE implementation, since UE can prepare the skipping operation in advance compared to the DCI-based signalling. Also, considering the existence of the jitter of the XR UL traffic, whether a certain measurement gap occasion semi-statically configured by RRC, is finally skipped or not, should be determined by UE, based on its decision rule reflecting the actual buffer status and the necessity of the skipping, e.g., UL transmission for delay-critical data overlaps with the measurement gap occasion, to minimize the side effect caused by unnecessary MG skipping.

Proposal 3: RAN2 to consider semi-static measurement gap skipping solution, where RRC configures a set of measurement gap occasions allowed to be skipped.

Proposal 4: RAN2 to consider semi-static measurement gap skipping solution, where UE makes the final decision of the actual skipping for each measurement gap occasion among the set of occasions configured by RRC.

Conclusion

Based on the discussion above, we have the following proposals:

Proposal 1: RAN2 to reflect the newly introduced state, i.e., not skipped, for an activated measurement gap in TS 38.321, as the additional condition of the UE operations conducted previously during an activated measurement gap.

Proposal 2: RAN2 to introduce a new UE capability signalling, indicating the support of the DCI-based measurement gap occasion skipping.

Proposal 3: RAN2 to consider semi-static measurement gap skipping solution, where RRC configures a set of measurement gap occasions allowed to be skipped.

Proposal 4: RAN2 to consider semi-static measurement gap skipping solution, where UE makes the final decision of the actual skipping for each measurement gap occasion among the set of occasions configured by RRC.