3GPP TSG-RAN WG2 Meeting #129 	   R2-2501153

# Athens, Greece, Feb. 17th – 21st, 2025

Source:	TCL

Title:	MAC CE for DSR enhancement and interworking with legacy DSR

Agenda item:	8.7.4.2

Document for:	Discussion and decision

- Introduction

In RAN2 #127bis meeting, discussion on DSR enhancement has been made a great progress. In this contribution we will discuss MAC CE design for DSR enhancement and interworking with legacy DSR.

- Discussion

  - MAC CE design for DSR enhancement

Based on the agreement regarding the enhanced DSR format, and with reference to the legacy DSR MAC CE format, one possible MAC CE format for DSR enhancement is illustrated in Figure 2: The only change is that the "R" bit preceding each pair of remaining time and buffer size in the legacy DSR format has been changed to an "E" bit. This "E" bit is used to indicate whether there is another one following a pair of remaining time and buffer size for an LCG.

Figure 2

Proposal 1: It is suggested RAN2 to adopt the above MAC CE design for the enhanced DSR.

  - Interworking with legacy DSR

Basically, the legacy and enhance DSR can operate independently per LCG. Regarding the usage of the legacy and enhanced MAC CE format, based on the MAC CE design above, in the case that there is at least one LCG configured with enhance DSR and all LCGs only include one pair of remaining time and buffer size, both legacy and enhanced DSR MAC CE is applicable. However, the co-existence could cause unnecessary confusion and no benefit compared to only using the enhance DSR MAC CE. So we propose to always use a new enhanced DSR MAC CE in case there is at least one LCG configured with enhance DSR. Otherwise, i.e. all the LCG are configured with legacy DSR, legacy DSR MAC CE will be used.

Proposal 2: It is suggested to use a new enhanced DSR MAC CE in case there is at least one LCG configured with enhance DSR.

- Conclusion

In this contribution, we discussed the MAC CE design for DSR enhancement and interworking with legacy DSR and provided our proposals as follows:

Proposal 1: It is suggested RAN2 to adopt the above MAC CE design for the enhanced DSR.

Proposal 2: It is suggested to use a new enhanced DSR MAC CE in case there is at least one LCG configured with enhance DSR.