3GPP TSG-RAN WG2 Meeting #129													                         R2-2501159

Athens, Greece, Feb 17th – 25th, 2025                                                           Revision of R2-2410636

Agenda item:	8.9.2

Source:	MediaTek Inc.

Title:	RAN2 impact on S&F mode

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1], one of the objectives is to support S&F operation, as below:

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore:

  - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2]

    - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3]

Note: Strive to minimise UE impact.

Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

A study report, 23.700-29 [2], on the integration of satellite components in the 5G architecture Phase 3 has been released by 3GPP SA2. The conclusion for Key Issue #2: Support of Store and Forward Satellite operation has been made. Two types of system architecture can be adopted by operation deployment: a full CN onboard the satellite or a Split MME architecture.

In this contribution, we share our view on S&F mode, incorporating input from SA2.

# Discussion

S&F operation indication

In the RAN2#127bis, RAN2 made the following agreements regarding S&F operation indication. This indication has three possible meanings:

- Absent: this NTN cell is operating in real-time mode.

- ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

- ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Agreements:

1.	The dynamic indication that “the cell is operating in S&F mode” is called “S&F operation” indication (we can come back on the exact name when putting this in the spec, if needed)

2.	RAN2 assumes that if an indication that “the cell is operating in S&F mode” is not provided in a NTN cell, the UE should assume that the NTN cell is operating in real-time mode (i.e. default/normal/”not-S&F” mode). RAN2 assumes there should be no distinction between the case that the UE is served by a NTN cell that do not support S&F capability and the case that the UE is served by a NTN cell that does support S&F capability but indication that “the cell is operating in S&F mode” is not provided.

3.	An S&F explicit capability indication by the serving cell, conceived as a static indication of whether the S&F capability is supported or not by a specific satellite/NTN-cell, in addition to the indication that “the cell is operating in S&F mode”, is not needed

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

SA2 has also mentioned this indication in the latest TS 23.401[3], the NAS needs to know if the satellite is operating in the S&F mode to indicate the S&F capability during Attach and TAU.

### 4.13.9 Support of Store and Forward Satellite Operation #### 4.13.9.1 General --omit-- When a satellite supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode, and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

Thus, the AS shall forward this S&F operation mode indication to NAS. Regarding the three possible meanings, the expected AS behaviour is summarized as following.

- If this indication is absent, the cell is operating in normal mode, AS should not forward this indication to NAS.

- If this indication is present and the value equals ‘1’, the NTN cell is operating in S&F mode for all UEs. In this case, AS should notify NAS.

- If this indication is present and the value equals ‘0’, idle Rel-19 UEs supporting S&F are barred and the cell is operating in S&F mode for UEs in Connected mode. For UE in connected mode, AS should notify NAS if SIB1 is updated. For UE in idle mode, if the cell is barred, this information is no needed for NAS.

Proposal 1: For Rel-19 UEs supporting S&F, if the S&F operation indication is present in SIB1, RRC will notify upper layer that this NTN cell is operating in S&F mode when camping on the cell or receiving updated SIB1 in Connected mode.

Time information

In the RAN2#128, RAN2 made the following agreements regarding S&F time information.

Agreements:

- Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

The information of remaining time of S&F mode is broadcasted. The intention of this time information is to delay some NAS procedure. The Wait Timer which is also used to delay some NAS procedure is notified by NAS signalling. To control the NAS procedure combined with Wait Timer and remaining time of S&F mode, the NAS should be the controller. Therefore, the information of remaining time of S&F mode should be forwarded from RRC to NAS when it is first received or updated thereafter.

Proposal 2: For Rel-19 UEs supporting S&F, the RRC will forward the remaining time of S&F mode to the upper layer upon reception and when updated.

Given that the NTN capable UE in idle mode does not need to keep the SIB31 updated, and the UE will not be informed about the SIB31 modification, if the information of remaining time of S&F mode is broadcasted in SIB31, the UE cannot get the updated information.

SIB1 contains information relevant when evaluating if a UE is allowed to access a cell. This information, which delays some NAS procedures, is not needed when camping on the cell. To keep the SIB1 slim and to keep cell camping swift, this information should not be broadcasted in SIB1 either.

SIB2 contains radio resource configuration information that is common for all Es. This information can be broadcasted here. Additionally, this information should be optional when the network cannot provide it. The NAS procedure can be delayed based on the Wait Timer only.

Proposal 3: The remaining time of S&F mode is optionally broadcasted in SIB2(-NB).

Normally it takes less than one day for a LEO/MEO satellite to circle the earth. Therefore, it should be less than one day to acquire the feeder link. The IE TimeOffsetUTC-r17, which is used to indicated t-service in Rel-17, provides the time offset to the beginning of week (Monday 00:00:00 UTC). Units in seconds. It should be more than enough.

Proposal 4: The remaining time of S&F mode is broadcasted in IE TimeOffsetUTC-r17.

Besides remaining time of S&F mode, the time information when the cell will enter the S&F mode can also be helpful. UE can trigger cell reselection despite other conditions when it is about to enter the S&F mode to look for a normal cell. Additionally, the device user can be notified of how long the feeder link will maintain available, allowing them to take the opportunity to start delay-sensitive transmissions.

Proposal 5: The time information when the cell will enter S&F mode can be broadcasted in system information.

Release Cause

It has been agreed that the SIB1 contains the S&F operation indication. The UE can be aware of the S&F mode change due to the SIB1 modification notification. It doesn’t appear necessary to inform the UE that the RRC connection release is because of switching to S&F mode, UE will know it anyway.

According to TS 24.301[3], if NAS procedure is blocked by the S&F mode, the NAS procedure will be rejected. There will be a new reject cause in NAS.

### 4.13.9 Support of Store and Forward Satellite Operation #### 4.13.9.1 General --omit-- When an MME is operating in S&F Mode: - If the MME cannot complete a NAS procedure with the information currently available on the satellite e.g. when the MME does not have UE security context or, if the MME needs to retrieve UE-specific authentication vectors or subscription information from the ground network, it shall reject the NAS procedure. In this case, if the UE supports Store and Forward Satellite operation, the MME shall include a reject cause indicating the NAS rejection is due to S&F operation.

Therefore, a new RRC release cause is not needed.

Proposal 6: New RRC release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not needed.

Satellite ID

In the study report [2], for the split MME architecture, a wait time and a list of satellite IDs will be provided to UE to re-attempt the Attach/TAU procedure.

23.700-29 v1.0.0 clause 8.2 b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

For the full CN onboard the satellite, a S&F monitoring list of satellites IDs will be optionally provided to the UE during the attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. Specifically, the S&F monitoring list may assist the UE in retrieving MT data.

23.700-29 v1.0.0 clause 8.2 - Optionally the MME provides the UE with a S&F monitoring list of satellites IDs, during attach/TAU. The UE uses the satellites in the S&F monitoring list for MO/MT data/signalling with the CN. The S&F monitoring list can be determination by the CN. How network determines the S&F monitoring list is outside the scope of 3GPP in this release of specification. NOTE 8: The S&F monitoring list may assist the UE in retrieving MT data.

NAS will need the current satellite ID to determine if it is in the satellite ID list, so that unsuccessful attach/TAU can continue in the current cell when the Wait Timer is off, or NAS can trigger connection establishment to retrieve MT data. Therefore, The RRC must notify the current satellite ID to NAS.

Proposal 7: The RRC will notify the current satellite ID to upper layer when the UE camping on the cell.

When the UE move to a S&F mode cell which is out of the satellite ID list as a result of cell selection or cell reselection, the satellite probably does not possess the UE context. UE may need to re-initiate the attach/TAU procedure, or the data transfer may not succeed due to lack the UE context. According to the latest SA2 CR S2-2501339 [4] Note B, when a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure.

### 4.13.9 Support of Store and Forward Satellite Operation #### 4.13.9.1 General --omit-- The S&F Monitoring List includes satellite(s) which belong to the same PLMN and indicates the satellite(s) that the UE may (re)attempt NAS procedures or receive MT data from. NOTE A: The S&F Wait Timer or S&F Monitoring List doesn’t affect the UE when accessing an eNodeB that does not broadcast an indication of operating in S&F Mode. NOTE B: When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. If the MME does not send a S&F Monitoring List or S&F Wait Timer in a NAS procedure, the UE deletes any stored S&F Monitoring List or S&F Wait Timer. NOTE Y: How UE behaves when receiving the S&F Monitoring List is up to UE implementation.

Therefore, if the satellite ID list is provided to RRC, UE can prioritize the cell on the satellite ID list when cell changes. This can be an optional feature without UE capability parameter.

In terms of how the satellite ID list is provided to RRC and how the UE do the prioritization, companies may have different views. According to the CR S2-2501339 [4] Note Y, how to utilize the S&F monitoring list is up to UE implementation. Therefore, we suggest that the way UE prioritize the cell in S&F monitoring list is also up to UE implementation.

Proposal 8: As an optional feature, during cell selection and cell reselection, UE can prioritize the cell that is included in satellite ID list.

Proposal 9: How UE do the prioritization is up to UE implementation.

To facilitate the UE prioritization of the cell on Satellite ID list, the satellite ID in the neighbour cell list can be used. For example, UE can search a cell on the frequency where the corresponding satellite ID is in the satellite ID list provided by NAS.

Currently, SIB31, SIB32, and SIB33 contain satellite IDs. The satellite ID in SIB31 belongs to serving cell. The satellite ID list in SIB32 and SIB33 refer to the neighbour cells.  To be able to use the satellite ID list, the satellite IDs in the SIB31, SIB32 and SIB33 must in a same name space, which is not clear according to current specification. And the satellite ID list in NAS is provided by MME belong to the same PLMN according to the CR S2-2501339 [4], hence the satellite IDs provided in SIB31, SIB32, and SIB33 are globally unique within the current PLMN.

Proposal 10: Network ensures that the satellite IDs provided in SIB31(-NB), SIB32(-NB), and SIB33(-NB) are globally unique within PLMN.

# Conclusion

S&F operation indication

Proposal 1: For Rel-19 UEs supporting S&F, if the S&F operation indication is present in SIB1, RRC will notify upper layer that this NTN cell is operating in S&F mode when camping on the cell or receiving updated SIB1 in Connected mode.

Time information

Proposal 2: For Rel-19 UEs supporting S&F, the RRC will forward the remaining time of S&F mode to the upper layer upon reception and when updated.

Proposal 3: The remaining time of S&F mode is optionally broadcasted in SIB2(-NB).

Proposal 4: The remaining time of S&F mode is broadcasted in IE TimeOffsetUTC-r17.

Proposal 5: The time information when the cell will enter S&F mode can be broadcasted in system information.

Release Cause

Proposal 6: New RRC release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable is not needed.

Satellite ID

Proposal 7: The RRC will notify the current satellite ID to upper layer when the UE camping on the cell.

Proposal 8: As an optional feature, during cell selection and cell reselection, UE can prioritize the cell that is included in satellite ID list.

Proposal 9: How UE do the prioritization is up to UE implementation.

Proposal 10: Network ensures that the satellite IDs provided in SIB31(-NB), SIB32(-NB), and SIB33(-NB) are globally unique within PLMN.

# References

RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

TR 23.700-29 v.1.0.0. Study on integration of satellite components in the 5G architecture Phase 3

TS 23.401 v19.1.0 General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access

S2-2501339, “Support of S&F Monitoring List in S&F Mode”, Huawei, HiSilicon, Samsung CR	Rel-19	23.401	5GSAT_Ph3-ARC