3GPP TSG-RAN WG2 Meeting #129	R2-2501160

Athens, GR, Feb. 17th-21st, 2025

Agenda Item:	8.8.2

Source: 	CSCN

Title: 	Discussion on Downlink Coverage Enhancements

Document for: 	Discussion and Decision

Introduction

Regarding downlink coverage enhancements in NR NTN, according to the updated WID (RP-243300) [1], the objective of the system-level enhancements is updated as follows:

- System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160ms

Furthermore, the following agreements were achieved in RAN2#128 [2].

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

2.	As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs)

3.	RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells

4.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behavior will be different from the NES states or the same). FFS on Cell DRX states.

In this contribution, we discuss system level downlink coverage enhancements in NR NTN based on the updated WID [1] and the report of RAN2#128 meeting [2].

Discussion

2.1 SMTC impacts

In the last RAN2 meeting, company mentioned to increase the number of configured SMTCs and introduce a separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs [3]. Meanwhile, RAN2 needs to investigate corresponding UE power consumption impact.

Concerning UE power consumption, an increase in the number of configured SMTCs leads to extended UE measurement time. Although the UE selects only one SSB for cell reselection, this process inevitably results in higher power consumption.

Regarding UE capability, the parameter parallelSMTC-r17 in the current spec indicates the maximum of SMTCs UE supports on a single frequency carrier is 4. It means that a new UE capability is required if introduce a separate signalling for increasing the number of configured SMTCs more than 4.

parallelSMTC-r17 Indicates whether the UE supports NTN SSB based RRM measurements on target cells belonging to 4 SMTC-s on a single frequency carrier. If a UE does not include this field but includes nonTerrestrialNetwork-r17, the UE supports NTN SSB based RRM measurements on target cells belonging to 2 SMTC-s on a single frequency carrier.

Observation 1: Increasing number of configured SMTCs will impact UE power consumption and UE capability.

To reduce the impact on UE power consumption and the requirements for UE capabilities, it is essential for the network to minimize the SMTC configuration for UEs as much as possible.

Proposal 1: To reduce the UE power consumption impact and the requirements on UE capabilities, the network should minimize the SMTC configuration for UEs as much as possible.

Network assisted UE measurement can efficiently reduce SMTC configuration. More specifically, the network could consider transmitting separate SMTC configurations to UEs in the serving cell. For example, ServingCell 0 and NeighborCell 1 to 6 are covered by a satellite shown as blow, NW sends the NeighborCell 1&2’s SMTC configurations of to UE A and NeighborCell 4&5’s to UE B in the same ServingCell 0. In this way, the UE does not need to receive SMTC configurations of all the neighbor cells, which makes UE much easier search SSB of neighbor cells.

Figure1 Network assisted SMTC configuration

Observation 2: NW assisted SMTC configuration is beneficial for UEs to search SSBs and enables NW to reduce SMTC configurations.

In fact, the network can roughly distinguish UE locations by SSB index especially in the case of DL-CE cells with beam hopping, even without knowing actual UE location in IDLE/INACTIVE mode. Furthermore, because of varying UE locations corresponding to different SSB indexes, the NW can indicate separate SMTC configurations associated with each specific SSB index of serving cell (e.g. SSB 5 associated SMTC with ServingCell 0, NeighborCell 1&2 and SSB 2 associated SMTC for ServingCell 0, NeighborCell 4&5). Therefore, SSB index associated SMTC configuration can be introduced to reduce the configuration overhead.

Proposal 2: RAN2 to discuss whether SSB index associated SMTC configuration can be supported for UE measurement.

Either UE movement or satellite movement may lead to changes in the SSB index detected by the UE. One possible scenario is that UE movement when UE is at the edge of serving SSB beam (e.g. footprint), or satellite beam sweeping over the cell (e.g. in EMC mode). Therefore, UE should reacquire the configuration to ensure the accuracy of UE measurement when UE detects the changes of SSB index.

Proposal 3: UE should reacquire the SMTC configuration when SSB index changes.

2.2 Access restriction

In the last meeting, RAN2 agreed to introduce a new barring bit in SIB1 to be able to selectively bar "UEs supporting DL-CE" if we need to bar UEs not supporting DL-CE via legacy mechanism (i.e. setting cellBarredNTN to barred), but the details are FFS (e.g. how many bits, etc.). We think one bit with the value {barred, notBarred} is sufficient to achieve this. More specifically, for UEs supporting DL-CE, UE shall follow the legacy mechanism if the new barring bit is not present, UE shall consider the cell is not allowed for access if the new barring bit is set to "barred" and allowed for access if the new barring bit is set to "not Barred".

Proposal 4: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE. The behaviors of UEs supporting DL-CE are as follows:

- Follow the legacy mechanism if the new barring bit is not present;

- Consider the cell is not allowed for access if the new barring bit is set to "barred";

- Consider the cell is allowed for access if the new barring bit is set to "notBarred".

Conclusions

Based on above analyses, proposals are summarized as follows.

Observation 1: Increasing number of configured SMTCs will impact UE power consumption and UE capability.

Observation 2: NW assisted SMTC configuration is beneficial for UEs to search SSBs and enables NW to reduce SMTC configurations.

Proposal 1: To reduce the UE power consumption impact and the requirements on UE capabilities, the network should minimize the SMTC configuration for UEs as much as possible.

Proposal 2: RAN2 to discuss whether SSB index associated SMTC configuration can be supported for UE measurement.

Proposal 3: UE should reacquire the SMTC configuration when SSB index changes.

Proposal 4: A new NTN bar bit with the value {barred, notBarred} should be introduced to achieve selectively bar UEs supporting DL-CE. The behaviors of UEs supporting DL-CE are as follows:

- Follow the legacy mechanism if the new barring bit is not present;

- Consider the cell is not allowed for access if the new barring bit is set to "barred";

- Consider the cell is allowed for access if the new barring bit is set to "notBarred".

Reference

[1] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3

[2] Report of 3GPP TSG RAN WG2 meeting #128

[3] R2-2410804, Downlink coverage enhancement SMTC impacts, Sequans Communications