3GPP TSG-RAN WG2 Meeting #129      											                         R2-2501161

Athens, Greece, Feb 17th – 25th, 2025                                                     Revision of R2-2410643

Agenda item:	8.9.4

Source:	MediaTek Inc.

Title:	Discussion on supporting PWS for NB-IoT

Document for:	Discussion and Decision

# Introduction

New objective was added to Rel-19 IoT NTN work item in RAN#105 [1]:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

This work item has been discussed in RAN2#127bis and RAN2#128 meeting, and the following agreements have been reached.

RAN2#127bis Agreements:

1.	RAN2 confirms the understanding that this WID objective covers all PWS services, including ETWS and CMAS

2.	The support for PWS introduced for NB-IoT NTN can also be made applicable for NB-IoT in TN, if this does not require additional NB-IoT TN specific changes.

3.	For NB-IoT NTN PWS, introduce the following new SIBs, taking the content within the corresponding LTE SIBs as a baseline (but also checking whether we can have some optimization/ skip some unnecessary fields):

- SystemInformationBlockType10-NB for primary ETWS notification;

- SystemInformationBlockType11-NB for secondary ETWS notification;

- SystemInformationBlockType12-NB for CMAS notification.

4.	Add the following PWS indication in direct indication information for NB-IoT (Come back in the next meeting on whether this is also added in Paging-NB):

- etws-Indication;

- cmas-Indication.

5.	Upon receiving the PWS notification from NB-IoT cell, the PWS-capable NB-IoT UE acquires the corresponding PWS message immediately (can come back to clarify further what immediately means)

6.	Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases)

RAN2#128 Agreement

- We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

In this contribution, we continue discussing the intended service area.

# Discussion

SIB1-NB acquisition

In the last RAN2 meeting, it has been agreed that we will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed.

It is clear that the SIB1-NB contains the schedulling information of system information block. UE cannot acquire SIB10/11/12 without this schedulling information.

The schedulling periodicity of SIB1-NB is 2560ms. The least NPDSCH repetition number in the 2560ms is 4, which means the UE can get a SIB1-NB at least in 640ms. The latency for receving the PWS message is acceptable.

Therefore, we think SIB1-NB acquisition after receiving the PWS notification is needed.

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

intended service area.

According to WID, support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome. The following RAN2 agreement regarding intended service area has been reached.

RAN2#126 Agreements:

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information.

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

RAN2#127 Agreements:

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2)

- For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

RAN2#127bis Agreements:

- For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

RAN2#128 Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

First of all, as it is intended in the WID, we should agree on supporting intend service area for ETWS message. Different from the terrestrial network, the satellite’s coverage can be vast, it can across borderlines and geography areas. The ETWS message carries the emergency message like earthquake and tsunami, which can be very disturbing. It may lead to widespread panic. It is beneficial to control the service area for the broadcasted ETWS message. In addition, according to SA1 replay LS S1-244857 and corresponding alignment CR S1-244753, SA1 already agreed to support intended service area for ETWS.

Primary ETWS is designed to send brief warning messages to user equipment as quickly as possible during emergencies. Its main goal is to immediately capture the user's attention, making them aware of the urgent situation. Secondary ETWS follows the initial alert by providing detailed emergency information and guidance, helping users understand the specific circumstances and the actions they should take. Together, they complement each other to form the complete ETWS warning mechanism, ensuring that during emergencies, users can swiftly receive alerts and obtain the necessary information to take appropriate measures. Therefore, only one copy of intend service area is needed for the primary ETWS and secondary ETWS. To minimize disturbance to the user, the intend service area should be provided together with primary ETWS.

Proposal 2: The intended service area is supported for primary ETWS.

Proposal 3: The intended service area is provided in SIB10-NB.

# Conclusion

Proposal 1: SIB1-NB acquisition is needed after receiving the PWS notification.

Proposal 2: The intended service area is supported for primary ETWS.

Proposal 3: The intended service area is provided in SIB10-NB.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)