3GPP TSG-RAN WG2 Meeting #129         											                     R2-2501164

Athens, Greece, Feb 17th – 25th, 2025                                                     Revision of R2-2410641

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on CB-Msg3 procedure

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we continue discussing the CB-Msg3 on details of DSA mechanism, Msg4 monitor window, RNTI, details of Msg4 and fallback.

# Discussion

Details of DSA mechanism

The CB-msg3 EDT for both SA and DSA has been agreed to be CE level specific. The number of levels and whether the same thresholds can be used require further discussion.

RAN2#128 Agreement: - Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels.

There are 3 CE levels for legacy NB-IoT and 4 CE levels for legacy eMTC. A set of PRACH resources (e.g., time, frequency, and preamble) is provided in the SIB, with each resource associated with a specific CE level. The number of PRACH repetitions and number of maximum preamble transmission attempts per CE level are also provided in SIB. UEs in same CE level use random access resources associated with the that CE level.

The main difference in CB-Msg3 is skipping of Msg1 and Msg2. However, CE level concept can still apply to CB-Msg3. A good DL RSRP level can reduce the PUSCH repetition number while a poor DL RSRP level will need to increase PUSCH repetition number to meet the link budget. The radio conditions for legacy RA and the CB-Msg3 are the same, the number of sets of PUSCH resource for CB-Msg3 can also be the same as the legacy. Additionally, there is no reason to introduce any changes to the thresholds for CE levels.

Proposal 1: CB-Msg3 can use the same number of CE levels and same thresholds as those of legacy.

The configured maximum TBS for CB-Msg3 has been agreed during the RAN2#128. Whether the maximum TBS is CE level specific is yet to be determined. The maximum TBS is highly related to the radio resource. Moreover, the radio resource is highly related to the repetition number which varies by the CE level. Clearly, the maximum TBS shall be specific to CE level.

RAN2#128 Agreement: - The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

Proposal 2: The maximum TBS CE level specific.

Regarding the DSA mechanism for CB-Msg3, it has been agreed that a configurable time window for selecting DSA occasion will be introduced. Within this time window, the UE will randomly select PUSCH resource in both time and frequency domains.

RAN2#128 Agreement: - There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts) - Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

First, the UE can select the nearest PUSCH occasion to transmit the first replica after the CB-Msg3 has been initiated. Once the UE has completed the transmission and there are more replicas to transmit as configured, the UE can start the time window. The exact start point can be the end of the PUSCH occasion.

The time window length will be affected by the length of PUSCH resources. Hence, this configuration parameter should also be specific to CE levels.

Proposal 3: The time window for CB-Msg3 starts at the end of the first replica’s transmission.

Proposal 4: The time window length for CB-Msg3 is specific to CE levels.

Msg4 monitor window

In the last RAN2 meeting, the PDCCH monitoring window was agreed for the CB-msg3 PUSCH transmission. For DSA, whether we have one or multiple PDCCH monitoring window(s) is still to be determined.

RAN2#128 Agreement: - For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time - For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs) - The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

When the DSA approach is applied, there will be multiple replicas to be transmitted. The UL resources in terms of the time domain and/or frequency domain for replicas should be different.  If only one PDCCH monitoring window is used, then the RNTI used for monitoring Msg4 will be the same for the entire PDCCH monitoring window. However, if the RNTI is derived based on the resource associated with first replica of PUSCH transmission, and if the network cannot decode that replica, the network will not know the RNTI for the remaining replicas. Therefore, the RNTI for Msg4 monitoring must be derived based PUSCH resource associated with the corresponding replica. Since the RNTIs of replicas are different, it is clear that multiple PDCCH monitoring windows should be used for DSA.

When the PDCCH monitoring window of replica A is initiated, the next replica B transmission may overlap within this window. For a half-duplex system, UE cannot simultaneously receive and transmit. The UE will not be able to monitor PDCCH while transmitting a replica. However, after completing the replica B transmission, the UE must continue monitoring PDCCH for replica A. Therefore, the PDCCH monitoring window must not stop due to subsequent replica transmissions.

For the potential conflicts between Msg4 scheduling and replica transmissions, the UE cannot address this issue. The network can detect this conflict situation and avoid it by delaying the corresponding Msg4 scheduling.

Proposal 5: UE starts PDCCH monitoring window for each replica (i.e. multiple monitoring windows). It does not stop when UE start to transmit next replica, but the UE does not monitor PDCCH when transmitting the replica.

Proposal 6: Network should try to avoid the potential collision with the replica transmission and the Msg4 scheduling.

RNTI

A new RNTI is needed for monitoring PDCCH for Msg4. RAN2 has agreed that the RNTI is derived based on the resources associated with the PUSCH occasion used for the contention based Msg3 EDT transmission.

RAN2#127bis Agreements:

The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

For the network, the new RNTI is used to scramble the PDCCH for the Msg4 transmission. UE also needs it to monitor PDCCH. How the RNTI is calculated is determined by how the Msg4 is organized.

If there is no DL response to the UL EDT transmission, there will be only a contention resolution MAC CE in the Msg4. So that there will be enough room left in a MAC PDU for other information. The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU. The signalling overhead efficiency will be improved accordingly.

Therefore, the time domain information (e.g., subframe index, SFN) will be part of formulation of new RNTI. The frequency domain information (e.g., subcarrier index) will not.

Proposal 7a: The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU.

Proposal 7b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for new RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Multi-carrier is supported in NB-IoT since Release 13. This feature becomes even more critical in NTN scenario where capacity may be a significant challenge due to extensive cell coverage. For CB-Msg3, multi-carrier support should also be implemented.

Proposal 8a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.

Additionally, since the carrier-level UL/DL resource can be provided in NB-IoT, UEs can send UL transmissions on different UL carriers and then monitor DL responses on the same DL carrier. To differentiate Msg4s responding to different UL carriers, the carrier index is needed in the formulation of new RNTI.

Proposal 8b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for new RNTI calculation.

Details of Msg4

Like legacy EDT, for some CB-Msg3 transmission, especially CP solution, the RRC message in Msg4 is not mandatory. In this case, a Msg4 with only MAC CE(s) should be allowed. It is referred to as L2 ACK.

Proposal 9: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Since the contention resolution is essential and cannot be done via DCI due to the size limitation, using the L1 ACK as the Msg4 for CB-Msg3 is not appropriate.

Proposal 10: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

Fallback

Same as legacy EDT, in CB-Msg3, network should be able to steer UE to RRC connected mode when there is sequential DL transmission. This is referred as fallback procedure.

Unlike EDT, the RAR that contains the Timing Advance Command is skipped in CB-Msg3. To maintain better UL timing advance in RRC connected mode, a Timing Advance Command MAC CE can be included in the fallback Msg4.

There is no RAR to start the time alignment timer, it is acceptable if the UE does not move to RRC connected mode. The UE must start the time alignment timer when receiving Msg4 which indicates fallback.

Proposal 11: A TAC MAC CE can be included in the Msg4 when fallback.

Proposal 12: UE starts the time alignment timer when receiving Msg4 which indicates fallback.

A new RNTI based on PUSCH resource is used to monitor Msg4. This RNTI cannot be used as the C-RNTI in RRC connected mode as this RNTI can be reused by another UE which selects the same PUSCH resource. Therefore, a new C-RNTI should be assigned by network when fallback. Normally it can be carried by RRC message. But if the contention resolution MAC CE and RRC message can be sent in different MAC PDU (i.e., early contention resolution is supported for CB-Msg3), the C-RNTI should be carried by MAC CE together with contention resolution MAC CE.

Proposal 13: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback.

# Conclusion

Details of DSA mechanism

Proposal 1: CB-Msg3 can use the same number of CE levels and same thresholds as those of legacy.

Proposal 2: The maximum TBS CE level specific.

Proposal 3: The time window for CB-Msg3 starts at the end of the first replica’s transmission.

Proposal 4: The time window length for CB-Msg3 is specific to CE levels.

Msg4 monitor window

Proposal 5: UE starts PDCCH monitoring window for each replica (i.e. multiple monitoring windows). It does not stop when UE start to transmit next replica, but the UE does not monitor PDCCH when transmitting the replica.

Proposal 6: Network should try to avoid the potential collision with the replica transmission and the Msg4 scheduling.

RNTI

Proposal 7a: The Msg4s responding to the Msg3s from the same time domains but the different frequency domains can be included in the same MAC PDU.

Proposal 7b: The time domain information of CB-Msg3 UL resource (e.g., subframe index, SFN) is taken into account for new RNTI calculation. The frequency domain information (e.g., subcarrier index) is not.

Proposal 8a: For NB-IoT, CB-Msg3 transmission on non-anchor carrier shall be supported.

Proposal 8b: For NB-IoT, the carrier index of CB-Msg3 UL resource is taken into account for new RNTI calculation.

Details of Msg4

Proposal 9: A Msg4 for CB-Msg3 with only MAC CE(s) is allowed when RRC message is not mandatory.

Proposal 10: Do not support the use of L1 ACK as the Msg4 for the CB-Msg3.

Fallback

Proposal 11: A TAC MAC CE can be included in the Msg4 when fallback.

Proposal 12: UE starts the time alignment timer when receiving Msg4 which indicates fallback.

Proposal 13: New C-RNTI should be included in the Msg4 for the CB-Msg3 when fallback.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)