3GPP TSG-RAN WG2 Meeting #129	R2-2501168

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.3

Source:	Samsung

Title:	Remaining Issues of L1 Event Triggered Measurement Report

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

Based on agreements RAN2 made, event-triggered L1 measurement enhancements for LTM has implemented in the MAC/RRC running CRs [1][2]. In addition, RAN2 had an offline discussion for RRC issues [3] and made a lot of proposals to implement the RAN2 agreements into the RRC running CR [2].

In this contribution, we will further address the remaining issues on the event triggered L1 measurement reporting in detail.

Discussion

In the MAC running CR [1], L1 Event Triggered Measurement Report MAC CE format with field descriptions are provided, and the basic rule for this format is following the legacy CSI report for LTM in 38.212, see below table.

Table 6.3.1.1.2-8C: Mapping order of CSI fields of one report for SSBRI/RSRP reporting for L1/L2-triggered mobility

| CSI report number | CSI fields |
| --- | --- |
| CSI report #n | SSBRI #1 as in Table 6.3.1.1.2-6, if reported |
|  | SSBRI #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | SSBRI # L×M as in Table 6.3.1.1.2-6, if reported |
|  | RSRP #1 as in Table 6.3.1.1.2-6, if reported |
|  | Differential RSRP #2 as in Table 6.3.1.1.2-6, if reported |
|  | … |
|  | Differential RSRP # L×M as in Table 6.3.1.1.2-6, if reported |
| NOTE: L is the number of reported cells provided by higher layer parameter nrOfReportedCells and M is the number of reported SSBRI/RSRP pairs per cell and equal to the value provided by higher layer parameter nrofReportedRS-PerCell. |  |

It means the index of measured resources (i.e. reference signalling resource index; RSRI) and the measured values (i.e. RSRP and Differential RSRP) are used in Rel-18 CSI reporting. In differential RSRP reporting format the first listed RSRP value is in absolute format (7 bit) and other listed values are in differential format (4 bit) relative to the absolute RSRP value. We think it would be better to align the Rel-19 MAC CE format similar as this format (i.e. example of MAC CE format was provided in [1]). Then we can save some signalling bits by using the differential RSRP instead of the explicit RSRP value for the measured beams.

Proposal 1: The differential RSRP values based on the first reported beam are used for the following measured beam results in MR MAC CE.

In the MAC running CR [1] for the enhanced mobility Ph4 WI, LTM event evaluation procedure is specified in 5.x.3 and there is an Editor’s note how the LTM event (report ID) and the L1 measurements are associated.

Editor’s NOTE: The structure for this clause could be further discussed/updated based on companies’ view. There are two methods to capture this part. Method#1: for each reportID (associated with event), report is triggered if any beam (associated with this reportID) fulfils the event by evaluating all beams during TTT for each beam. Method#2: for each beam, report is triggered if any event (from the event associated with this beam) associated with reportID fulfils the event by evaluating all corresponding events during TTT associated each event. Method#1 is adopted as below. Companies are invited to provide views on which method is preferred.

From our understanding, RAN2 assumes there is the L1 resource configuration which is associated with each report configuration (report ID) i.e. Method #1 based on earlier RAN2 agreements. In this method, each event (per report ID) could be evaluated separately and those measurement results of triggered events could be simultaneously reported to the NW.

Proposal 2: RAN2 confirms for each reportID (associated with event) the measurement report is triggered if any beam (associated with this reportID) fulfils the event by evaluating all beams during TTT for each beam.

Another issue for measurement report triggering is how the measured values are handled in the MAC entity for L1 event triggered measurement and report procedure. In the MAC running CR, the new UE variables for UE to maintain the measured/evaluated measurements during the LTM event triggered measurement procedure, see below UE variables excerpted from [1].

-	VarMeasReportList: includes the L1 event triggered measurement report information, for which the L1 measurement report triggering conditions have been met;

-	numberOfReportsSent: represents the number of L1 event triggered measurement report performed by UE if the triggering condition for the corresponding event is met for each ltm-CSI-ReportConfigId;

-	VarBeamsTriggeredList: includes the reference signaling resource index of LTM candidate cell(s) for each ltm-CSI-ReportConfigId, for which the L1 measurement report triggering conditions have been met.

All LTM measurement report triggering and reporting procedures in MAC is the new procedure and this procedure refers the Layer3 measurement triggering and reporting procedure in RRC specification. We think these new MAC variables which includes multiple entries (i.e. list) in each variable is necessary if we follows the Layer 3 measurement triggering and reporting procedure in RRC specification.

Proposal 3: RAN2 introduce the new UE variables (e.g. VarMeasReportList, NumberOfReportsSent, VarBeamsTriggeredList) in MAC entity for the measurement triggering and reporting procedure.

In Rel-18 TEI, RAN2 introduced the enhancements to measurement report features which handles the reportOnBestCellChange, firstEntering and cellsMetReportOnLeaveList [4]. We think it is also good for RAN2 to discuss to apply this enhancements in LTM event triggered measurement report.

Based on the measurement reporting format in MAC CE, the first reported beam has the highest RSRP values and the following measured beam values could use the differential RSRP compared with the first reported beam. Then we think sorting the beams in UE variables are needed and it could be specified in the MAC procedures. In addition, there are two types of triggering (i.e. entry condition and leaving condition) for each report ID, and how the VarMeasReportList is handled is specified in MAC running CR for both entry condition and leaving condition. However, there are no ways for gNB to identify this event is triggered by entry or leaving conditions when gNB receives the MR MAC CE and there are no UE variables for the beam list met leaving condition. We think at least some indication that the event is triggered by the leaving condition is helpful to gNB.

Proposal 4: RAN2 discuss to support the best triggered beam change in UE variables (i.e. VarBeamsTriggeredList) and need of BeamsMetReportOnLeaveList.

Proposal 5: Introduce the 1 bit indication that leaving condition is met for the event in MR MAC CE.

Based on the L1 event triggered measurement report procedure, it is possible that the multiple L1 events are simultaneously triggered then how to report these events are considered.

- Option 1: LTM MR MAC CE could include multiple triggered measurement events.

- Option 2: LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

We think it is possible to merge all triggered event in an LTM MR MAC CE since the truncated MR MAC CE will be introduced in this feature but we prefer the simple approach in terms of UE implementation.

Proposal 6: LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: The differential RSRP values based on the first reported beam are used for the following measured beam results in MR MAC CE.

Proposal 2: RAN2 confirms for each reportID (associated with event) the measurement report is triggered if any beam (associated with this reportID) fulfils the event by evaluating all beams during TTT for each beam.

Proposal 3: RAN2 introduce the new UE variables (e.g. VarMeasReportList, NumberOfReportsSent, VarBeamsTriggeredList) in MAC entity for the measurement triggering and reporting procedure.

Proposal 4: RAN2 discuss to support the best triggered beam change in UE variables (i.e. VarBeamsTriggeredList) and need of BeamsMetReportOnLeaveList.

Proposal 5: Introduce the 1 bit indication that leaving condition is met for the event in MR MAC CE.

Proposal 6: LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Introduction of event-triggered measurement reporting for RRC spec	Huawei	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[3]	R2-250xxxx		Summary of [POST128][108][MOB] RRC running CR (Huawei)	Huawei.

[4]	R2-2406021		Enhancements to measurement report [meas_report_enh]	TEI18		38.331	4803	Rel-18.