3GPP TSG-RAN WG2 Meeting #129	R2-251169

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.4

Source:	Samsung

Title:	Remaining issues of Conditional Intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

Introduction

In RAN2#127bis and 128 meetings, RAN2 made following agreements to support the Conditional intra-CU LTM, and RAN2 prepared the MAC/RRC running CRs based on these agreements.

Agreements on C-LTM in RAN2#127bis

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Agreements on C-LTM in RAN2#128:

1.	The triggering condition of conditional LTM can be based on L3 measurement.

2.	CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

3.	The L1 execution condition of a candidate cell is associated to only one triggering event.

4.	For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

5.	To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

6.	The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

7.	For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

8.	The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

9.	The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

10.	Candidate cell TA is maintained by a new timer.

11.	For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

12.	For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

In this contribution, we will further address remaining issues to support the conditional intra-CU LTM in detail.

Discussion

Issue 1: Beam(s) used for C-LTM event evaluation

For beam selection of the current beam (source cell) for event evaluation (execution condition), we think the conditional LTM is slightly different with the Rel-19 LTM principle. For the event triggered measurement report, the current beam to evaluate the pre-defined event is the NW indicated beam which is based on the L1 measurement report. Based on this indicated TCI state, UE measure the beams for the candidate cells and evaluate the event, then UE report the measurement results if any event condition is met.

However, it is not convinced that the indicated TCI state is always the best beam, the indicated TCI state could be determined based on the CSI report (i.e. legacy L1 report). If this CSI measurement configuration is configured frequently to both LTM and CLTM, the performance of those feature would be same, but I think there are some real cases that this TCI indication is not frequently provided because of the rare CSI report form the UE. Again, the indicated beam from NW would be based on the measurement report from UE but it is possible that UE could not report measurement report and trigger the conditional LTM directly based on the best beam measurement. So, it is more reasonable that UE selects the current beam to evaluate the execution condition. This current beam could be the best beam in the source cell by UE measurement or any beam indicated by the TCI state activation MAC CE).

- Option 1: Current beam of the source cell could be the indicated beam (TCI state).

- Option 2: UE could select the best beam for the source cell to evaluate the execution condition. (or use the any beam which is activated by Candidate cell TCI states activation/deactivation MAC CE)

We think above Option 2 is an optimization for both LTM and CLTM but it is more easily applied to the CLTM without further specification impacts.

Proposal 1: UE could select the best beam for the source cell to evaluate the execution condition i.e. use either the best beam or any beam which is activated by TCI states activation/deactivation MAC CE.

Similar as current beam selection for the conditional LTM, there are issues related with the target beam: 1) how many target beams are used for evaluation 2) which TCI state is used after cell switch by the conditional LTM.

For L1 measurement based CLTM, we think each execution condition is evaluated per candidate beam because it is simple and we see no critical reason to enhance the L1 CLTM by applying the consolidated quality of multiple beams of candidate cell.

Proposal 2: UE evaluates the L1 CLTM execution condition for candidate beam and triggers the LTM cell switch procedure when at least one beam meets the associated execution condition.

In Rel-18 LTM cell switch command MAC CE, the TCI state information is mandatory so UE should use this TCI state for the LTM target cell (i.e. the SpCell of the target configuration indicated by the Target Configuration ID field). However, there are no signaling to indicate the activated TCI state for data tx/rx in the target cell after cell switch for the conditional LTM. We think there are new rules for selecting the beam after cell switch case by case as below.

- For RACH based C-LTM, latest beam reported to network is the beam indicated during the RA procedure. So this should be used until a new TCI state is explicitly activated by network after cell switch.

- For RACH less C-LTM, UE will select beam (basically best beam or beam above threshold at the time of UL transmission) for CG resource selection. CG resources are mapped to beams and based on CG resource in which network receives the first UL transmission, network will know the beam and this should be used until a new TCI state is explicitly activated by network after cell switch.

Proposal 3: UE selects the beam will be used after the cell switch for CLTM as below:

- For RACH based C-LTM, the beam indicated during the RA procedure is used until a new TCI state is explicitly activated by network after cell switch.

- For RACH less C-LTM, UE will select beam for CG resource selection and this beam is used until a new TCI state is explicitly activated by network after cell switch.

Issue 2: Whether to support RACH-less LTM with DG

We think the CG based RACH-less conditional LTM is the baseline as RAN2 already agreed in the last meeting. In Rel-18, RACH-less LTM introduced both the CG-based first UL transmission and DG-based first UL transmission. For the DG-based approach, the NW has to know when the UE monitor the PDCCH to reduce the resource waste, otherwise, network send the PDDCH even there are no UE in the cell area. In the conditional LTM, NW has no idea on the conditional LTM i.e. which cell UE triggered conditional LTM, when UE triggers the conditional LTM.

Proposal 4: For RACH-less conditional LTM, DG-based first UL transmission on target cell is not supported.

Issue 3: Security aspects

Since the scope of Rel-19 CLTM is limited to intra-CU, no security key change issues are foreseen. It means that the master key change is not supported and this restriction could be added in the specification similar as Rel-18 LTM.

Proposal 5: For intra-CU conditional LTM, master key change is not supported.

Issue 4: Corrections in the MAC running CR

In the MAC running CR [1], L1 LTM events associated with CLTM are defined using eventLTM3 and eventLTM5 which means the same execution conditions used for L1 event triggered measurement report are used CLTM as well. However, we think it would be better to introduce the separate execution conditions for CLTM as legacy L3 CHO (e.g. condEventLTM3 /condEventLTM5, like condEventA3/A5 for CHO).

Proposal 6: For CLTM L1 execution conditions, separate execution conditions for CLTM compared with the L1 event triggered measurement report are introduced (e.g. condEventLTM3 /condEventLTM5, like condEventA3/A5 for CHO).

There is an FFS regarding how UE selects the CLTM event if there are multiple events meet the execution condition simultaneously. We think the simple solution without specification impact is just follow the UE implementation

Editor’s NOTE: FFS on the case that multiple beams satisfy the event for CLTM. E.g. if multiple beams satisfy the event for CLTM, it is UE implementation to select one of them to indicate to upper layer.

Proposal 7: UE could select any CLTM event which meet the execution condition if multiple CLTM events are triggered simultaneously.

Issue 5: Corrections in the RRC running CR

CLTM supports both L1 and L3 execution condition. In practice, both execution conditions may not be fulfilled at the same time. Whenever either condition is satisfied, the UE can perform the cell switch. This is similar to normal LTM, i.e., either L1 and L3 measurement results can trigger the cell switch command transmission. Thus, the NW can configure the both L1 and L3 execution conditions to the UE. Companies may have concerns on the UE complexity on the execution condition evaluation since the UE may need to perform both L1 and L3 evaluations. This may not be a big problem, it can be up to UE implementation to determine whether to perform evaluation of both conditions.

Proposal 8: RAN2 is kindly asked to agree that both L1 and L3 execution condition can be configured to the UE simultaneously.

As indicated by WID, the CLTM is limited to the non-DC case. Thus, the CLTM for SCG is not supported in Rel-19.

Proposal 9: RAN2 is kindly asked to confirm that CLTM for SCG is not supported in Rel-19.

In the RRC running CR, if CLTM is configured, all of the configured LTM candidates are configured as CLTM candidates. As Rel-19 supports only Intra-CU CLTM, this will cause issues if Intra-CU CLTM is supported along with Inter-CU LTM. So we would first like to check RAN2 understanding on whether this configuration is supported.

Proposal 10: RAN2 is kindly asked to discuss if Inter CU LTM + Intra-CU CLTM is supported in R19.

If Inter CU LTM + Intra-CU CLTM is a supported scenario, UE need to perform the measurements and evaluations for CLTM only when the CLTM candidate is Intra-CU cell. We can reuse the existing ltm-ServingCellNoSecurityChangeID-r19/ ltm-NoSecurityChangeID-r19 to identify if the CLTM candidate is intra-CU candidate.

Proposal 11: If Inter CU LTM + Intra-CU CLTM is supported, UE performs CLTM measurements/evaluation when the ltm-NoSecurityChangeID of the candidate is same as that of ltm-ServingCellNoSecurityChangeID.

Even for Intra-CU CLTM, when CLTM is configured, it is not clear whether all the LTM candidates need to be always CLTM candidates also. Allowing a mix of LTM candidates and CLTM candidates will increase the flexibility of network deployment. For e.g. some candidate DUs in a CU can support only R18 LTM while some DUs can support CLTM along with R18 LTM. This may also help the UE to reduce unnecessary evaluations, and will be also similar to the legacy Layer3 conditional mobility where every neighbour cell needn’t be a CHO candidate. On the other hand, with the current RRC structure and procedures for CLTM in the running CR, this may need some additional ways to distinguish between CLTM candidate cells and candidates which are only LTM candidate cells in the ltm-Config, either by using  CLTM-ExecutionConditions  IE or by using a new flag. So RAN2 is requested to discuss it first.

Proposal 12: RAN2 is kindly asked to confirm if it is possible to have candidates supporting only LTM when CLTM is configured.

Conclusion

Based on the discussion, we have the following proposals.

Proposal 1: UE could select the best beam for the source cell to evaluate the execution condition i.e. use either the best beam or any beam which is activated by TCI states activation/deactivation MAC CE.

Proposal 2: UE evaluates the L1 CLTM execution condition for candidate beam and triggers the LTM cell switch procedure when at least one beam meets the associated execution condition.

Proposal 3: UE selects the beam will be used after the cell switch for CLTM as below:

- For RACH based C-LTM, the beam indicated during the RA procedure is used until a new TCI state is explicitly activated by network after cell switch.

- For RACH less C-LTM, UE will select beam for CG resource selection and this beam is used until a new TCI state is explicitly activated by network after cell switch.

Proposal 4: For RACH-less conditional LTM, DG-based first UL transmission on target cell is not supported.

Proposal 5: For intra-CU conditional LTM, master key change is not supported.

Proposal 6: For CLTM L1 execution conditions, separate execution conditions for CLTM compared with the L1 event triggered measurement report are introduced (e.g. condEventLTM3 /condEventLTM5, like condEventA3/A5 for CHO).

Proposal 7: UE could select any CLTM event which meet the execution condition if multiple CLTM events are triggered simultaneously.

Proposal 8: RAN2 is kindly asked to agree that both L1 and L3 execution condition can be configured to the UE simultaneously.

Proposal 9: RAN2 is kindly asked to confirm that CLTM for SCG is not supported in Rel-19.

Proposal 10: RAN2 is kindly asked to discuss if Inter CU LTM + Intra-CU CLTM is supported in R19.

Proposal 11: If Inter CU LTM + Intra-CU CLTM is supported, UE performs CLTM measurements/evaluation when the ltm-NoSecurityChangeID of the candidate is same as that of ltm-ServingCellNoSecurityChangeID.

Proposal 12: RAN2 is kindly asked to confirm if it is possible to have candidates supporting only LTM when CLTM is configured.

References

[1]	R2-250xxxx		Running MAC CR for enhanced mobility Ph4	vivo	38.321	18.4.0		NR_Mob_Ph4-Core	Rel-19.

[2]	R2-250xxxx		Rapporteur RRC for inter-CU and conditional LTM	Ericsson	38.331	18.4.0		NR_Mob_Ph4-Core	Rel-19.