3GPP TSG-RAN-WG2 Meeting #129	R2-2501183

Athens, Greece, 17th– 21st Feb, 2025

Agenda item	:		8.13.2

Source	: 	Sharp

Title	: 	discussion on Relay discovery and (re)selection for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 reached following agreements [1];

Agreements: - A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. - If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. - The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. - The following cases are supported for L2 multihop relay: - - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. - FFS if the last relay UE can use the same L2ID for both of the connections in either case. - Cases with two indirect paths to the gNB for the same remote UE are not supported. - An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. - RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

  - Discovery transmission

According to the SA’s potential solution, the discovery procedure has the following characteristics;

- Model A: discovery message is firstly transmitted by “last relay UE.” And, similar to U2U relay discovery, intermediate UEs forward the discovery to the remote UE.

- Model B: discovery message is firstly transmitted by “remote UE.” If first relay UE is last relay UE, first relay UE responses to discovery message directly, else, similar to U2U relay discovery, first relay UE forwards the discovery message to the other relay UE and then responses to discovery message when receives response message from the other relay UE.

Based on this, we need to consider following aspects;

- The condition under which the last relay UE initiates discovery Model A

- The condition under which the intermediate relay UEs forward discovery message for Model A

- The condition under which the remote UE initiates discovery Model B and/or relay (re)selection

- The condition under which the intermediate relay UEs forward discovery message for Model B

- The condition under which the last relay UE responds to discovery message for Model B

For discovery transmission conditions, RAN2 reached following agreements;

If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18.

If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds).

FFS discovery conditions for the intermediate relay UEs.

For the condition under which the last relay UE initiates discovery Model A, it was already agreed as above, i.e. the condition is the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold.

For the condition under which the intermediate relay UEs forward discovery message for Model A, it is still FFS. The situation of intermediate relay UE is similar with U2U relay UE during model B discovery. Therefore, we can reuse the condition, i.e. intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

Proposal 1. Intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

SA2 sent LS reply to RAN2 [2] on whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message. The answer is:

- In Discovery model A, intermediate relay establishes PC5 connection to a potential parent UE before forwarding the announcement message.

- In Discovery model B, intermediate relay doesn’t establish PC5 connection to a potential parent UE before forwarding the response message.

We think it is related to the forwarding condition, and establishment of the PC5 connection is not needed if the PC5 RSRP is not well since the connection may not be used for multi-hop relay.

Proposal 2. Intermediate relay UE doesn’t establish PC5 connection to a potential parent UE if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself isn’t above a threshold.

For the condition under which the remote UE initiates discovery Model B, it was already agreed as above, i.e. the condition is the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell. Furthermore, we want to clarify that there is no PC5 RSRP condition of remote UE for discovery transmission since remote UE should be able to reselect other first relay UE to reduce a number of hops.

Proposal 3. Remote UE is not configured with PC5 threshold for model B discovery transmission.

For the condition under which the intermediate relay UEs forward discovery message for Model B, it is still FFS. As mentioned above, for the PC5 condition, RAN2 should reuse U2U relay mechanism, i.e. Intermediate relay UE forwards solicitation message of model B if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

Proposal 4. Intermediate relay UE forwards solicitation message of model B if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

For the condition under which the last relay UE responds to discovery message for Model B, it was discussed in the email discussion. In Rel-17/18, PC5 AS condition for the last relay UE (i.e. PC5 link quality between the remote UE and the last relay UE) can be evaluated by the remote UE since it was single-hop. However, in multi-hop case, the remote UE cannot evaluate PC5 AS condition for the last relay UE. For discovery model A, PC5 AS condition is NOT needed since intermediate relay UE which directly receives discovery announce message from the last relay UE can evaluate PC5 AS condition for the last relay UE when the intermediate relay UE determines whether to forward it. However, for discovery model B, PC5 AS condition is needed since there is no UE which evaluates PC5 link quality between the intermediate relay UE and the last relay UE if the relay UE doesn’t evaluate PC5 AS condition. This is similar idea with the agreement of R18 U2U relaying; “For Model-B discovery, after receiving a discovery message from a relay UE, a target remote UE transmits the discovery response message only if the PC5 RSRP between the target remote UE and the relay UE is above a configured threshold.” Therefore, we think PC5 AS condition is needed for Model B discovery at the last relay UE.

Proposal 5. Last relay UE responds to solicitation message if PC5 RSRP between UE transmitted the solicitation message to the last relay UE and itself is above a threshold.

RAN2 agreed that;

The following cases are supported for L2 multihop relay:

- One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE).

- Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

FFS if the last relay UE can use the same L2ID for both of the connections in either case.

For the FFS part, we understood that the L2 ID is used per unicast link. And the unicast link is established per RSC even if the UEs are same. Therefore, in the former case, the same ID cannot be used, but in the latter case, the same ID may be used (it depends on RSC).

Observation 1. L2 ID is used per unicast link and the unicast link is established per RSC.

Proposal 6. RAN2 confirms that; the last relay UE cannot use the same L2ID in a case that one last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE), and the last relay UE can use the same L2ID in a case that two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

And it is FFS if there would be additional restrictions for the intermediate relay UE in the case of no upper bound.

Proposal 7. There are no additional restrictions to act as an intermediate relay UE in the case of no upper bound.

RAN2 already agreed that the multi-path topology should not be supported in multi-hop relay. In that case, remote UEs and intermediate relay UEs should use the SIBs broadcasted on the last relay UE’s serving cell. In such a case, to prevent mis-configuration due to multipath topology, the last relay UE's child UEs need to know the last relay UE's serving cell. In Rel-17 relay discovery, the RRC container is included in the discovery message, and the cell-related information is included in the RRC container. In multi-hop relay, based on the discovery message received from the last relay UE, the corresponding cell-related information needs to be included in the discovery message to be transmitted.

Proposal 8. When the UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

Conclusion

Proposal 1. Intermediate relay UE forwards discovery message of model A if PC5 RSRP between UE transmitted the discovery message to the intermediate relay UE and itself is above a threshold.

Proposal 2. Intermediate relay UE doesn’t establish PC5 connection to a potential parent UE if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself isn’t above a threshold.

Proposal 3. Remote UE is not configured with PC5 threshold for model B discovery transmission.

Proposal 4. Intermediate relay UE forwards solicitation message of model B if PC5 RSRP between UE transmitted the solicitation message to the intermediate relay UE and itself is above a threshold.

Proposal 5. Last relay UE responds to solicitation message if PC5 RSRP between UE transmitted the solicitation message to the last relay UE and itself is above a threshold.

Observation 1. L2 ID is used per unicast link and the unicast link is established per RSC.

Proposal 6. RAN2 confirms that; the last relay UE cannot use the same L2ID in a case that one last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE), and the last relay UE can use the same L2ID in a case that two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE.

Proposal 7. There are no additional restrictions to act as an intermediate relay UE in the case of no upper bound.

Proposal 8. When the UE transmits discovery message, the UE includes the cell access info received in parent UE’s discovery message.

Reference

- R2-2410914, Report from session on positioning and sidelink relay

- R2-2500068, Reply LS on relay discovery announcement