3GPP TSG-RAN-WG2 Meeting #129	R2-2501184

Athens, Greece, 17th– 21st Feb, 2025

Agenda item	:		8.13.3

Source	: 	Sharp

Title	: 	discussion on C-plane procedure for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#128 meeting, RAN2 reached the following agreements [1];

Agreements: In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE. The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted. In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED. In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection. The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted. For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation. This does not preclude the possibility of assistance information from the UE(s). If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network. FFS how to split the PDB over each of the individual remaining hops. In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s). FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE). Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell. 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. All agreements in this WI apply to both these cases unless otherwise specified. Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel. This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs. Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

In this paper, we discuss how to introduce multi-hop relay.

Discussion

SA2 sent an LS [2] related authorization. In the LS, there are 3 types of authorization for each UE type, i.e. last relay UE, intermediate relay UE and remote UE. It means that, depending on each UE type, specific actions can be taken.

Observation 1. Each UE (last relay UE, intermediate relay UE, and remote UE) can perform different actions based on the authentication information.

In the email discussion [3], CR rapporteur suggests that RAN2 discusses/confirms that if multiple remote UEs connected to single intermediate relay UE, the multiple remote UEs must connect to the same cell. And it is FFS if operation as an intermediate relay UE towards a different cell would be supported. One example of such topology is below:

Figure 1. One example of multi-hop links with an intermediate relay UE towards a different cell

If the last relay UEs support different RSCs, this scenario might be considerable. However, in this case, intermediate relay UE needs to use a SIB of the one cell, have one virtual PCell, etc. In other words, operation must be performed according to the cell to which the last relay UE belongs. Additionally, this is a multipath scenario as well. RAN2 decided not to support multi-path scenario in multi-hop. Therefore, we propose that if multiple remote UEs connected to a single intermediate relay UE, the multiple remote UEs must connect to the same cell.

Proposal 1. If multiple remote UEs connected to a single intermediate relay UE, the multiple remote UEs must connect to the same cell.

  - Configuration for multi-hop relaying

RRC connection maintenance

RAN2 needs to discuss RRC connections between UEs and NW, and PC5 RRC connections between UEs. In the last meeting, RAN2 reached following agreements;

Agreements:

The following connections are assumed as a baseline to be needed:

-	From last Relay UE perspective:

A direct (non-relayed) PC5 connection with the first or an intermediate Relay UE, and

A direct (non-relayed) Uu connection with serving gNB, if in RRC_CONNECTED.

-	From intermediate relay UE perspective (including first relay UE):

A direct (non-relayed) PC5 connection with each of two adjacent (remote or relay) UEs, and

An end-to-end Uu connection with serving gNB, if in RRC_CONNECTED.

FFS what RRC states are supported for the intermediate relay UE.

-	From U2N Remote UE perspective:

A direct (non-relayed) PC5 connection with Intermediate Relay UE, and

An end-to-end Uu RRC connection with serving gNB, if in RRC_CONNECTED.

According to the agreements, we can illustrate the connections as shown in figure 1.

Figure2. RRC connections between UEs and between UEs and gNB for two-hop U2N relay

Next, RAN2 needs to discuss whether the legacy U2N relay mechanism can work on this architecture.

In this phase, we should discuss whether the current mechanism can be reused. We discuss following contents in this paper;

- Paging delivery

- SIB delivery

- RRC connection establishment

- SRAP configuration for C-plane and U-plane

- QoS handling

- Failure handling

Firstly, current SIB delivery is supported by following mechanism [7];

- The remote UE in RRC_IDLE or RRC_INACTIVE can inform the interested SIB(s) to the connected relay UE and receive the SIB(s) from the relay UE via PC5-RRC message.

- The remote UE in RRC_CONNECTED receives SIB1 and other SIBs by dedicated RRC message and performs on-demand SI request.

- For SIB1 forwarding, for L2 U2N Remote UE, both request-based delivery and unsolicited forwarding are supported by L2 U2N Relay UE. If SIB1 changes, for L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE, the L2 U2N Relay UE always forwards SIB1.

According to the email discussion, most companies support SIBs monitoring performed by a last relay UE. And, there might be consensus about reusing R17 mechanism and signalling related to SIB delivery request and forwarding. based on that;

- Remote UE and intermediate relay UE in RRC_IDLE or RRC_INACTIVE inform the interested SIB(s) to the parent UE

- Intermediate relay UE inform the requested SIB(s) to the parent UE when child UE requests SIB delivery

- Last relay UE receives SIB(s) including requested SIB(s)

- Requested SIB received by UE is forwarded to UE in hop-by-hop manner

Here, following points should be further discussed;

- Whether the intermediate relay UE receives required SIB by the cell on Uu

In Rel-17, we think the behaviour of remote UE receiving SIB directly on the Uu interface is not specified. On the other hand, there is no statement restricting this behaviour. And we assume the intermediate relay UE also acts as a remote UE. Therefore, Rel-17 behaviour of remote UE should be applied. So, we propose that direct reception of SIBs does not need to be restricted or specified.

Proposal 2. Direct reception of SIBs on Uu by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

- Whether the intermediate relay UE send an indication to a parent UE when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu to initiate/cancel SI forwarding by the parent relay

As mentioned above, we assume that direct reception of SIBs on Uu by the intermediate relay UE is not specified. Furthermore, the intermediate relay UE should be located near the cell edge. If it is trigger of the request, the intermediate relay UE may transmit a request frequently since the UE frequently enter and exit the cell. Therefore, it should not be trigger of transmission of the request.

Proposal 3. The intermediate relay UE sends SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

- Whether the SIB provisioning from NW via RRC signalling for child UEs should be supported

There is no reason to restrict receiving SIB via dedicated RRC signalling as with Rel-17.

Proposal 4. The intermediate relay UE in RRC_CONNECTED can receive SIBs by dedicated RRC signalling.

- Whether the UE ID should be included in the SIB request

The parent UE needs to correctly understand which SIBs are requested by multiple child UEs, and performs forwarding processing based on that requests. Some companies propose that UE ID is useful for this purpose. However, we think there is no need to explicitly indicate UE ID to the parent UE. We will describe how to request and forward required SIBs by using figure 2.

Step 0. UE#1, UE#2 and UE#3 have their own wish lists

Step 1. UE#1 sends a request to UE#2. UE#2 stores the information related to the request, i.e. makes internal routing table based on who transmitted the request.

Step 2. UE#2 sends a request including information related to its own wish list and UE#1’s request to UE#3. UE#3 stores the information related to the request, i.e. makes internal routing table based on who transmitted the request.

Step 3. UE#3 sends a request including information related to its own wish list and UE#2’s request (that also includesUE#1’s request) to UE#4. UE#4 stores the information related to the request, i.e. makes internal routing table based on who transmitted the request.

Step 4. UE#4/3/2/1 receive requested SIBs and forward the SIBs based on the internal routing table.

Figure 2. an example of SIB delivery mechanism in multi-hop relaying

Therefore, we think UE ID is not needed in SIB request.

Observation 2. Relay UEs can forward SIBs based on the internal routing table as shown in figure 2.

Proposal 5. UE ID is not required in SIB request message and SIB delivery message.

Secondly, current paging delivery is supported by following mechanism [7];

- Method 1: The L2 U2N Relay UE monitors POs of its connected L2 U2N Remote UE(s) if the active DL BWP of the L2 U2N Relay UE is configured with common search space including paging search space

- Method 2: The delivery of the L2 U2N Remote UE's paging can be performed through a dedicated RRC message from the gNB to the L2 U2N Relay UE. The dedicated RRC message for delivering L2 U2N Remote UE paging to the RRC_CONNECTED L2 U2N Relay UE may contain one or more Remote UE IDs (5G-S-TMSI or I-RNTI).

According to the email discussion, most companies support method 1 performed by a last relay UE. And, there might be consensus about reusing R17 mechanism and signalling related to paging delivery request and paging record forwarding. based on that;

- Remote UE and intermediate relay UE requests paging delivery to a parent UE when it is in IDLE/INACTIVE

- Intermediate relay UE inform the paging information to the parent UE when child UE requests paging delivery

- Last relay UE monitors POs of the child UEs

- Paging record(s) received by UE is forwarded to requested UE in hop-by-hop manner

Here, following points should be further discussed;

- Whether the intermediate relay UE monitors PO for child UEs by the cell on Uu

As with SIB delivery, monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified.

Proposal 6. Monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

- whether the intermediate relay UE also releases paging information for the child UEs in a parent UE and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

For the paging information of the remote UE, the intermediate relay UE can receive paging information for the remote UE via dedicated RRC message. And an RRC message is forwarded at SRAP layer of a last relay UE (without reaching the RRC layer). Therefore, we think it is better to send paging record via dedicated RRC signaling. Thus, if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

Proposal 7. if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

- Whether the reception of paging records from NW via RRC signalling for child UEs should be supported

There is no reason to restrict receiving SIB via dedicated RRC signalling as with Rel-17.

Proposal 8. The intermediate relay UE in RRC_CONNECTED can receive paging record(s) by dedicated RRC signalling for child UE(s).

SRAP configuration

In RAN2#127, RAN2 agreed with following protocol stack for control plane as below.

Figure 7. candidate protocol stack for C-plane in multi-hop relay

SRAP layer supports the routing function, i.e. determination of egress RLC channel/link, determination of UE/BEARER ID field and data transfer. For routing function, gNB configures relay UE(s) and remote UE with local UE ID and mapping rule. Then, the relay UE(s) and remote UE determines the route of the packet based on the SRAP header of the packet. There are two types of SRAP header, one is for U2N SRAP data PDU and another one is for U2U SRAP data PDU. The both of SRAP header has common fields, i.e. D/C field, BEARER ID field and UE ID field(s). D/C field is exactly same. So, it can be reused for multi-hop relay. The value of BEARER ID refers appropriate bearer identity. It means that, in case of U2N relay, the value refers srb-Identity or drb-Identity (minus 1) and, in case of U2U relay, the value refers SL-SRB identity or slrb-PC5-ConfigIndex. In the multi-hop case, the U2N relay format can be reused since E2E RB is not sidelink specific bearer. The discussion point is UE ID field. In case of the U2N relay, one UE ID of remote UE’s local ID and, in case of U2U relay, two UE IDs of source remote UE and destination remote UE. In our understanding, two IDs are needed for the destination remote UE that receives the SRAP PDU to determine to which PDCP the corresponding SRAP SDU should be transferred if E2E SL-DRBs from multiple source UEs are mapped to one ingress PC5 RLC channel. In case of multi-hop relay, such a situation occurs in gNB. Then, gNB understands who is source UE based on one UE ID field (i.e. remote UE’s local ID). Also, the intermediate UEs can understand whether the SRAP SDU is UL or DL based on the received RLC even if there is only one UE ID, because one RLC entity cannot be established for multiple destinations. Therefore, SRAP header for legacy U2N relay can be reused.

Proposal 9. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 10. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay, i.e. remote UE’s local ID is set to the UE ID.

And, as with legacy, to avoid the local ID collisions, gNB should allocate local ID for each remote UE for managing sources of UL data and destinations of DL data.

Proposal 11. gNB should allocate local ID for remote UE(s) for SRAP configuration.

QoS handling

The latency requirement is important when we consider the QoS handling. Especially, some use cases, e.g. public safety, wearable and V2X, needs very low E2E latency. Therefore, optimization should be done to achieve low latency.

Proposal 12. QoS related optimization should be considered since very low E2E latency is required for some use cases.

When we consider QoS related optimization, it is important to know achievable QoS at each hop. Here, gNB can guarantee at least one hop (i.e. last relay hop) QoS if gNB allocate sidelink and/or uplink grant for the last relay UE using resource allocation mode 1. Thus, we propose that the principles of resource allocation for U2N relay operation follow the legacy mechanism.

Proposal 13. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 3. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

In U2U relay, U2U relay UE split the E2E QoS for each hop, and then each UE configures each hop configuration to guarantee the split QoS. Because U2U relay UE knows quality of both hops and can configure appropriate values. In multi-hop relay, there is no special UE like a U2U relay UE. However, if gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality. Thus, we propose that the gNB sets the appropriate (split) QoS to be met for each hop and UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Observation 4. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 14. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

- 2.2	Failure handling

For path failure handling, we can consider legacy mechanism as baseline. In legacy, there are some failure cases;

- U2N relay UE detects Uu RLF/ detects RRC failure/ receives HO command/ initiates cell-reselection

- U2U relay UE detects PC5 RLF

- U2N remote UE detects PC5 RLF/ detects RRC failure

- U2U remote UE detects PC5 RLF/ detects PC5 RRC failure

In case of multi-hop relay, intermediate relay UE is like a U2N remote UE and a U2U relay UE. Therefore, there are following failure cases from remote UE’s point of view;

- Last relay UE detects Uu RLF

- Last relay UE detects RRC failure

- Last relay UE receives HO command

- Any of the intermediate UEs detects PC5 RLF

- Any of the intermediate UEs detects RRC failure

- Any of the intermediate UEs receives path failure notification

- Remote UE detects PC5 RLF

- Remote UE detects RRC failure

- Remote UE receives path failure notification

In case of intermediate UE’s failure, legacy NotificationMessageSidelink can be reused. If the intermediate UEs detecting any failures needs to notify remote UE(s) of the failure, there should be E2E PC5-RRC connections for each UE combination. However, as mentioned above, establishment of E2E PC5-RRC connections for each UE combination will increase maintenance costs and complicate the system. Therefore, UE should use hop-by-hop PC5-RRC message in order to send a notification to remote UEs.

Proposal 15. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 16. NotificationMessageSidelink should be sent hop-by-hop if needed.

Here, we consider the case where failure information is notified for all remote UEs if last relay UE or intermediate UE detects a failure. Firstly, the UE detecting the failure notifies the failure to the nearest UE located on the branch side of the tree, and then the nearest UE notifies the failure to other nearest UE(s). Thus, all remote UEs in the outer branch of the tree to which the UE detected the failure belongs will perform RRC re-establishment. This behavior needs huge radio resource instantaneously. In addition, remote UE configured with multi-hop relay is unlikely to discover a new cell, and is also unlikely to discover any relay UEs other than the currently connected relay UE. Therefore, there are cases where it is effective, for a remote UE configured for multi-hop relay, to remain connected to the relay and wait for the relay to reconnect to the gNB. It means that the remote UE doesn’t have to perform RRC re-establishment if a connected (intermediate) relay UE initiates RRC re-establishment procedure.

Observation 5. There are cases where it is effective, for a remote UE configured for multi-hop relay, to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 17. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

We also consider the content of notification UE sends to another UE(s) when UE received the notification. One option is setting the same indicationType with the received notification. However, UE cannot understand whether the event indicated by the indicationType is occurring in the UE sent the notification, or the UE simply reflecting it. If UE doesn’t have to perform RRC re-establishment, such information is important to determine UE behavior. Then, another option is setting specific indicationType (e.g. receiving_Notification, perform_reestablish) for multi-hop.

Proposal 18. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

In the January meeting, i.e. SA2 AdHoc, SA2 agreed that intermediate UE can initiate relay reselection while keeping PC5-RRC connection with child UEs upon detecting PC5-unicast link failure [4]. In the CR, the intermediate relay UE is also acting as a remote UE. Therefore, there is no need to support a relay reselection procedure initiated by an intermediate relay UE separated from a relay reselection procedure initiated by a remote UE but a relay reselection procedure initiated by a remote UE can be supported by an intermediate relay UE which is also acting as a remote UE from RAN2 perspective.

Observation 6. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 19. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

In this case, i.e. intermediate relay UE initiated relay reselection, child UEs should be able to re-establish an RRC connection via the same intermediate relay UE. However, because transmissions during a link failure may result in packet loss, the child UEs need to know that a link failure has occurred and the relay reselection is occurring.

Proposal 20. An intermediate relay UE should indicate to a child UE that there is a PC5 link failure and the UE is performing relay reselection.

Furthermore, a child node should send RRCReestablishmentRequest to NW when the relay reselection performed by the intermediate relay UE is completed. So, to initiate sending RRCReestablishmentRequest, the intermediate relay UE should indicate the relay reselection completion if the relay reselection performed by the intermediate relay UE is completed.

Proposal 21. An intermediate relay UE should indicate the completion of the relay reselection to a child UE.

Conclusion

Observation 1. Each UE (last relay UE, intermediate relay UE, and remote UE) can perform different actions based on the authentication information.

Proposal 1. If multiple remote UEs connected to a single intermediate relay UE, the multiple remote UEs must connect to the same cell.

Proposal 2. Direct reception of SIBs on Uu by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

Proposal 3. The intermediate relay UE sends SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Proposal 4. The intermediate relay UE in RRC_CONNECTED can receive SIBs by dedicated RRC signalling.

Observation 2. Relay UEs can forward SIBs based on the internal routing table as shown in figure 2.

Proposal 5. UE ID is not required in SIB request message and SIB delivery message.

Proposal 6. Monitoring POs on Uu performed by the intermediate relay UE does not need to be restricted or specified as with Rel-17 remote UE.

Proposal 7. if there is no concern about forcing the network to send paging records for remote UE by dedicated RRC signaling, the intermediate relay UE that is RRC_CONNECTED releases paging related information for the remote UE in a parent UE.

Proposal 8. The intermediate relay UE in RRC_CONNECTED can receive paging record(s) by dedicated RRC signalling for child UE(s).

Proposal 9. D/C field and BEARER ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop relay.

Proposal 10. The one UE ID field in U2N SRAP Data PDU format with SRAP header are reused for multi-hop

relay, i.e. remote UE’s local ID is set to the UE ID.

Proposal 11. gNB should allocate local ID for remote UE(s) for SRAP configuration.

Proposal 12. QoS related optimization should be considered since very low E2E latency is required for some

use cases.

Proposal 13. The principles of resource allocation follow the legacy mechanism, i.e. last relay can be configured with mode 1 resource allocation, and remote UE and other relay UEs are configured with mode 2 resource allocation.

Observation 3. gNB can guarantee at least one hop (i.e. last relay hop) QoS using mode 1 resource allocation or UL grant allocation.

Observation 4. If gNB knows quality of each hop, gNB can configure appropriate QoS for each hop and also select appropriate indirect path with good quality.

Proposal 14. UE reports hop-by-hop link quality to gNB for path selection and split QoS configuration.

Proposal 15. NotificationMessageSidelink can be reused to send path failure information to remote UEs over multiple hops.

Proposal 16. NotificationMessageSidelink should be sent hop-by-hop if needed.

Observation 5. There are cases where it is effective, for a remote UE configured for multi-hop relay, to remain connected to the relay and wait for the relay to reconnect to the gNB.

Proposal 17. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

Proposal 18. RAN2 discusses whether the UE received NotificationMessageSidelink sets the same indicationType with the received notification or multi-hop specific indicationType.

Observation 6. SA2 agreed that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 19. RAN2 confirm that intermediate relay UE which is also acting as a remote UE can initiate relay reselection while keeping PC5-RRC connection with child UEs.

Proposal 20. An intermediate relay UE should indicate to a child UE that there is a PC5 link failure and the UE is performing relay reselection.

Proposal 21. An intermediate relay UE should indicate the completion of the relay reselection to a child UE.

Reference

- R2-2410914, Report from session on positioning and sidelink relay

- R2-2500064, LS on Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN

- R2-250xxxx, Report of [POST128][401][Relay] Control plane baseline solution (InterDigital)

- S2-2501292, Clarification on Multi-hop Relay Reselection