3GPP TSG RAN WG2 #129                                	R2-2501192

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.5.3

Source: 	Sharp

Title: 	Discussion on remaining details of on-demand SIB1

Document for:	Discussion

Introduction

The Rel-19 WID on enhancements to NES for NR was approved in [1], which includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

At RAN2#127b, it was agreed that the UE should check whether SIB1 is currently being transmitted before requesting OD-SIB1.

RAN2#127b agreements: - If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

At RAN2#128, it was agreed that the legacy cell reselection criterion is reused as the triggering condition of OD-SIB1 request, i.e., UL WUS.

RAN2#128 agreements: - Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

In this contribution, we will provide our views on the remaining details of on-demand SIB1 (OD-SIB1) operation.

Discussions

Cell-specific OD-SIB1 operation

So far, the discussions on OD-SIB1 seem to focus on cell-specific OD-SIB1, which means the OD-SIB1 on/off is per cell but not per SSB beam.

Based on the agreements made so far, our understanding on the baseline UE behaviors is summarized as below:

- A UE camps on a Cell A, and receives UL WUS configuration of a neighbor NES cell.

- There is a neighbor NES cell that is better than the serving cell according to the cell reselection criterion.

- The UE checks whether OD-SIB1 is being broadcast.

- If OD-SIB1 of the NES cell is not being broadcast, the UE initiates OD-SIB1 acquisition procedure. The UE optionally (not agreed yet) receive NW’s response to OD-SIB1 request sent by other UEs for a while. The UE selects an SSB and transmits UL WUS (as an RA procedure).

- The UE monitors NW’s response. If no NW’s response is received, the UE may re-transmit UL WUS, according to the UL WUS configuration until the OD-SIB1 request is considered success or failure by the UE.

Beam-specific OD-SIB1 operation

Since Rel-15, an SIB1 transmission is QCLed with the corresponding SSB. OD-SIB1 can be cell-specific or beam-specific. For cell-specific OD-SIB1, once the NES cell starts broadcasting OD-SIB1, the NES cell transmits OD-SIB1 for all SSBs. For example, the NES cell transmits eight SSBs, but only one UE in the coverage of SSB index 2 sends OD-SIB1 request. The NES cell has to turn on OD-SIB1 for all SSBs rather than for SSB index 2 only. This may not be energy-efficient. On the other hand, for beam-specific OD-SIB1, the NES cell can turn on OD-SIB1 for SSB index 2 only, which is more energy-efficient.

Beam-specific OD-SIB1 operation can further save NW’s energy, compared to cell-specific OD-SIB1 operation.

RAN2 is suggested to discuss the feasibility of beam-specific OD-SIB1 operation.

OD-SIB1 broadcasting status indication

The most intuitive design on OD-SIB1 broadcasting status is to provide an indication in the UL WUS configuration or in the MIB. For cell-specific OD-SIB1 operation, the indication may be a Boolean. For beam-specific OD-SIB1 operation, the indication may be a bitmap, with each bit corresponding to an SSB index.

However, UL WUS configuration is provided via system information, which means the modification period is relatively long. In addition, MIB repetition period is 80 ms, which is not short. Therefore, if OD-SIB1 can be turned on/off very dynamically, this design may let the UE misunderstand OD-SIB1 broadcasting status.

Providing OD-SIB1 broadcasting status in UL WUS configuration or MIB will lead to higher modification latency.

Monitoring NW’s response to OD-SIB1 request sent by other UEs

In previous meetings, some companies mentioned that if some UEs have already sent OD-SIB1 request to the NES cell and then the UEs are waiting for NW’s response, and if a newcomer UE just initiates a OD-SIB1 acquisition procedure, there is no need for the newcomer UE to send OD-SIB1 request. What the newcomer UE should do is to monitor NW’s response first. To achieve this, for cell-specific OD-SIB1 operation, the NW’s response can be designed as a PDCCH addressed by a common RNTI. The common RNTI can be provided  in the UL WUS configuration. It shall be noted that using RA-RNTI cannot achieve this purpose. For beam-specific OD-SIB1 operation, the NW’s response can be designed as a PDCCH addressed by an RNTI value corresponding to the selected SSB (during RA procedure or during cell reselection). The RNTI values corresponding to each SSB can be provided in the UL WUS configuration.

RAN2 WG to confirm that UE shall monitor NW’s response before sending OD-SIB1 request.

For cell-specific OD-SIB1, the UL WUS configuration provides a new RNTI dedicated for OD-SIB1 request. NW’s response to UL WUS is a PDCCH addressed by the new RNTI.

For beam-specific OD-SIB1 (if supported), the UL WUS configuration provides a list of new RNTIs dedicated for OD-SIB1 request, where each RNTI corresponds to one SSB index. NW’s response to UL WUS is a PDCCH addressed by the RNTI corresponding to the SSB the UE used for transmitting UL WUS.

According to the discussions so far, the designs on OD-SIB1 broadcasting status can be categorized as follows and they may co-exist:

- Option 1: The UE checks an indication in the UL WUS configuration corresponding to the NES cell or in the MIB of the NES cell.

- Option 2: The UE monitors NW’s response PDCCH addressed by a common RNTI value for a while. If a PDCCH addressed by the RNTI value is detected, the UE determines that OD-SIB1 is being broadcast. If no PDCCH addressed by RNTI is detected, the UE determines that OD-SIB1 is not being broadcast.bit

Option 1 has been discussed above. For Option 2, if the UE needs to check whether there is any other UE who has requested OD-SIB1 before transmitting UL WUS, the UE shall check the broadcasting status of the OD-SIB1 corresponding to the best or acceptable SSB. However, SSB selection occurs during cell reselection (for deriving the cell quality) and during OD-SIB1 acquisition procedure. The relationship between SSB selection, OD-SIB1 broadcasting status and OD-SIB1 request needs to be further studied.

RAN2 WG to discuss the contents of OD-SIB1 broadcasting status for both cell-specific and beam-specific OD-SIB1 operation.

For beam-specific OD-SIB1, the relationship between SSB selection, OD-SIB1 broadcasting status and OD-SIB1 request needs to be further studied.

OD-SIB1 request procedure after radio link failure

In RAN#106 meeting, RP-243297 [6] was endorsed and RAN2 WG would address how to support OD-SIB1 request under specific case of radio link failure event. We will analyze the scenario of radio link failure event and how to support OD-SIB1 request procedure after a RLF condition is met. In addition, we would also suggest RAN2 WG to consider whether to consider RAN sharing scenario for OD-SIB1 request procedure. Based on our analysis, additional network information is suggested to be included in UL-WUS configuration to support the OD-SIB1 request procedure after a RLF event.

Fig. 1 Way Forward on NES endorsed during RP#106 meeting

In this document, we would discuss how to support OD-SIB1 request procedure upon a radio link failure event occurs at the UE side.

OD-SIB1 support for radio link failure

Based on R-19 NES meeting progress and current 3GPP specification [7], a UE may firstly receive and store a UL-WUS configuration from the source cell. The source cell is also the cell A of the UE since the UE receives the UL-WUS configuration of neighbor cells from the source cell. So, the UE would store a valid UL-WUS configuration while the UE is staying in RRC connected state. Then, the UE detects a radio link failure condition is met at the UE side. Based on [7], the UE would start the timer T311 when the UE initiates a cell selection procedure and the UE would stop the timer T311 if the UE selects a suitable cell for the following random access procedure for RRCReestablishmentRequest message transmission. However, to support OD-SIB1 request, now the UE may select an OD-SIB1 cell while T311 is running and so different treatments should be considered in Release-19.

Fig. 2 Signaling flow of UL-WUS configuration, OD-SIB1 request procedure, and RRC re-establishment procedure

Radio link failure

Based on [7], a UE may consider a radio link failure event happens in different scenarios:

- 5.3.7.2 Initiation The UE initiates the procedure when one of the following conditions is met: 1> upon detecting radio link failure of the MCG and t316 is not configured, in accordance with 5.3.10; or 1> upon detecting radio link failure of the MCG while SCG transmission is suspended, in accordance with 5.3.10; or 1> upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing, in accordance with 5.3.10; or 1> upon detecting radio link failure of the MCG while the SCG is deactivated, in accordance with 5.3.10; or 1> upon re-configuration with sync failure of the MCG, in accordance with clause 5.3.5.8.3; or 1> upon mobility from NR failure, in accordance with clause 5.4.3.5; or 1> upon integrity check failure indication from lower layers concerning SRB1 or SRB2, except if the integrity check failure is detected on the RRCReestablishment message; or 1> upon an RRC connection reconfiguration failure, in accordance with clause 5.3.5.8.2; or 1> upon detecting radio link failure for the SCG while MCG transmission is suspended, in accordance with clause 5.3.10.3 in NR-DC or in accordance with TS 36.331 [10] clause 5.3.11.3 in NE-DC; or …… 1> if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with clause 5.3.10 while the transmission of indirect path is suspended as specified in 5.3.5.17; or 1> if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) in accordance with 5.3.10 while MP indirect path addition or change is ongoing; or ……

Based on TS 38.331, the following radio link failure events are included to trigger the cell selection procedure while the UE is staying in RRC connected state:

  - Condition#1: upon detecting radio link failure of the MCG and t316 is not configured,

  - Condition#2: upon detecting radio link failure of the MCG while SCG transmission is suspended,

  - Condition#3: upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing,

  - Condition#4: upon detecting radio link failure of the MCG while the SCG is deactivated,

  - Condition#5: upon detecting radio link failure for the SCG while MCG transmission is suspended.

- In addition, if multi-path (MP) relay scenario is jointly considered. Additional cell selection procedure and random access procedure of RRC re-establishment may also happen:

  - Condition#6: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while the transmission of indirect path is suspended, and

  - Condition#7: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while MP indirect path addition or change is ongoing.

RAN2 WG should identify whether all or a subset of the RLF conditions in 3GPP specifications would be supported for OD-SIB1 request procedure.

  - Condition#1: upon detecting radio link failure of the MCG and t316 is not configured

  - Condition#2: upon detecting radio link failure of the MCG while SCG transmission is suspended

  - Condition#3: upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing

  - Condition#4: upon detecting radio link failure of the MCG while the SCG is deactivated.

  - Condition#5: upon detecting radio link failure for the SCG while MCG transmission is suspended

  - Condition#6: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while the transmission of indirect path is suspended

  - Condition#7: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while MP indirect path addition or change is ongoing

Cell selection

After a RLF condition is met, the UE would start a timer T311 and initiate a cell selection procedure. Also, based on [7], the running T311 would be stopped after the UE selects a suitable cell for which the UE has valid and up-to-date essential system information.

5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: 1> ensure having valid and up to date essential system information as specified in clause 5.2.2.2; 1> stop timer T311;

However, please note the network identity list supported by a cell, which is also part of the essential system information, is included in its SIB1. Therefore, it may not be possible for the UE to decide the selected OD-SIB1 cell is suitable cell or not if the UE does not know the network(s) supported by the target cell. Therefore, it is suggested that the UL-WUS configuration of one OD-SIB1 cell can further include the information of the network(s) supported by the OD-SIB1 cell. To further address this issue, RAN2 WG is suggested to confirm whether RAN sharing is supported between the Cell A (which is the source cell in Fig. 2) and the target OD-SIB1 cell.

RAN2 WG is suggested to decide whether OD-SIB1 request supports RAN sharing.

If RAN sharing is supported among cell A and OD-SIB1 cell. Then, the networks supported by the source cell (Cell A) and the networks supported by the target cell (OD-SIB1 cell) could be further analyzed based on the following Case 1/Case 2/Case 3:

Fig. 3 Different cases when RAN sharing are supported among cell A/OD-SIB1 cell.

Case 1: UE’s registered network is supported by both the source cell and the selected cell

Case 2: UE’s registered network is supported by the source cell but not supported by the selected cell

Case 3: UE’s registered network is supported by the selected cell but not supported by the source cell

Firstly, case 3 isn’t a valid case since the source cell should act as a suitable cell for to support RRC connection between the UE and RAN. In addition, in case 1, UE can initiate a random access procedure for RRCReestablishmentRequest message transmission without additional issue. However, in case 2, it is clear that additional signaling overhead and latency to the RRC re-establishment procedure would be caused if the selected cell is not a suitable cell to the UE, which means the UE has to reselect another cell for RRC re-establishment procedure (if T311 is still running). In addition, it is also observable that a RRC re-establishment procedure may fail more easily if the UE can’t find another suitable cell before T311 expiry. Therefore, to avoid un-necessary signaling overhead and latency, we propose the information of the supported network of one OD-SIB1 cell should be included as part of UL-WUS configuration to the UE. The details of the network information may be further discussed. To initiate the discussion, RAN2 WG is proposed to confirm that R-19 OD-SIB1 request procedure is supported by public networks and non-public network respectively.

The information of the supported network of one OD-SIB1 cell should be included as part of UL-WUS configuration to the UE. The details of the network information can be further discussed based on the networks supported by cell A and OD-SIB1.

RAN2 WG is suggested to confirm R-19 OD-SIB1 request procedure is supported by public network and non-public network. So, the supported network information of one OD-SIB1 cell may be a PLMN identity list or a SNPN identity list.

Please also note proposals 9 and 10 would also be helpful to idle mode UE.

In contrast, if the UE can’t obtain the supported network information of a selected OD-SIB1 cell, then RAN 2 WG is suggested to discuss whether T311 should be stopped if the UE selects an OD-SIB1 cell, which the UE can’t yet identify it is suitable cell or not.

RAN2 WG is suggested to decide whether a running T311 should be stopped upon the UE selects an OD-SIB1 cell, which the UE can’t justify it is a suitable cell to the UE or not yet.

Finally, please note, during the cell selection procedure after RLF event, the UE may select a neighbor cell or the original source cell for RRC re-establishment. However, the OD-SIB1 request procedure to the original source cell may be a rare case since the UE may have stored a valid SIB1 of the original source cell and so we would not expect UE to initiate a OD-SIB1 request procedure to the original source cell. Please also note, based on the way forward [6], a cell may switch from a cell A to a OD-SIB1 cell and it may happen after a RLF event occurs at the UE side. Even so, it is assumed the switch would not influence the RRC connected UE.

RAN2 WG is suggested to confirm that OD-SIB1 request procedure to the source cell after the RLF event would not be expected.

Backhaul support

In addition, during the RRC re-establishment procedure, UE context may to be moved between different radio nodes if the source cell and the selected cell are configured by different nodes. To maintain the RRC connection between the UE and the serving RAN. However, since the UE needs to obtain the SIB1 of the target cell before the UE initiates a RRC re-establishment request procedure, additional latency caused by the OD-SIB1 request procedure should be included.

Processing latency in air interface and backhaul may not be ignorable for UE context retrieval if OD-SIB1 request is implemented during the RRC re-establishment procedure.

Therefore, to a radio access network, it is possible that the OD-SIB1 request may be supported between the serving radio node of the UE (which supports the source cell) and a subset of neighbor radio nodes (which support some neighbor cells around the UE) but not supported between the serving radio node and other neighbor radio nodes. RAN2 WG is suggested to confirm whether OD-SIB1 request can be supported between every radio node to maintain an OD-SIB1 capable UE’s RRC connection. Since this issue belongs the scope of RAN3 WG, a liaison to RAN3 WG may need to be considered.

RAN2 WG is suggested to confirm whether all of the radio nodes in RAN should support a UE in RRC Connected state to initiate an OD-SIB1 request procedure before a RRC re-establishment procedure. A liaison to RAN3 WG may be needed if backhaul latency is a potential impact factor to this issue.

Conclusions

Based on the discussion, we have the following observation and proposals:

- Beam-specific OD-SIB1 operation can further save NW’s energy, compared to cell-specific OD-SIB1 operation.

- RAN2 to discuss beam-specific OD-SIB1 operation.

Providing OD-SIB1 broadcasting status in UL WUS configuration or MIB will lead to higher modification latency.

The UE shall monitor NW’s response before sending OD-SIB1 request.

For cell-specific OD-SIB1, the UL WUS configuration provides a new RNTI dedicated for OD-SIB1 request. NW’s response to UL WUS is a PDCCH addressed by the new RNTI.

For beam-specific OD-SIB1 (if supported), the UL WUS configuration provides a list of new RNTIs dedicated for OD-SIB1 request, where each RNTI corresponds to one SSB index. NW’s response to UL WUS is a PDCCH addressed by the RNTI corresponding to the SSB the UE used for transmitting UL WUS.

RAN2 to discuss the contents of OD-SIB1 broadcasting status for both cell-specific and beam-specific OD-SIB1 operation.

For beam-specific OD-SIB1, the relationship between SSB selection, OD-SIB1 broadcasting status and OD-SIB1 request needs to be further studied.

Based on TS 38.331, the following radio link failure events are included to trigger the cell selection procedure while the UE is staying in RRC connected state:

  - Condition#1: upon detecting radio link failure of the MCG and t316 is not configured,

  - Condition#2: upon detecting radio link failure of the MCG while SCG transmission is suspended,

  - Condition#3: upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing,

  - Condition#4: upon detecting radio link failure of the MCG while the SCG is deactivated,

  - Condition#5: upon detecting radio link failure for the SCG while MCG transmission is suspended.

- In addition, if multi-path (MP) relay scenario is jointly considered. Additional cell selection procedure and random access procedure of RRC re-establishment may also happen:

  - Condition#6: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while the transmission of indirect path is suspended, and

  - Condition#7: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while MP indirect path addition or change is ongoing.

Proposal#1: RAN2 WG should identify whether all or a subset of the RLF conditions in 3GPP specifications would be supported for OD-SIB1 request procedure.

  - Condition#1: upon detecting radio link failure of the MCG and t316 is not configured,

  - Condition#2: upon detecting radio link failure of the MCG while SCG transmission is suspended,

  - Condition#3: upon detecting radio link failure of the MCG while PSCell change or PSCell addition is ongoing,

  - Condition#4: upon detecting radio link failure of the MCG while the SCG is deactivated,

  - Condition#5: upon detecting radio link failure for the SCG while MCG transmission is suspended,

  - Condition#6: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while the transmission of indirect path is suspended, and

  - Condition#7: if MP is configured, upon detecting radio link failure of the MCG (i.e. direct path) while MP indirect path addition or change is ongoing.

The information of the supported network of one OD-SIB1 cell should be included as part of UL-WUS configuration to the UE. The details of the network information can be further discussed based on the networks supported by cell A and OD-SIB1.

RAN2 WG is suggested to confirm R-19 OD-SIB1 request procedure is supported by public network and non-public network. So, the supported network information of one OD-SIB1 cell may be a PLMN identity list or a SNPN identity list.

RAN2 WG is suggested to decide whether a running T311 should be stopped upon the UE selects an OD-SIB1 cell, which the UE can’t justify it is a suitable cell to the UE or not yet.

RAN2 WG is suggested to confirm that OD-SIB1 request procedure to the source cell after the RLF event would not be expected.

Processing latency in air interface and backhaul may not be ignorable for UE context retrieval if OD-SIB1 request is implemented during the RRC re-establishment procedure.

RAN2 WG is suggested to confirm whether all of the radio nodes in RAN should support a UE in RRC Connected state to initiate an OD-SIB1 request procedure before a RRC re-establishment procedure. A liaison to RAN3 WG may be needed if backhaul latency is a potential impact factor to this issue.

References

RP-242354, Revised WID: Enhancements of network energy savings for NR.

Chair’s notes RAN2#127, May 20th – May 24th, 2024

Chair’s notes RAN2#127bis, August 19th – August 23rd, 2024.

Chair’s notes RAN2#128, October 14th – October 18th, 2024.

Chair’s notes RAN2#128, November 14th – November 18th, 2024.

RP-243297, “Way forward on NES”

3GPP TS 38.331 V18.4.0 (2024-12-20)