3GPP TSG-RAN WG2 Meeting #129	R2-2501201

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss RAN1 LS received in R2-2500012 and continue the remaining discussion on the CONNECTED mode LP-WUS.

# 2	Discussion

## 2.1	RAN1 LS

RAN1 sent an LS in R2-2500012 asking if the legacy DRX Command MAC CE and the Long DRX Command MAC CE can be used for the following purposes:

- In option 1-1, to stop drx-onDurationTimer and drx-InactivityTimer

- In option 1-2, to stop the new timer

RAN2 has not yet discussed this issue yet. However, in DRX, the UE stops onDurationTimer and drx-InactivityTimer based on the received the DRX Command MAC CE or the Long DRX Command MAC CE. Therefore, it would be logical to stop drx-onDurationTImer and drx-InactivityTImer in option 1-1 based on the (Long) DRX command MAC CE.

In option 1-2, RAN2 decided to introduce the new timer, which serves the same purpose as drx-InactivityTimer but allows flexibility in configuring the timer value. Consequently, it would also be reasonable to stop this new timer based on the (Long) DRX Command MAC CE.

Proposal 1: RAN2 sends a reply LS to RAN1, indicating that the (Long) DRX Command MAC CE stops drx-onDurationTimer and drx-InactivityTimer for option 1-1, as well as the new timer for option 1-2.

## 2.2	Option 1-2

### 2.2.1	Start of the new timer

In RAN1, time window containing multiple MOs is under discussion, which might impact the start time of the new timer (drx-lpwusInactivityTimer). For instance, drx-lpwusInactivityTimer could start either at the end of the time window, i.e., at the last MO, or immediately upon detection of the LP-WUS indication (without waiting until the end of time window).

In our understanding, the network can select any MO, even multiple MOs, within the time window. In this case, it would be safe to start the new timer at the end of time window to make sure that the active time is aligned between the UE and the network.

Proposal 2: In option 1-2, with the time window containing multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

### 2.2.2	DRX cycle handling

RAN2 agreed in RAN2#128bis that:

For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time.

Since On-Duration is determined based on the currently used DRX cycle, whether short or long, this agreement raises the question of which DRX cycle is used during LP-WUS monitoring to determine the location of On-Duration, i.e., the time given by the configured drx-onDurationTimer.

Assuming both the DRX command MAC CE and the long DRX Command MAC CE are used to stop the new timer for option 1-2, one approach would be to follow the legacy handling of DRX cycle – using either short or long DRX cycle based on the received DRX command and DRX configuration. Additionally, DRX cycle can also change from short to long when drx-shortCycleTimer expires. This approach might also require determining the DRX cycle when the newly introduced timer expires.

Another approach is to continue using one of the DRX cycles while the UE monitors LP-WUS. For instance, the network can configure which DRX cycle to use, or the UE can continue using its current DRX cycle without switching to another DRX cycle during LP-WUS monitoring. The intent is to avoid unnecessary switching from the short to the long DRX cycle, as LP-WUS monitoring occasion may be independent of the DRX cycle. Moreover, there is no good justification for changing the frequency of CSI/L1-RSRP reporting during LP-WUS monitoring.

Proposal 3: RAN2 discuss how to handle the DRX cycle during LP-WUS monitoring in option 1-2:

- Option 1: Allow DRX cycle switching during LP-WUS monitoring

- Option 2: Prevent DRX cycle switching during LP-WUS monitoring

### 2.2.3	CG/SPS

It has not yet been discussed whether the MR keeps the SPS and CG activated while the UE monitors the LP-WUS.

If SPS and CG are kept activated, transmission or reception of MAC PDU on SPS/CG will start the corresponding timers, i.e., drx-HARQ-RTT-TimerDL/UL and drx-RetransmissionTimerDL/UL, which might then keep the MR awake. To save more power, it might be better to deactivate SPS and CG. As it is possible to deactivate SPS and CG by PDCCH and activation/deactivation MAC CE, respectively, it seems not necessary to define additional deactivation mechanism such as the UE by itself deactivate the SPS or CG when starting LP-WUS monitoring.

Proposal 4: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

If SPS and CG are deactivated while monitoring LP-WUS, the gNB needs to send the LP-WUS to wake up the MR first and then activates the SPS/CG. This would introduce additional latency because the UE waits for receiving the activation command after starting the PDCCH monitoring. Moreover, the gNB may intend to rely on SPS/CG scheduling instead of dynamic scheduling in the MR, in which cases further PDCCH monitoring after SPS/CG activation would be needlessly consuming power.

Thus, RAN2 needs to discuss how to reduce unnecessary PDCCH monitoring when using SPS/CG. For instance, we can consider that the LP-WUS indicates SPS/CG activation.

Proposal 5: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

## 2.3	Option 1-3

Option 1-1 and 1-2 are being actively discussed in RAN1 and RAN2, and the detailed procedures are gradually taking shape. However, option 1-3 has not yet been discussed.

Option 1-3 is understood that the UE monitors LP-WUS inside C-DRX active time. As concluded from the study [TR 38.869], the power saving gain is expected by reducing unnecessary PDCCH monitoring within C-DRX active time. However, there may be limited use of this option 1-3 considering the length of On duration [R2-2404380].

In our view, we already have some mechanism to end or reduce unnecessary PDCCH monitoring during active time, e.g., sending DRX command MAC CE or PDCCH skipping. Thus, it would be desirable to focus more on first two options, i.e., option 1-1 and option 1-2.

Proposal 6: Option 1-3 is excluded in RAN2.

# 3	Conclusion

In this paper, we discuss the RAN1 LS and the remaining issues for connected mode LP-WUS, and propose:

Proposal 1: RAN2 sends a reply LS to RAN1, indicating that the (Long) DRX Command MAC CE stops drx-onDurationTimer and drx-InactivityTimer for option 1-1, as well as the new timer for option 1-2.

Proposal 2: In option 1-2, with the time window containing multiple MOs, the UE starts the new timer (tentatively named drx-lpwusInactivityTimer) at the last MOs of the window.

Proposal 3: RAN2 discuss how to handle the DRX cycle during LP-WUS monitoring in option 1-2:

- Option 1: Allow DRX cycle switching during LP-WUS monitoring

- Option 2: Prevent DRX cycle switching during LP-WUS monitoring

Proposal 4: In option 1-2, when the UE starts LP-WUS monitoring, SPS and CG are kept as it is unless activated or deactivated by the network explicitly.

Proposal 5: For option 1-2, RAN2 to discuss the LP-WUS impact activation of SPS and CG Type 2.

Proposal 6: Option 1-3 is excluded in RAN2.

# 4	Agreements

Agreements from RAN2#126

In RRC_CONNECTED mode, RAN2 to further discuss the impacts of LP-WUS operation methods identified in RAN1. For Option 1-1 (as described in RAN1 agreement), the LP-WUS monitoring occasion locates at a configured time offset before the start of drx-onDurationTimer. The range of time offset can be determined by RAN1. For Option 1-1, RAN2 assumes the solutions/ operations introduced for DCP mechanism is taken as baseline. RAN2 assume that legacy DCP and Option 1-1 is not configured simultaneously for a UE. The LP-WUS related configuration for RRC CONNECTED state UE is provided via dedicated RRC message.

Agreements from RAN2#127

For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1.

Agreements from RAN2#127bis

For Option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. In option 1-2, a new timer triggered by LPWUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LPWUS monitoring. When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected.

Agreements from RAN2#128:

The drx-onDurationTimer is not started with Option 1-2 LP-WUS. For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE.

Agreements from RAN1#119:

Agreement LP-WUS in RRC CONNECTED mode does not include Scell dormancy indication. - Note: LP-WUS indication does not impact SCell dormancy behavior. Agreement For LP-WUS CONNECTED mode operation, the following case is not supported: - Both Option 1-1 and Option 1-2 are simultaneously configured for the same UE in a given cell. Agreement - For LP-WUS MOs in connected mode for Option 1-1, following is considered further discussion and possible down-selection - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE selectively monitors LP-WUS in the LP-WUS MOs identified by other parameters such as time offset prior to the start of drx-onDurationTimer. - Approach 2: - LP-WUS MOs, including periodicity and offset, are identified based on the slot that drx-onDurationTimer would start and a new RRC parameter indicating the time offset until the slot that drx-onDurationTimer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with a same C-DRX cycle; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same C-DRX cycle of the UE, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same C-DRX cycle. - For LP-WUS MOs in connected mode for Option 1-2, following is considered further - Approach 1: - LP-WUS MOs, including periodicity and offset, are configured independently from the C-DRX periodicity/offset by new RRC parameter(s). - UE monitors LP-WUS in the LP-WUS MOs and starts a new timer for PDCCH monitoring triggered by LP-WUS, after a time offset. - Approach 2: - Periodicity/offset of the slot that the new timer for PDCCH monitoring may be potentially triggered by LP-WUS are configured by RRC parameters. - LP-WUS MOs, including periodicity and offset, are identified based on the slot that new timer would start and a new RRC parameter indicating the time offset until the slot that new timer would start. - FFS: An RRC parameter indicates the duration where the UE monitors LP-WUS [consecutively in time] - FFS: the consecutive LP-WUS MOs in time identified by the RRC parameter are associated with the same slot that the new timer would start; - FFS: LP-WUS MOs in time identified by the RRC parameter provides consistent indication for the same slot that the new timer would start, i.e., UE does not expect to detect more than one LP-WUSs with different indications for the UE in the LP-WUS MOs associated with the same slot that the new timer would start. Agreement For Option 1-2 of LP-WUS CONNECTED mode operation, for explicit indication to stop the new timer for PDCCH monitoring in Option 1-2 and drx-InactivityTimer for the DRX group, RAN1 identified the following potential solution - Use existing DRX Command MAC CE and Long DRX Command MAC CE - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask the feasibility of the above potential solution. Conclusion For Option 1-1 of LP-WUS CONNECTED mode operation, RAN1 assumes existing DRX Command MAC CE and Long DRX Command MAC CE can be used to stop drx-onDurationTimer and/or drx-InactivityTimer for the DRX group - When UE is not in C-DRX active time, UE monitors LP-WUS Send LS to RAN2 to ask whether RAN2 has same understanding or not. Agreement For Option 1-2, for LP-WUS CONNECTED mode operation, the periodic CSI/L1-RSRP reporting operation is same as Rel-16 DCP, that is the UE can be configured with a parameter to enable/disable periodic CSI/L1-RSRP reporting, respectively: - If the parameter is NOT configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is not reported. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - If the parameter is configured: - If the UE is not indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the time given by the configured drx-onDurationTimer in DRX-Config for the case when UE is outside C-DRX active time. - If the UE is indicated to wake up by LP-WUS, the periodic CSI/L1-RSRP is reported during the DRX active time. - FFS: Whether RAN1 spec impact is required.