3GPP TSG-RAN WG2 Meeting #129	R2- 2501202

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.6

Source:	Nokia, Nokia Shanghai Bell

Title:	XR rate control

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The following agreements were made in RAN2#128:

Agreements on XR rate control - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

In this contribution, we are discussing the identified open issues related to XR rate control based on the agreements from previous RAN2 meetings.

# 2	Discussion

## 2.1	UE or CN based indication of QoS flows for rate control

In XR applications, some services, such as video streaming, use adaptive codecs that adjust to network congestion, while others lack this capability. Therefore, it is essential to identify which QoS flows can dynamically adjust their bit rate.

At RAN2#128, it was agreed that the gNB could be informed about which QoS flows can be throttled, though whether this indication originates from UE or CN remains FFS. Two options were considered:

- Option 1: UE based approach – The UE informs gNB about QoS flow(s) capable of adjusting their bitrate.

- Option 2: CN based approach – The gNB obtains the information from CN entity (e.g. SMF).

Both options are technically feasible from RAN2 perspective – option 1 could be implemented via RRC signalling e.g., UAI, whereas option 2 may require additional work in RAN3 and SA2, as the CN currently does not provide such information to the gNB.

Since RAN2 assumes that the gNB can detect congestion in a QoS flow without involving the UE, option 2 would offer more centralized framework for uplink rate control while minimizing the impact on the UE side and Uu interface. Therefore, we propose to go for option 2 and send an LS to RAN3 and SA2 for further discussion.

Proposal 1: For XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

## 2.2	Design of Rate control signalling

In the last meeting, RAN2 sent an LS to SA4 and RAN3 [R2-2411218] to assess whether indicating QoS flows for rate control is feasible from their perspective. While we are still awaiting their response, RAN2 can discuss the signaling aspects of the rate control procedure.

During the previous meeting, RAN2 briefly explored potential approaches and identified two options for rate control signaling:

- Option 1: QoS flow information for rate control is included in the rate control MAC CE.

- Option 2: The QoS flow information for rate control is provided via RRC, while the DRB information is included in the rate control MAC CE.

In option 1, the rate control MAC CE directly indicates the index of any QoS flow to which the rate control is applied, offering more flexibility, especially when multiple QoS flows are subject to rate control. Currently, the maximum number of QFI is 63 as defined in TS 38.331, so QoS flow index field could use 6 bits in the MAC CE. However, as discussed in Section 1.1, not all QoS flows may be applicable to rate control. Thus, instead of using the actual QFI value, a newly assigned index could be used in the MAC CE, potentially reducing the size of QoS flow index field to be fewer than 6 bits.

In option 2, the intent is to indicate LCID or radio bearer (RB) ID instead of QoS flow ID because CU-DU split structure on the network side cannot not easily support use of QoS flow in DU. Currently, QoS flows are primarily handled on the CU, meaning option 2 would require CU-DU interaction on the network side. However, in SA2, discussions on reporting the available data rate for a QoS flow is ongoing. The latest LS from SA2 to RAN3 [S2-2412720] clarified that the gNB needs to report the estimated available data rate for GBR to CN. Our understanding is that reporting the available data for a QoS flow will require enhancements to both UP (NG-U/F1-U) and CP (NGAP/F1AP/XnAP) in RAN3. This implies that the DU would need to manage the QoS flows accordingly. Thus, supporting the use of QoS flows in DU might not be a critical challenge in option 2.

In option 2, the network needs to configure a single QoS flow within one RB for which the rate control can be indicated via MAC CE. Since option 2 requires RRC reconfiguration to change the QoS flow to which rate control can be indicated via MAC CE, this option would be more suitable under the assumptions: 1) A single QoS flow is mapped to a RB, 2) only one QoS flow is subject to rate control for a given RB, or 3) QoS flow requiring rate control does not change frequently. One may think option 2 allows reuse of the existing RBR MAC CE as this MAC CE includes 6 bits of LCID field already. However, we think it may depend on the range of supported bit rate for the rate control since LS response from SA4 indicates that the required bit rate range of 100kbps - 40Mbps with 1% - 10% of granularity.

It seems that the design of rate control signaling will depend on the ongoing discussions in SA4 and RAN3 regarding how QoS flow can be effectively managed at the network side. Therefore, we suggest waiting for further progress in SA4 and RAN3 to avoid complicating the discussions in those WGs.

Proposal 2: RAN2 wait for further progress in SA4 and RAN3 to decide the design of MAC CE for rate control.

## 2.3	Refined bit rate values for XR

During RAN2#127bis an LS [R2-2409273] was sent to SA4 to request input on appropriate range and granularity of uplink bit rates that need to be supported for the MAC layer signaling enhancements to enable XR applications to adapt bit rates in response to uplink congestion. SA4 replied to the RAN2 LS [S4-242182] with a bit rate range which varies from 100kbps to 40Mbps. The granularity values are indicated to vary from 1% to 10%.

In TS 38.321, for the Recommended Bit Rate procedure, a fixed bit rate table is defined with an index mapping to a bit rate value, allowing indication of the bit rate values not listed in the table through the use of the bit rate multiplier (x1, x40, x70, x100, x200). The combination of the bit rate index and the multiplier results in a large number of potential bit rate values from 0Kbps to around 1.6Gbps and a quantization error varying from 6.7% to 30.8%.

However, this table was mainly designed for voice services and the single generic bit rate table based on this table may not be efficient and future proof due to different and competing needs of different applications. Therefore, designing a new table with the proper values would the most efficient way to achieve good granularity across the entire bit rate ranges.

Considering these points and LS response from SA4, if a new bit rate table with the bit rate range of 100kbps to 40Mbps and granularity of 1% to 10% is defined for XR application, it would allow more precise rate control without requiring excessive effort to create a highly generalized bit rate range applicable to all types of QoS flows from different applications. In our view, there can be two ways this new table can be generated:

- Option 1: Static bit rate table selected based via RRC signalling

- Option 2: Dynamic bit rate table generated from exchanged parameters

In option 1 a new static bit rate table can be defined in the specification for XR application as well as more bit rate tables for any other applications’ needs. This way there can be multiple bit rate table each to cater the requirements of a specific application or network scenarios. Which table to use for a specific application or network scenario can be indicated during the connection setup via the RRC signalling.

In option 2 a dynamic bit rate table can be generated for each application based on the attributes exchanged between the UE and the network. Example of attributes involved in the table negotiation may be scaling factor parameter, offset parameter, minimum bit rate value, maximum bit rate value, quantization function, etc. This way a bit rate table is generated which can meet the needs of the application at hand.

The bit rate table generated by any of the options above can work following the same principles of Recommended Bit Rate procedure as in legacy. That is, the UE and the RAN node can exchange MAC CEs for Bit Rate Recommendation query (UE to RAN node) and/or Bit Rate Recommendation (RAN node to UE) to indicate their preference to operate another bit rate value within the new bit rate table. This can be achieved by keeping the same format of the RBR MAC CE as in the current specifications (TS 38.321). Should the needs arise, it may also be possible to enhance the format of the MAC CE with additional extensions compared to the legacy MAC CE.

Observation 1: A fixed table with generalized bit rate ranges and a multiplier may not be efficient in defining the bit rate range needed for different applications.

Proposal 3: RAN2 to consider whether to allow defining/configuring different bit rate tables or a dynamic bit rate table for XR rate control.

## 2.5	Scenarios with dual connectivity

In the scenarios with dual connectivity (DC) and split bearer, the data from a single DRB or QoS flow is sent over multiple cells e.g. MCG and SCG. When congestion occurs either in MCG or SCG or both, the UE may receive the recommended bit rate MAC CE with updated bit rate from at least one of the connected cells in case no coordination between MCG and SCG. In this case, due to different congestion situations in the connected cells, UE may or may not need to change the bit rate at application layer. As another possibility, it is also feasible that only MCG sends the updated recommended bit rate MAC CE to cover the latest recommended bit rate from both cells. From UE point of view, it might be easier to determine whether to change the application bit rate or not based on a single MAC CE. While as the first step, RAN2 needs to discuss whether to consider rate control for DC scenarios in Rel-19 XRM work.

Proposal 4: RAN2 to specify how to support XR rate control in case with DC.

# 3	Conclusion

This document has made the following observation and proposals:

Proposal 1: For XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control (i.e., Option 2). Send an LS to RAN3 and SA2.

Proposal 2: RAN2 wait for further progress in SA4 and RAN3 to decide the design of MAC CE for rate control.

Observation 1: A fixed table with generalized bit rate ranges and a multiplier may not be efficient in defining the bit rate range needed for different applications.

Proposal 3: RAN2 to consider whether to allow defining/configuring different bit rate tables or a dynamic bit rate table for XR rate control.

Proposal 4: RAN2 to specify how to support XR rate control in case with DC.