3GPP TSG-RAN WG2 Meeting #129		R2-2501210

Athens, Greece, 17th – 21st Feb. 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR | 1242 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE capability for BWP_Wor (option2-306CR) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_BWP_wor-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TS 38.306, the current field description for ncd-SSB-BWP-Wor-r18 only mentions the Bandwidth of UE-specific RRC configured BWP may not include bandwidth of the CORESET#0 (if CORESET#0 is present) and CD-SSB for PCell/PSCell (if configured). For PSCell case, the current wording may not be clear enough on whether it includes the case for CD-SSB-less PSCell, e.g. when the UE is configured only with the initial BWP which contains NCD-SSB, and there is no UE-specific BWP or other NCD-SSB; or when NCD-SSB is associated with both initial BWP and UE specific BWP, and UE’s active BWP is the initial BWP. In this way, the current specification may be mis-interpreted as ncd-SSB-BWP-Wor-r18 is not applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. According to RAN4 specifcation, UE will not measure NCD-SSB unless UE supports ncd-SSB-BWP-Wor-r18 in this case. Thus, it is clear that ncd-SSB-BWP-Wor-r18 is also applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. In order to avoid any mis-interpretation on the current field decription for ncd-SSB-BWP-Wor-r18, it is better to change the corresponding clarifcation for PSCell in the field description for ncd-SSB-BWP-Wor-r18 as: For PSCell (if configured), bandwidth of UE-specific RRC configured BWP needs not include bandwidth of the CORESET#0 (if CORESET#0 is present) and SSB indicated by absoluteFrequencySSB (either CD-SSB or NCD-SSB). Considering this part is more like stage-2 description, it could be moved to stage-2 specification and the corresponding part in TS 38.306 should be removed. Same issue as for rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 and bwpOperationMeasWithoutInterrupt-r18. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The corresponding clarifcation in the field description for ncd-SSB-BWP-Wor-r18, rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 and bwpOperationMeasWithoutInterrupt-r18 is moved to TS 38.300. The related part in TS 38.306 is removed. Impact analysis Impacted 5G architecture options: MR-DC Impacted functionality ncd-SSB-BWP-Wor Inter-operability: - If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. - If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear whether ncd-SSB-BWP-Wor-r18, rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 and bwpOperationMeasWithoutInterrupt-r18 are applicable for the case that PSCell’s initial BWP or the active BWP is associated with NCD-SSB. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.7.2, 4.2.7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR TS 38.300 CR 0965 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

4.2.7.2	BandNR parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |
| ncd-SSB-BWP-Wor-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on NCD-SSB within active BWP. NCD-SSB within the active DL BWP can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on NCD-SSB, where the NCD-SSB is within the active DL BWP. NOTE: This feature applies only to PCell and PSCell (if configured). It is not applicable to RedCap or eRedCap UEs. | Band | No | N/A | N/A |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |
| rlm-BM-BFD-CSI-RS-OutsideActiveBWP-r18 Indicates whether the UE supports RLM/BM/BFD measurements based on CSI-RS, when CD-SSB is outside active DL BWP. The UE also supports CSI-RS within active DL BWP for RLM/BM/BFD measurements can be QCLed with CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s). The UE supporting this feature shall also indicate support of csi-RS-RLM, beamManagementSSB-CSI-RS and maxNumberCSI-RS-BFD,maxNumberSSB-BFD, maxNumberCSI-RS-SSB-CBD. The UEs indicating the support of this feature group shall not indicate the support of bwp-WithoutRestriction. NOTE: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig. It is not applicable to RedCap or eRedCap UEs. | Band | No | N/A | N/A |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |

Next change

4.2.7.5	FeatureSetDownlink parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |
| bwpOperationMeasWithoutInterrupt-r18 Indicates whether the UE supports RLM/BM/BFD and gapless L3 intra-frequency measurements based on CD-SSB outside active BWP without interruptions. CD-SSB outside active DL BWP but within the bandwidth of the corresponding carrier(s) to be measured can be used as the QCL source for other reference signal. UE performs L3 intra-frequency measurements without gaps based on CD-SSB, where the CD-SSB is outside the active DL BWP but is within the bandwidth of the corresponding carrier(s) to be measured. NOTE 1: The CD-SSB is still within the bandwidth of the carrier configured by SCS-SpecificCarrier of downlinkChannelBW-PerSCS-List in ServingCellConfig. NOTE 2: If a UE is configured with more than one UE-specific DL BWP configurations, the CD-SSB is within the bandwidth of at least one of the UE-specific DL BWP configurations. NOTE 3: Void. NOTE 4: If a UE additionally indicates support of NeedForGap or NeedForGapNCSG and/or NeedForInterruption, the UE shall report no gap and no interruption/no NCSG for intra-frequency measurement. This capability is not applicable to RedCap or eRedCap UEs. | FS | No | N/A | N/A |
| ------------------------------Omit the unrelated part------------------------------------- |  |  |  |  |

End of change