3GPP TSG-RAN WG2 Meeting #129		R2-2501212

Athens, Greece, Feb 17 – 21, 2025

Agenda Item:	8.10.2.1

Source: 	LG Electronics

Title:  	MRO for LTM

Document for:	Discussion

Introduction

In this contribution, we focus on the Rel-19 LTM MRO based on the last agreements.

Discussion

Failure logging

      - Explicit indication

At the RAN2#128, RAN2 made the following agreement [1]:

Agreements - No need to include the specific access type (i.e. RACH-based or RACH-less) in the RLF report explicitly. It can be known implicitly. This can be revisited if RAN2 concludes that there are issues. - Reuse the existing ra-InformationCommon for the RA-based LTM failure. - If RAN3 does not address this meaning that we need a RAN2 solution, add a list indicating which LTM candidates the UE had at RLF.

During the last meeting, there was a discussion about the highlighted part, and we still believe that there are issues regarding this part, so we want to revisit the discussion. We still believe that in some cases, based on implicit access information, the network will not be able to accurately determine which access type the UE used for the entire LTM cell switch attempt.

A typical example is when the UE attempts a RACH-less LTM cell switch based on a dynamic grant (i.e., DG-based RACH-less LTM). In this case, the UE receives a RACH-less LTM CSC (cell switch command) from the source cell and monitors the PDCCH in the target cell for the UL grant. However, for certain reasons from the network side, the target cell may command the UE to use RACH for LTM. Then, if RLF is detected in this case, the UE may include both RACH-less and RACH information as the access information. Without explicit access type information, the network cannot determine which access type the UE attempted until RLF unless the network receives this information explicitly.

Scenario Example:

- The UE receives an LTM cell switch command to perform DG-based RACH-less LTM from the source cell. The UE starts T304.

- The UE applies the RRC Reconfiguration related to LTM.

- The UE performs PDCCH monitoring to obtain the first UL grant for sending the RRC Reconfiguration complete to the target cell (since it is DG-based).

- During PDCCH monitoring on the target cell, the target cell commands the UE to perform RACH.

- The UE attempts to send the RRC Reconfiguration complete message based on RACH information in the target cell.

- The UE repeatedly fails RACH and declares HOF due to T304 expiry.

- The UE performs RLF-logging due to HOF, logging RACH information even though it was initially commanded to perform RACH-less LTM.

By above reason, we think the implicit access type information will leave a fundamental lack of trust in the access type information reported by the UE seems not be enough. Thus, we think that the UE should log explicit access type information to provide clear information preventing any potential confusion from the network side.

Proposal 1: In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

In the above example of DG-based RACH-less LTM, we confirmed that the UE may face a situation where RACH-less LTM CSC is commanded but RACH-based LTM should be performed. We think RAN2 need to consider more about this case where the UE actually tried different access type from the access type commanded by the network in LTM CSC.

In the above example, it is unclear whether the UE should log the RACH actually performed as the access type instead of the RACH-less commanded by the network. If the UE logs the access type as RACH, the network may not know that the LTM CSC has been indicated to RACH-less, the network may misunderstand that the cause of HOF/RLF is due to improper early synchronization before LTM. Otherwise, if the UE logs the access type as RACH-less, the network doesn’t know that the UE had actually tried RACH based LTM until HOF occurs and the network may misunderstand that the UE failed RACH-less procedure during LTM.

Since the above inaccurate logging information cannot help the network’s MRO, we think, for this case, the UE should be able to report both the access type actually performed by the UE and the access type commanded by the network. The access type commanded by the network can only need to be optionally logged if the information is different from the access type actually performed by the UE.

Proposal 2: In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

      - Beam information for RACH-less LTM failure

We think beam information should be newly considered in RLF-report

The network triggers the RACH-less LTM, the network needs to indicate which beam to be applied for the RACH-less LTM. For the beam to be applied for the mobility, the network can indicate SSB index or TCI state ID associated with the target cell. For the association, the network needs to be pre-configured the UE with the SSB configuration/TCI state configuration of the target cell. Then, the UE applies the received beam information, during the RACH-less LTM, for monitoring of PDCCH at the target cell for receiving dynamic grant for transmitting mobility completion message or the received beam information is applied for transmission of mobility completion message via configured grant. The network may indicate TA value to be applied for the mobility or may indicate the UE to apply the UE-calculated TA value for the mobility.

The problem is that the received beam information may not be suitable for the UE due to, e.g., the beam information is outdated, or the UE’s local movement makes the beam information unsuitable. If such unsuitable beam is used for the RACH-less LTM, the mobility fails, i.e. the received TA or the UE calculated TA value may not be suitable for the UE, and in that case, the mobility fails. In case LTM failure happens, the current RLF-report cannot provide any information of the beam information used by the UE for the mobility. Hence, it is difficult for network to analyse the failure reason.

Thus, we propose to include the beam information for LTM trial in RLF-report.

For the beam information, the UE may log below information for logging the failed RACH-less LTM:

- SSB and TCI state information

This is for indicating which SSB index and associated TCI state was used for RACH-less LTM

- Other qualified beam information

This is for indicating whether there is any other qualified beam information for RACH-less LTM

Proposal 3: In the RLF-report for MCG LTM, the UE includes SSB and TCI state with other qualified beam to indicate which beam was used and can be used for LTM in RLF-report.

      - Beam information after LTM success

In the RAN2#128 meeting, RAN2 discussed a scenario where the UE performs LTM cell switching based on a suboptimal beam:

R2-2409754 Further considerations on MRO ZTE Corporation, Sanechips discussion Rel-19 NR_ENDC_SON_MDT_Ph4-Core Focus on P2-P3 Proposal 2 RAN2 introduces a new near failure case where BFR happens soon after a successful LTM cell switch, and UE re-selects to another beam in the same target cell. Proposal 3 Introduce a new SHR trigger: a beam failure together with a BFR happens soon after an LTM cell switch is successfully executed. On P2: - vivo think that there is not need for new triggers. ZTE think that T304 can reused, but that does not exclude other failure causes. Samsung think this causes a lot of additional complexity and need more time to look in to this if we should consider it. LG also want more time. - Nokia think that MRO is per cell not per beam and are not fine with this proposal. On P3: - LG think that the issue is different since if BFR happens then its not really a failure since the UE recovered. Qualcomm think it is a very rare case that the LTM succeeded but the beam failed. Ericsson want to keep this open and want to look more in to this. - Postponed

We need to discuss this scenario further because the network would not know that the selected beam was suboptimal if the LTM was successfully completed.

Even though the LTM was successful based on the suboptimal beam, a beam failure could occur and lead to the BFR procedure. In this scenario, the following two cases should be considered.

- BFR success shortly after LTM cell switching based on the suboptimal beam:

This means that, after LTM cell switching, the UE performs BFR due to the failure of the suboptimal beam and the BFR is successful.

Since we think beam failure and recovery are inherent to normal NR system design and can be expected to occur during the UE's lifetime in a cell (i.e. irrespective of how the UE entered that cell), this case can be considered as normal operation to ensure stability and robustness.

- BFR failure shortly after LTM cell switching based on the suboptimal beam

This means that, after LTM cell switching, the UE performs BFR due to the failure of the suboptimal beam but RLF is declared due to the BFR failure.

In this situation, the initial selection of the suboptimal beam directly results in a connection failure, which should be recognized by the network. While the target cell may be aware of the beam failure event, the source cell remains unaware that the LTM decision resulted in this unstable connection.

Given that previous SON enhancements have addressed RLF shortly after HO as a network optimization case, a similar principle could be applied here if the LTM decision leads to a beam-related connection failure (BFR failure) rather than a successful case.

Based on the above consideration, we think RAN2 needs to discuss the scenario of the LTM cell switching based on the suboptimal beam, and RLF-report needs to be enhanced for the case where the BFR fails soon after the LTM cell switching.

Proposal 4:	In the RLF-report for MCG LTM, the UE logs a suboptimal beam information for the failure case where BFR fails soon after a successful LTM cell switch based on suboptimal beam.

Near failure logging

      - Candidate cell list optimisation

At the RAN2#127, RAN2 made the following agreement to deduce the unnecessary LTM candidate cells [1]:

We aim to log some info to deduce the ltmCandidate (similar like choCandidate) in SHR to indicate whether a neighbour cell is an LTM candidate cell or not, TBD if explicit/implicit.

choCandidate provides information on whether the neighbor cell was a candidate cell when including measurement results for the neighbor cell in SHR [2]:

choCandidate-r17                    ENUMERATED {true}                                     OPTIONAL,

For the purposes of this agreement, there seems to be no reason to provide ltmCandidate information implicitly. To introduce ltmCandidate similarly to choCandidate, we believe it would be simplest to define ltmCandidate as an explicit field of the same enum type.

Proposal 5: In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

In addition, to deduce the LTM candidate cell more properly, we think there is another case to be considered that the candidate cell had selected for LTM cell switch but the candidate cell may be unnecessary candidate cell.

For example, let's assume that the UE has configured candidate cell 1, candidate cell 2, and candidate cell 3 for subsequent conditional mobility. If the UE performs subsequent mobility consecutively in the order of candidate cell 1 to candidate cell 3, and the time spent in candidate cell 2 is very short, there might be no problem even if mobility is performed directly from candidate cell 1 to candidate cell 3. Since the network doesn’t know this information, candidate cell 2 would keep being configured unnecessarily for the subsequent mobility.

To address this issue, we believe it is necessary for the UE to make determinations based on pre-defined/configured information between the network and the UE, such as the actual time the UE had stayed on the candidate cell. If detailed discussion seems necessary, it would be beneficial to discuss this together when discussing candidate cell list optimization above.

Proposal 6: In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	In the RLF report for MCG LTM, RAN2 should revisit and decide whether the UE should explicitly log the access type.

Proposal 2:	In the RLF report for MCG LTM, the UE logs not only the access type but also the commanded access type which was indicated LTM CSC if the access type is different from the commanded access type.

Proposal 3:	In the RLF-report for MCG LTM, the UE includes SSB and TCI state with other qualified beam to indicate which beam was used and can be used for LTM in RLF-report.

Proposal 4:	In the RLF-report for MCG LTM, the UE logs a suboptimal beam information for the failure case where BFR fails soon after a successful LTM cell switch based on suboptimal beam.

Proposal 5:	In the SHR for MCG LTM, ltmCandidate is explicitly defined as the same enum type as choCandidate.

Proposal 6:	In the SHR for MCG LTM, RAN2 discuss that the UE log candidate cells not only for those candidate cells unused but also for those candidate cells selected yet not requiring actual cell switching.

References

- R2-2410916 Report from session on maintenance, CovEnh, SON/MDT and eRedCap Session chair (Ericsson)

- 3GPP TS 38.331 V18.2.0 (2024-06)