3GPP TSG RAN WG2 Meeting #129	 R2-2501218

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.1.2.2

Source:	Futurewei

Title:	Discussion on LCM for UE-Side Model for Beam Management use case

Document for:	Discussion

# Introduction

During RAN2 meeting #128 [7], discussions were focused on the following topics.

- Functionality Applicability

- Functionality Management during mobility

- Data Collection and Model Training

For functionality applicability, the discussion was on the reporting of applicable and non-applicable functionalities, including

- Whether it needs to be reported (and how to report) when the status of a functionality changes, for example, from applicable to non-applicable, or from non-applicable to applicable.

- How to handle a functionality when it becomes non-applicable? For example, can a UE autonomously deactivate the activated functionality when it becomes non-applicable?

- When a functionality becomes non-applicable, should it be reported explicitly or implicit methods can be used?

For functionality management during mobility, the discussion was on whether to support and how to handle functionality reporting during handover. Multiple options have been discussed, including

- Option 1: Target cell requests applicability reporting during or after handover.

- Option 2: Source cell transfers source cell’s applicability information to target cell to assist inference configuration in target cell.

- Option 3: Source cell requests applicability reporting for target cell with target cell’s network-side additional conditions.

The group agreed that functionality management during mobility will be supported; in essence, the target cell will handle the transmission of information such as NW-side additional conditions as part of HO command, and the UE will transmit the applicability report to the target gNB after the handover is completed. In this entire process, existing signalling such as UAI will be used.

For data collection and model training, the discussion was focused on the mechanism of initiating the data collection and whether the network should have the control. Also discussed was whether associated ID should be part of the configuration of data collection. The group agreed that UE can request for data collection and associated ID can be part of the configuration for data collection. The NW controls the initiation and configuration of data collection.

The agreements reached are listed below.

Agreements

- When a functionality configured by the network to be reported via UAI, becomes from non-applicable to applicable, the UE can reports it to the network.   FFS detailed design

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design.

- Applicable functionality reporting at handover is supported with the same RRC procedure that will be specified within a cell, as a baseline, i.e. the NW-side additional conditions and/or the inference configuration related to the target gNB are transmitted by the target gNB as part of the HO command, and the UE in response transmits the applicability report (either in RRCReconfigurationComplete or in UAI) to the target gNB after completing the handover.

- Source cell UAI (as is) can be sent from source cell to target cell using existing signaling.   No further optimizations will be considered in RAN2 related to UAI.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

For the questions RAN2 asked RAN1 on applicable functionality reporting, RAN 1 made the following agreements as documented in [8].

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction - In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete. - In Step 5, NW can optionally configure CSI-ReportConfig for inference configuration in RRCReconfiguration, where the associated ID may be configured in CSI framework as working assumption applied. - Note: Step 5 may be optional if UE has already been configured with CSI-ReportConfig in Step 3 Agreement For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability. Conclusion For the CSI-ReportConfig for inference configuration provided in Step 5, - aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after RRCReconfigurationComplete. - periodic CSI Report is considered as activated after RRCReconfigurationComplete. - Note: UE is not expected to be configured with a CSI-ReportConfig for inference configuration for a non-applicable set of inference parameters or a non-applicable CSI-ReportConfig - Any specification impact is a separate discussion

In this contribution, we will further discuss LCM for UE-side models focusing on the details, clarifications and revisions of the LCM procedures.

# Procedures for UE-side LCM

## Background Information

In RAN2 meeting #127 [5], the procedures for UE-side LCM were discussed, based on the result of post-meeting 126 email discussion. The following agreements were achieved [5].

- Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities.

- Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network may provide NW-side additional condition.

- FFS on the RRC signalling and whether it is mandatory or optional.

3) FFS on configuration (e.g. inference configuration) of supported functionalities.

- FFS on the content of configuration.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality in the following scenarios:

- Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

- As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

  - FFS via UAI or RRCReconfigurationComplete, etc

- Step 5:

1) Network configures inference configuration to UE after applicable functionality reporting, if inference configuration based on supported functionality is not provided in Step 3 (i.e. inference configuration is provided in Step 5).

2) If inference configuration based on supported functionality is provided in Step 3, it is up to network implementation whether to provide an updated configuration or not.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities can be activated at the same time.

  - FFS what is the granularity of functionality

- We will write an LS to RAN1 to provide our agreements and ask specific questions that RAN2 needs to enable progress.

In the following sections, the discussions will be based on the above agreements.

## NW-side additional conditions

In Step 1 and Step 2 of the agreed LCM procedure, NW configures the UE to report supported functionalities and UE reports them. It is our understanding that the main purpose of Step 3 is for the NW and the UE to exchange necessary information so that the UE can decide on the applicable functionalities. For example, the NW provides NW-side additional conditions so that the UE can choose the right applicable functionality. Therefore, we think this step (i.e., the NW providing additional conditions to the UE) should be mandatory, unless the model was trained specifically for this NW.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

## Additional conditions and associated IDs

In the LS RAN2 sent to RAN1, RAN2 asked the following question related to additional conditions.

Q2: What is the content of NW-side additional condition, i.e. is it correct the RAN2 assumption of a NW-side additional condition assumed as associated ID?

In RAN1’s response to RAN2 [8], RAN1 provided the following answer to RAN2’s question.

RAN 1 did not have agreement on the content of NW-side additional condition. RAN1 agreed to support associated ID and it can be used to ensure the consistency of NW-side additional condition across training and inference for UE-sided model for BM-Case 1 and BM Case 2. UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associated ID, while FFS whether/how to define similar properties of a DL Tx beam or beam set/list.

Note that the associated ID was initially designed to reflect the configuration for data collection, and then was extended to a broader concept to ensure the consistency of NW-side additional condition across training and inference for UE-sided model. In both RAN1 and RAN2, there isn’t a definition for associated ID. What is it exactly for? Will it be used for other purposes too? We think it is necessary to clearly define the concept of associated ID and its usage.

Observation 1: The exact definition and usage of Associated ID was not clear and needs to be clarified.

## Determining applicable functionalities

Based on current agreement, UE decides the applicable functionalities after NW’s configuration in Step 3. The RAN2 agreement on this step says that

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

This agreement lists two topics for further study. The first one being whether other configuration can be considered by UE (e.g. inference configuration). This topic has been discussed in RAN1 with results, as a reply to RAN2’s LS. The agreements reached in RAN1 meeting #118bis are listed below.

Agreement

RAN 1 further study the following options for applicability for inference for UE-side model:

Option 1:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2)+3) NW configures one or more CSI-ReportConfig for inference configuration, where the associated ID may be configured in CSI framework as working assumption applied.

  - FFS on whether some IEs in the CSI report configuration can be removed or modified

  - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3

- In Step 4, UE reports applicability(ies) of the above CSI-ReportConfig

  - FFS on one or more of the above CSI-ReportConfig to be reported

- FFS on activation (including when/how) of inference report after obtaining the applicability from UE Step 4.

- FFS: whether Step 5 is needed,

Option 2:

- In Step 3, following configurations are provided from NW to UE:

  - UE is allowed to do UAI reporting via OtherConfig,

  - NW configures one set or multiple sets of inference related parameters

    - Note: the set of inference related parameters is not configured by CSI-ReportConfig

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

  - The associated ID(s) may be configured

    - wherein the associated ID(s) may be

      - FFS: a) part of one set of the inference related parameters, or

      - FFS: b) independently from the one set of the inference related parameters.

- In Step 4, UE reports applicability of the above one or multiple sets of inference related parameters, where the associated ID information may be associated.

- In Step 5, NW configures configuration(s) for CSI report for inference

Option 3:

- In Step 3, following configurations are provided from NW to UE:

  - 1) UE is allowed to do UAI reporting via OtherConfig,

  - 2) The associated ID(s) may be provided to UE, e.g., a new RRC parameter.

- In Step 4, UE reports by UAI

  - the applicable one or multiple sets of inference related parameters may be included.

    - FFS on the set of inference related parameters, at least including:

      - Set A related information

      - Set B related information

      - Report content related information

      - For BM-Case 2,

        - Time instances related information for measurements

        - Time instances related information for prediction

    - Note: not applicable may also be replied by UE

    - Note: if the inference related parameters are not supported for reporting, only the applicability(ies) or not is reported in Step 4.

  - the associated ID(s) may be included

    - FFS: a) as part of the inference related parameters, or

    - FFS: b) independently from the set of the inference related parameters.

- In Step 5, NW configures configuration(s) for CSI report for inference.

Note: There is no impact of configuring CSI report configuration for non-AI beam management in RRCReconfiguration.

We can see that in both Option 1 and Option 2, inference configuration can be done in Step 3. In Option 3, it is done in Step 5. Because all three options have been determined for further study in RAN1, we think RAN2 can just wait for RAN1’s final decision on the option they will choose from the three.

Observation 2: Given that other configurations (e.g. inference configuration) by the UE to determine applicable functionalities after Step 3 is currently being studied in RAN1. RAN2 can make a decision after RAN1 gets back to us.

The second topic for further study is how the applicable functionality is decided if NW-side additional conditions are not provided in step 3. It is common understanding that the additional conditions are used to align NW-side and UE-side conditions/configurations between training and inference stages to ensure expected model performance. The NW should provide additional conditions to the UE if it thinks these conditions are important for the performance. Therefore, when the additional conditions are not provided from the NW, it is reasonable to assume either they have already been considered by the supported functionality reported to the NW in Step 2 (e.g., in the case of NW-specific model) or they are not important for the performance. In this case, applicable functionality can be determined based on the supported functionality without additional conditions. In fact, for each use case or sub use case, there should be only one functionality so the determination should be straightforward (but there could be multiple models).

Observation 3: Applicable functionalities can be determined by selecting from supported functionalities reported to the NW in Step 2, without considering additional conditions after Step 3, if they are not provided from the NW. How this is done is a UE implementation issue.

## The reporting of non-applicable functionalities

In RAN2 meeting #128 [7], the group agreed to further study whether a UE should explicitly report non-applicable functionalities, when there is a change of applicability (see below the agreement).

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design.

In the same meeting, the group agreed that

- When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

We can see from this agreement that when a functionality becomes non-applicable, the NW will receive a report from the UE. Therefore the key to the FFS is in word “explicitly”. We argue that an explicit report of non-applicable functionalities is usually faster and cleaner than an implicit report, which we have not discussed how to do it. A faster, explicit report to the NW of a non-applicable functionality will allow timely deactivation of the functionality.

Proposal 2: UE reports a non-applicable functionality explicitly when there is a change of applicability, i.e., from applicable to non-applicable.

## Activating multiple applicable functionalities at the same time

After Step 5, the applicable functionality may be activated by the received inference configuration in Step 5. In this stage, one of the topics listed for further discussion is whether multiple applicable functionalities can be activated at the same time.

  - FFS if multiple applicable functionalities can be activated at the same time.

However, we think the question itself needs to be clarified first.

When we talk about multiple functionalities, there could be two different things.

- In the first case, when these multiple applicable functionalities are for the same use case/sub-use case, they are likely developed for different applicable situations. In this case we need some mechanisms to differentiate different applicable functionalities (e.g., functionality ID).

- In the second case, when these multiple applicable functionalities are not for the same use case/sub-use case, for example, one for beam management and the other for CSI prediction, then there is no reason to activate them at the same time, for the same use case/sub use case. Note that it should be always allowed to activate different functionalities for different use cases or sub use cases.

Based on the above discussion, we think the FFS belongs to the first case. We hence propose the following revision to the agreement. After this clarification, whether they can be activated at the same time can be further studied.

Proposal 3: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

## UE-side Data collection

During RAN2 meeting #128 [7], the following agreements have been made on UE-side data collection.

- For BM use case for UE-side model, data collection related configuration(s) (e.g., measurement resources configuration) and associated ID(s) can be included in training data collection configuration.

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- The network can provide the data collection configuration (at any point in time), with or without UE request.

- The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration.

These agreements allow the network to send instructions to UE to start/stop data collection, together with the configuration for data collection. What needs further study is whether an indication from UE to network is needed when UE can’t perform data collection as the network configured.

As was discussed, common reasons for a UE to become incapable of data collection are

- The UE is running out of storage space;

- The UE is low in battery level.

We believe it is necessary for the UE to send in indication to the NW to inform the NW that it is no longer able to collect data. This is especially important if the UE is not allowed to stop data collection by itself, as in this case it needs to wait for the NW’s instruction to stop data collection. In addition, we think the reason code for the UE’s inability of data collection should also be reported to the NW so that the NW understands the UE’s situation for what to do or not to do later.

Proposal 4: An indication from UE to network is needed when UE can’t perform data collection based on received configuration.

## Reporting of UE internal capability/capacity

Besides AI/ML-based UE capability reporting (which associates with AI/ML functionality), another important aspect the UE needs to report to the NW is the UE’s internal capability or capacity for model inference, for example, its computational power and memory. Note this is different from AI/ML-based UE capability reporting; it is used for the NW to decide whether the UE will be able to handle the task of certain model inference.

This is also touched in the agreements, between Step 3 and Step 4, as listed below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

The “UE-side additional conditions (internally known by UE)” in this proposal can include the UE’s capability or capacity for model inference. we believe this information should also be made known to the NW (not only to the UE itself) for various reasons. This information may include UE’s computational power, memory, storage, model inference time, battery level etc. that are applicable for AI/ML functionality operations.

With this information, the NW can have a better understanding of the resources available at the UE side and activate/deactivate AI/ML-based functionality at the right time/situation. For example, if the UE’s current computational power or memory is not able to support the inference of a certain functionality, the NW will not instruct the UE to activate the functionality.

To capture this aspect, we proposed a revised agreement for the above text and Step 4.

Proposal 5: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# Conclusions

In this contribution, we discussed our views on LCM for UE-side models for the beam management use cases. Based on the discussions in the previous sections, our proposals are as follows.

Observation 1: The exact definition and usage of Associated ID was not clear and needs to be clarified.

Observation 2: Given that other configurations (e.g. inference configuration) by the UE to determine applicable functionalities after Step 3 is currently being studied in RAN1. RAN2 can make a decision after RAN1 gets back to us.

Observation 3: Applicable functionalities can be determined by selecting from supported functionalities reported to the NW in Step 2, without considering additional conditions after Step 3, if they are not provided from the NW. How this is done is a UE implementation issue.

Proposal 1: Revised “Step 3” as below.

- “Step 3”: Following configurations are provided from NW to UE:

1) UE is allowed to do UAI reporting via OtherConfig.

2) Network shall may provide NW-side additional condition if applicable.  FFS on the RRC signalling. and whether it is mandatory or optional.

3) FFS on The configuration (e.g. inference configuration) of supported functionalities is up to individual use case.

Proposal 2: UE reports a non-applicable functionality explicitly when there is a change of applicability, i.e., from applicable to non-applicable.

Proposal 3: Revise the procedures for UE-side LCM as below.

- The applicable functionality may be activated by receiving its inference configuration when it is provided in Step 5.

  - FFS the initial activation state.

  - FFS on initial state of applicable functionality if inference configuration of supported functionality is provided in Step 3.

  - FFS on additional L1/L2 signalling for activation/deactivation.

  - FFS if multiple applicable functionalities for the same use case/sub use case can be activated at the same time.

  - FFS what is the granularity of functionality

Proposal 4: An indication from UE to network is needed when UE can’t perform data collection based on received configuration.

Proposal 5: Revise the procedures for UE-side LCM as below.

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE, including UE’s capabilities and capacities that are related to AI/ML functionality operations) and model availability in device.

  - FFS whether other configuration can be considered by UE (e.g. inference configuration).

  - FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

- “Step 4”: UE reports applicable functionality and its internal capabilities/capacities in the following scenarios:

  - Upon being configured to provide applicable functionality and upon change of applicable functionality via UAI

  - As response to NW-side additional condition requesting applicable functionality reporting in step 3, FFS other network configuration (e.g. inference configuration).

    - FFS via UAI or RRCReconfigurationComplete, etc

# References

Chair’s notes, R2_126_ChairNotes_24-05-24_final, RAN2 meeting #126, Fukuoka, May 2024

RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN meeting #102, Qualcomm (moderator)

RP-233946, 3GPP TR 38.843, Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18) v2.0.1 (2023-12)

Chair’s notes, R2_125bis_ChairNotes_24-04-19_final, RAN2 meeting #125bis, Changsha, April 2004

Chair’s notes, R2_127_ChairNotes_ 24-08-23_17-00_eom, RAN2 meeting #127, Maastricht, Aug 2024

Chair’s notes, R2_127bis_ChairNotes_ 24-10-18_15-45-_eom_clean, RAN2 meeting #127bis, Hefei, Oct 2024

Chair’s notes, R2_128_ChairNotes_24-11-22_17-00_eom, RAN2 meeting #128, Orlando, Nov 2024

R2-2500011 (R1-2410898), Reply LS on applicable functionality reporting for beam management UE-sided model, TSG RAN WG1, Orlando, Nov 2024