3GPP TSG-RAN WG2 Meeting #129	R2-2501226

Athens, Greece, Feb. 17th – 21st, 2025

Source:        ZTE Corporation

Title:           Consideration on the STx2P UE Feature (40-6-3a-1/40-6-3b-2)

Agenda item:   7.0.1

Document for:  Discussion and Decision

- Introduction

In the post-email discussion of the last meeting, the UE capability mega CR was implemented based on the RAN1 feature list [1] [2]. However, the implementation on the following2 features are postponed because of 2 issues.

| 40-6-3a-1 | 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |
| --- | --- | --- | --- | --- |

| 40-6-3b-2 | 1. Require additional timeline to process multiple TBs for non-codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |
| --- | --- | --- | --- | --- |

The first issue is about whether the value for the 15k/30k SCS is needed while the second issue is whether the choice or sequence structure should be used.

In this contribution, we share our further views on these two issues.

- Discussion

In this chapter, we will discuss the above-mentioned two issues one by one based on the original/draft implementation in the post email discussion. Since Feature 40 - 6 - 3b - 2 follows the same logic as Feature 40 - 6 - 3a - 1, we take Feature 40 - 6 - 3a - 1 as an example.

FeatureSetUplinkPerCC-v1840 ::= SEQUENCE { -- R1 40-6-3a-1: UE STxMP processing capability for codebook twoPUSCH-CB-MultiDCI-STx2P-DG-DG-AdditionalTime-r18 SEQUENCE { scs-15kHz-r18 ENUMERATED{sym1, sym2, sym4} OPTIONAL, scs-30kHz-r18 ENUMERATED{sym1, sym2, sym4, sym8} OPTIONAL, scs-60kHz-r18 ENUMERATED{sym2, sym4, sym8, sym16} OPTIONAL, scs-120kHz-r18 ENUMERATED{sym4, sym8, sym16, sym32} OPTIONAL, scs-480kHz-r18 ENUMERATED{sym16, sym32, sym64, sym128} OPTIONAL, scs-960kHz-r18 ENUMERATED{sym32, sym64, sym128,sym256} OPTIONAL }

| twoPUSCH-CB-MultiDCI-STx2P-DG-DG-AdditionalTime-r18 Indicates whether the UE supports additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG. A UE supporting this feature shall also indicate support of twoPUSCH-CB-MultiDCI-STx2P-DG-DG-r18. This capability is also applied to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for CG+DG if UE supports twoPUSCH-CB-MultiDCI-STx2P-CG-DG-r18. NOTE: A UE reports this capability independently for each minimum SCS between SCS of the scheduling DCI and SCS of the scheduled PUSCH. | FSPC | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

Issue 1: Whether the value for the 15k/30k SCS is needed considering that this capability was supported on FR2 only

This capability was used to indicate whether the UE supports additional timeline for the MDCI STx2P case. According to the following note:

NOTE: A UE reports this capability independently for each minimum SCS between SCS of the scheduling DCI and SCS of the scheduled PUSCH.

If the SCS of the scheduling DCI is 120K and the SCS of the scheduled PUSCH is 480K, the UE should report the value corresponding to the 120K. Thus whether the value for the 15k/30k SCS is needed would depend on whether cross-carrier scheduling is supported. For example, if the FR2 CC PUSCH can be scheduled by the FR1 CC, then the SCS of the scheduling DCI could be 15k/30k SCS and the UE should report the value for the 15k/30k SCS. However according to RAN1 discussion, there is a conclusion in the RAN1#112 meeting that any combination of Rel-16 MDCI MTRP PDSCH/PUSCH operation and cross-carrier scheduling is not supported.

RAN1#112 R1-2301973 Summary of Type-1 HARQ-ACK codebook when MDCI based MTRP operation Moderator (ZTE) Conclusion It is RAN1 understanding that any combination of Rel-16 MDCI MTRP PDSCH/PUSCH operation and cross-carrier scheduling is not supported.

Based on this conclusion, the value for the 15k/30k SCS is not needed considering that this capability was supported on FR2 only. The same logic was also used for the RAN1 Feature 40-6-3b-2. However, these are RAN1 features, an LS needs to be sent to confirm RAN1’s understanding of this issue.

Proposal 1: Send an LS to RAN1 to confirm whether the cross-carrier scheduling is supported for MDCI MTRP PDSCH/PUSCH operation.

Proposal 1a: If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the value for the 15k/30k SCS is not needed considering that these 2 capabilities were supported on FR2 only.

Issue 2: Whether the choice or sequence structure should be used

Considering that this capability is reported per FSPC, the SCS of the DL and UL would be reported separately for a CC, e.g.

- UL: FeatureSetUplinkPerCC->supportedSubcarrierSpacingUL

- DL: FeatureSetDownlinkPerCC->supportedSubcarrierSpacingDL

If there is no cross-carrier scheduling, the UE can determine the minimum SCS based on these two capabilities and report the corresponding value.

Proposal 2: If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the UE can determine the minimum SCS based on the existing two capabilities (i.e. UL: FeatureSetUplinkPerCC->supportedSubcarrierSpacingUL/DL: FeatureSetDownlinkPerCC-> supportedSubcarrierSpacingDL)and report the corresponding value.

If proposal 2 is confirmed, a choice structure can be used instead of the sequence structure.

Proposal 2a: If proposal 2 is confirmed, a choice structure can be used instead of the sequence structure for the RAN1 Feature 40-6-3a-1/40-6-3b-2.

However, according to the current RAN1 feature table, the UE would report candidate value independently for each SCS in units of symbols, which would conflict with the above RAN2 understanding. Thus, it’s better to include these proposals (including proposal 1a) in the LS for clarification.

Proposal 3: Send an LS to RAN1 to confirm the understanding in proposal 1a/2/2a.

Based on the above proposals, a draft LS is also prepared as in [3].

Proposal 4: Approve the draft LS as in [3].

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: Send an LS to RAN1 to confirm whether the cross-carrier scheduling is supported for MDCI MTRP PDSCH/PUSCH operation.

Proposal 1a: If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the value for the 15k/30k SCS is not needed considering that these 2 capabilities were supported on FR2 only.

Proposal 2: If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the UE can determine the minimum SCS based on the existing 2 capabilities (i.e. UL: FeatureSetUplinkPerCC->supportedSubcarrierSpacingUL/DL: FeatureSetDownlinkPerCC-> supportedSubcarrierSpacingDL)and report the corresponding value.

Proposal 2a: If proposal 2 is confirmed, a choice structure can be used instead of the sequence structure for the RAN1 Feature 40-6-3a-1/40-6-3b-2.

Proposal 3: Send an LS to RAN1 to confirm the understanding in proposal 1a/2/2a.

Proposal 4: Approve the draft LS as in [3].

- Reference

- R1-2409043 Updated RAN1 UE features list for Rel-18 NR after RAN1_118bis  RAN1

- R1-2410666 Updated RAN1 UE features list for Rel-18 NR after RAN1#119  RAN1

- R2-2501227  [Draft] LS on the STx2P UE Feature  To RAN1     ZTE