3GPP TSG-RAN WG2 Meeting #129	R2-2501227

Athens, Greece, Feb. 17th – 21st, 2025

Title:	[Draft] LS on the STx2P UE Feature

Response to:

Release:	Rel-18

Work Item:	NR_MIMO_evo_DL_UL

Source:	ZTE [to be RAN2]

To:	RAN1

Cc:

Contact person:	Wenting Li

Li.wenting@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1	Overall description

In RAN2#129, companies discussed two issues regarding the following two RAN1 STx2P UE Features:

| 40-6-3a-1 | 1. Require additional timeline to process multiple TBs for codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |
| --- | --- | --- | --- | --- |
| 40-6-3b-2 | 1. Require additional timeline to process multiple TBs for non-codebook multi-DCI based STx2P PUSCH+PUSCH for DG+DG Note. This FG can be applied for CG+DG also if UE can support those FG. | Per FSPC | FR2 only | candidate values: UE reports candidate value independently for each SCS in unit of symbols For 15kHz SCS: {1,2,4} For 30kHz SCS: {1,2,4,8} For 60kHz SCS: {2,4,8,16} For 120kHz SCS: {4,8,16,32} For 480kHz SCS: {16,32,64,128} For 960kHz SCS: {32,64,128,256} Note: SCS above is the minimum between SCS of the scheduling DCI and SCS of the scheduled PUSCH. |

- Issue 1: Whether the value for the 15k/30k SCS is needed considering that these two capabilities are supported on FR2 only.

- Issue 2: Whether the choice or sequence structure should be used for these two capabilities.

For these two issues, RAN2 thinks that the key point is whether the cross-carrier scheduling is supported for MDCI MTRP PDSCH/PUSCH operation and concluded that:

=> If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the value for the 15k/30k SCS is not needed considering that these two capabilities were supported on FR2 only. => If the cross-carrier scheduling is not supported for MDCI MTRP PDSCH/PUSCH operation, for the RAN1 Feature 40-6-3a-1/40-6-3b-2, the UE can determine the minimum SCS based on the existing two capabilities (i.e. UL: FeatureSetUplinkPerCC->supportedSubcarrierSpacingUL/DL: FeatureSetDownlinkPerCC-> supportedSubcarrierSpacingDL)and report the corresponding value, thus a choice structure can be used instead of the sequence structure for these two capabilities. E.g. FeatureSetUplinkPerCC-v1840 ::= SEQUENCE { -- R1 40-6-3a-1: UE STxMP processing capability for codebook twoPUSCH-CB-MultiDCI-STx2P-DG-DG-AdditionalTime-r18 CHOICE{ scs-60kHz-r18 ENUMERATED{sym2, sym4, sym8, sym16} OPTIONAL, scs-120kHz-r18 ENUMERATED{sym4, sym8, sym16, sym32} OPTIONAL, scs-480kHz-r18 ENUMERATED{sym16, sym32, sym64, sym128} OPTIONAL, scs-960kHz-r18 ENUMERATED{sym32, sym64, sym128,sym256} OPTIONAL }

On whether the cross-carrier scheduling is supported for MDCI MTRP PDSCH/PUSCH operation, RAN2 also notices that there was a related RAN1 agreement for R16:

RAN1#112 R1-2301973 Summary of Type-1 HARQ-ACK codebook when MDCI based MTRP operation Moderator (ZTE) Conclusion It is RAN1 understanding that any combination of Rel-16 MDCI MTRP PDSCH/PUSCH operation and cross-carrier scheduling is not supported.

RAN2 respectfully asks RAN1 to confirm whether the combination of the cross-carrier scheduling and MDCI MTRP PDSCH/PUSCH operation is supported in R18.  If not, RAN2 would like RAN1 to confirm RAN2’s above conclusions.

2	Actions

To SA2, CT1 group

ACTION: 	RAN2 respectfully asks RAN1 to confirm whether the combination of the cross-carrier scheduling and MDCI MTRP PDSCH/PUSCH operation is supported in R18.  If not, RAN2 would like RAN1 to confirm RAN2’s above conclusions.

3	Dates of next TSG-RAN WG2 meetings

TSG-RAN2 Meeting #129bis             07 - 11 April 2025	                             China, CN

TSG-RAN2 Meeting #130	19 - 23 May 2025	Malta , MT