3GPP TSG-RAN2 Meeting #129	R2- 2501243

# Athens, Greece, Feb 17th – 21st, 2025

Agenda item:		8.7.4.2 (NR_XR_Ph3)

Source:	LG Electronics Inc.

Title: 	Discussion on DSR enhancement for XR

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 XR[1], it is agreed to specify the DSR enhancements with multiple pairs of remaining time and buffer size for an LCG.

- Specify Enhancements for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs, as follows [RAN2]: - Specify additional Logical Channel priority handling using delay/deadline information of packets; - Specify enhanced DSR (Delay Status Report) reporting with multiple pairs of remaining time and buffer size for an LCG.

This document continues discussion on further details of DSR enhancement for XR.

2.	Discussion

In Rel-19, it is agreed that the Rel-19 DSR may report remaining time information with multiple reporting time thresholds. In RAN2#127bis meeting [2], it is agreed to include data volume and the shortest remaining time among buffered data in each portion, for each portion with BS>0, for each LCG:

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion.

For the details of Rel-19 DSR, “buffer size of each portion” and “PDCP SDUs buffered in each portion” should be defined clearly.

On the other hand, in RAN2#128 meeting [3], it is agreed to configure reporting time threshold independently from the triggering time threshold:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR.

In other words, “buffer size of each portion” reported in Rel-19 DSR is determined independently from the data volume of delay-critical data, which is reported in Rel-18 DSR.

Therefore, in order to avoid any confusion from existing terminologies used for Rel-18 DSR (e.g., delay-critical data, for Rel-18 DSR MAC CE), new terminologies are needed to identify PDCP/RLC data associated with reporting time threshold. For discussion purpose for Rel-19 DSR with multiple reporting thresholds, it is suggested to use following terminologies:

- “delay-reporting data” refers to “PDCP SDUs buffered in each portion” in RAN2#127bis agreement, i.e., PDCP/RLC data associated with a reporting threshold.

- “delay-reporting data volume” refers to “buffer size of each portion” in RAN2#127bis agreement, i.e., the data volume to be reported in Rel-19 DSR MAC CE associated with a reporting threshold.

Proposal 1. For discussion purpose, use following terminologies for Rel-19 DSR:

- delay-reporting data: PDCP/RLC data associated with a reporting threshold.

- delay-reporting data volume: data volume to be reported in Rel-19 DSR MAC CE associated with a reporting threshold.

Meanwhile, in RAN2#128 meeting [3], it is agreed to support including the non-delay critical data ahead of delay-critical data in buffer size calculation of Rel-19 DSR, with a capability indication:

The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

However, the above agreement only considers the non-delay critical data ahead of delay-critical data, which is determined by DSR triggering threshold. In our understanding, the intention of the above agreement is to report the exact buffer size to transmit all data whose remaining time till PDCP discard expiry is less than reporting time threshold for Rel-19 DSR MAC CE. In this sense, the RAN2#128 agreement should be applied for the delay-reporting data associated with each reporting time threshold, rather than delay-critical data which is determined by triggering threshold. Therefore, to avoid the confusion on the existing delay-critical data definition, the intention of the above agreement should be interpreted as “the UE may also support including other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 2. Clarify RAN2#128 agreement as “the UE may also support including other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

For the details of the delay-reporting data volume calculation, it should be separately discussed for the cases 1) when the UE does not support to include other data ahead of delay-reporting data in the buffer size calculation for DSR and 2) when the UE supports to include other data ahead of delay-reporting data in the buffer size calculation for DSR.

Delay-reporting data volume calculation without considering other data ahead of delay reporting data

In Rel-19 DSR MAC CE with multiple reporting time threshold, the multiple pairs of buffer size field and remaining time field can be reported. Here, each buffer size field should include delay reporting data volume associated with each reporting threshold.

In order to avoid duplicated reporting in multiple buffer size fields in Rel-19 DSR MAC CE, a PDCP/RLC data should be considered in delay-reporting data volume calculation associated with only up to one reporting time thresholds. In other words, any SDU should not be reported both in delay-reporting data volume associated with reporting threshold 1 and delay-reporting data volume associated with reporting threshold 2.

In this sense, when the PDU set discard is not configured, the delay-reporting data volume for each reporting threshold should be determined based on the PDCP/RLC data whose remaining time until PDCP discard timer expiry is between the two consecutive reporting thresholds. For simplicity, it is suggested to configure the reporting thresholds in ascending order and to define ‘delay-reporting data associated with i:th reporting threshold’ as PDCP/RLC data whose remaining time is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold (for i > 1) or lager than 0 (for i = 1).

Proposal 3. When the multiple reporting thresholds are configured, the reporting thresholds are configured in ascending order.
Proposal 4. If PDU set discard is not configured, delay reporting data associated with i:th reporting threshold is defined as follows:

- For i > 1, PDCP/RLC data whose remaining time till PDCP discard timer expiry is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold.

- For i = 1, PDCP/RLC data whose remaining time till PDCP discard timer expiry is less than i:th reporting threshold and larger than 0.

When the PDU set discard is configured, when the PDCP discard timer of a PDCP SDU expires, all PDCP SDUs belonging to the PDU set is discarded. In this sense, when the delay-critical data is defined in Rel-18 DSR, delay-critical PDCP SDU is defined as a PDCP SDU belonging to a PDU Set of which at least one PDCP SDU has the remaining time till PDCP discard timer expiry is less than the triggering threshold (i.e., remainingTimeThreshold). Similarly, if PDU set discard is configured, the delay-reporting data associated with i:th reporting threshold is defined based on the shortest remaining time of among PDCP SDUs belonging to a PDU Set. Specifically, when the PDU set discard is configured, delay-reporting data associated with i:th reporting threshold is PDCP/RLC data belonging to a PDU set, when shortest remaining time till PDCP discard timer expiry of the PDU set is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold (for i > 1) or larger than 0 (for i = 1).

Proposal 5. If PDU set discard is configured, delay-reporting data associated with i:th reporting threshold is defined based on the shortest remaining time till PDCP discard timer expiry among the PDCP SDUs belonging to the PDU Set:

- For i > 1, PDCP/RLC data belonging to a PDU set, when the shortest remaining time till PDCP discard timer expiry of the PDU Set is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold.

- For i = 1, PDCP/RLC data belonging to a PDU set, when the shortest remaining time till PDCP discard timer expiry of the PDU Set is less than i:th reporting threshold and larger than 0.

Based on Proposal 4 and Proposal 5, the PDCP/RLC data whose remaining time until PDCP discard timer expiry is between the two consecutive reporting thresholds is defined as ‘delay-reporting data associated with i:th reporting threshold.’ Then, in order to calculate delay-reporting data volume for i:th reporting threshold, the delay-reporting data associated with i:th reporting threshold should be considered, which is defined based on whether the PDU set discard is configured.

Proposal 6. When the UE does not include other data ahead of delay-reporting data in Rel-19 DSR, the UE should consider delay-reporting data associated with i:th reporting threshold for delay-reporting data volume calculation associated with i:th reporting threshold.

With Proposal 3 to Proposal 6, for an example case of UE the buffer status as depicted in Figure 1, delay reporting data volume associated with each reporting threshold is calculated as follows. Here, non-delay critical SDU does not trigger DSR procedure, e.g., low importance data when the PSI-based discard is activated:

Figure 1.

- When the PDU set discard is not configured,

  - Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 150 bytes

    - Delay reporting PDCP SDU: SDU2

  - Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 700 bytes

    - Delay reporting PDCP SDU: SDU3, SDU4

  - Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 1650 bytes

    - Delay reporting PDCP SDU: SDU6, SDU7, SDU9

- When the PDU set discard is configured,

  - Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 600 bytes

    - Delay reporting PDCP SDU: SDU2, SDU4, SDU7

  - Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 1900 bytes

    - Delay reporting PDCP SDU: SDU3, SDU6, SDU9

  - Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 0 bytes

    - Delay reporting PDCP SDU: None

Delay-critical data volume calculation considering other data ahead of delay-reporting data

In RAN2#128 meeting, it is agreed to indicate the UE capability to the network on whether the UE may support including other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR. Since the intention of the agreement is to have a common understanding between the network and the UE on delay-reporting data volume calculation, it would be straightforward to define the UE behaviour on Rel-19 DSR based on the network configuration, including whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation. Here, the network may configure whether to include other data ahead of delay-reporting data, after the network receives the UE capability on whether the UE supports the corresponding functionality.

Proposal 7. In the Rel-19 DSR configuration, the network may configure whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation, based on UE capability signalling.

As discussed above, the delay-reporting data associated with i:th reporting threshold can be defined as follows:

- If PDU set discard is not configured, PDCP/RLC data whose remaining time till PDCP discard timer expiry is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold (for i > 1) or lager than 0 (for i = 1).

- If PDU set discard is configured, PDCP/RLC data belonging to a PDU set, when shortest remaining time till PDCP discard timer expiry of the PDU set is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold (for i > 1) or larger than 0 (for i = 1).

Then, when the UE considers other data ahead of delay reporting data for Rel-19 DSR, the delay reporting data volume should further consider all the buffered data ahead of last buffered delay-reporting data associated with i:th reporting threshold, in addition to the delay-reporting data associated with i:th reporting threshold. Given that the transmitting PDCP entity allocates COUNT value when the PDCP entity receives PDCP SDU from the upper layer, it can be assumed that COUNT value is allocated for PDCP SDUs in ascending order, i.e., the order of buffered data can be identified based on the COUNT value. Therefore, in order to report all the buffered data ahead of last buffered delay-reporting data associated with i:th reporting threshold, delay-reporting data volume should include PDCP SDUs associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU.

On the other hand, if a PDCP SDU is already considered in delay-reporting data volume associated with lower reporting threshold, the PDCP SDU should not be considered as delay-reporting data volume associated with any other reporting threshold, in order to avoid duplicated reporting. For example, if the buffer status at UE as depicted in Figure 1, SDU1 should not be considered as delay-reporting data volume associated with reporting threshold 2, even though the COUNT value of SDU1 is smaller than the largest COUNT value delay-reporting data associated with reporting threshold 2 (e.g., SDU 4 when PDU set discard is not configured).

Meanwhile, when the UE further considers other data (i.e., which is not identified as a delay-reporting data) ahead of last buffered delay-reporting data for delay-reporting data volume calculation, the UE does not need to consider whether the other data belongs to a PDU set, since it is not related to delay budget. For example, if the UE buffer status is as in Figure 1 and PDU set discard is configured, even though SDU3 and SDU6 in PDU set 2 are considered in delay-reporting data volume associated with reporting threshold 1, it is not required to further consider SDU9 in the PDU set 2 for delay-reporting data volume associated with reporting threshold 1. For this case, it is sufficient to consider SDU9 in delay-reporting data volume associated with reporting threshold 2, since the delay budget for PDU set 2 is larger than the reporting threshold 1 anyway.

In this sense, for delay-reporting data volume calculations considering the other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data, the same principle can be applied regardless of whether the PDU set discard is configured or not.

Proposal 8. When the UE includes other data ahead of delay reporting data in Rel-19 DSR, the UE should consider followings for delay-reporting data volume calculation associated with i:th reporting threshold:

- delay-reporting data associated with i:th reporting threshold, and are not considered as delay-reporting data volume associated with any of the k:th reporting threshold where k < i;

- PDCP SDUs associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU associated with i:th reporting threshold, and are not considered as delay-reporting data volume associated with any of the k:th reporting threshold where k < i;

For example, if the UE buffer status is as depicted in Figure 1 and PDU set is not configured, the delay-reporting data associated with each reporting threshold is calculated as follows if the UE considers other data ahead of delay-reporting data:

Figure 2.

- Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 250 bytes

  - Delay reporting PDCP SDU: SDU2

  - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU1

- Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 700 bytes

  - Delay reporting PDCP SDU: SDU3, SDU4

- Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 3300 bytes

  - Delay reporting PDCP SDU: SDU6, SDU7, SDU9

  - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU5, SDU8

On the other hand, if the UE buffer status is as depicted in Figure 1 and PDU set discard is configured, the delay-reporting data associated with each reporting threshold is calculated as follows if the UE considers other data ahead of delay-reporting data. Note that SDU3 and SDU6 are considered as delay-reporting data volume associated with the 1st reporting threshold, while the SDU9 is considered as delay-reporting data volume associated with the 2nd reporting threshold, even though SDU3, SDU6, SDU9 belongs to the same PDU set.

Figure 3.

- Delay reporting data volume associated with the 1st reporting threshold (= 10ms): 2650 bytes

  - Delay reporting PDCP SDU: SDU2, SDU4, SDU7

  - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU1, SDU3, SDU5, SDU6

- Delay reporting data volume associated with the 2nd reporting threshold (= 20ms): 1500 bytes

  - Delay reporting PDCP SDU: SDU9

  - Other data (i.e., PDCP SDU ahead of delay-reporting data): SDU8

- Delay reporting data volume associated with the 3rd reporting threshold (= 30ms): 0 bytes

Handling of PDCP/RLC control PDU and PDCP/RLC data to be retransmitted for delay-reporting data volume

In Rel-18 XR, it was discussed whether the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs should be considered as delay-critical PDCP data volume. Based on the discussion in Rel-18 XR, it was agreed to include the PDCP control PDUs as delay-critical PDCP data volume, since PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs are expected to be prioritised over other delay-critical PDCP data PDUs. Therefore, in order to keep the consistency with the delay-critical data volume definition discussed in Rel-18 XR, the PDCP control PDUs and PDCP SDUs/PDCP Data PDUs to be retransmitted for AM DRBs should be considered as delay-reporting PDCP data volume associated with the shortest reporting time threshold.

Proposal 9. The PDCP layer should consider followings for delay-reporting PDCP data volume calculation associated with first reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Similarly, in RLC layer, followings should be considered as delay-reporting RLC data volume associated with the 1st reporting threshold, since the AM RLC entity prioritize RLC Control PDU and RLC data PDUs containing previously transmitted RLC SDUs or RLC SDU segments over other RLC AMD PDUs:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Proposal 10. The RLC layer should consider followings for delay-reporting RLC data volume calculation associated with first reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Rel-19 DSR MAC CE vs. Rel-18 DSR MAC CE

In RAN2#127bis meeting [2], it was remained as FFS on whether the Rel-18 DSR MAC CE can be transmitted when there is at least one LCG configured with multiple reporting time thresholds, considering that Rel-18 DSR MAC CE has smaller size than the Rel-19 DSR MAC CE.

FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

Meanwhile, on the details of Rel-19 DSR MAC CE, it is agreed to include each portion of remaining time only if the amount of buffered data is larger than 0, in order to reduce the overhead of Rel-19 DSR MAC CE.

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: - Buffer size of data volume in each portion - Shortest remaining time among PDCP SDUs buffered in each portion.

In other words, if the amount of buffered data for a portion is zero, corresponding buffer size field and the remaining time field would not be included in the enhanced DSR MAC CE.

During RAN2#127bis meeting, some companies have suggested to transmit Rel-18 DSR MAC CE if the amount of data volume exists in only one portion of the remaining time for a LCG, in order to reduce the overhead of enhanced DSR MAC CE. However, in our understanding, there would be no big size difference between the Rel-19 DSR MAC CE and Rel-18 DSR MAC CE in this case, since only one pair of buffer size field and the remaining time field is included in Rel-19 DSR MAC CE. Therefore, the benefits of transmitting Rel-18 DSR MAC CE in this case would be marginal.

On the other hand, if the UE is allowed to transmit the Rel-18 DSR MAC CE, e.g. based on the grant size and/or buffer status, the additional complexity is expected due to mixed procedure of Rel-19 DSR and Rel-18 DSR. In addition, the additional discussion is needed in order to determine in which case the Rel-18 DSR MAC CE can be transmitted and whether there is any benefit to transmit Rel-19 DSR MAC CE in each case. For example, even though the amount of data is non-zero for only one portion of remaining time, transmitting Rel-19 DSR MAC CE could be beneficial, since it may provide detailed delay information with finer granularity.

Therefore, for simplicity, it would be better to not mix the Rel-18 DSR MAC CE and Rel-19 DSR MAC CE, i.e., always use Rel-19 DSR MAC CE when the UE needs to report buffer status of at least one LCG configured with reporting thresholds.

Proposal 11. When the UE needs to report buffer status of at least one LCG configured with reporting thresholds, the UE always transmits Rel-19 DSR MAC CE.

3.	Conclusion

In this contribution, it is discussed for the DSR enhancement for XR. This document includes following proposals.

Proposal 1. For discussion purpose, use following terminologies for Rel-19 DSR:

- delay-reporting data: PDCP/RLC data associated with a reporting threshold.

- delay-reporting data volume: data volume to be reported in Rel-19 DSR MAC CE associated with a reporting threshold.

Proposal 2. Clarify RAN2#128 agreement as “the UE may also support including other data (i.e., which is not identified as a delay-reporting data) ahead of delay-reporting data for buffer size calculation of Rel-19 DSR, based on the capability indication”

Proposal 3. When the multiple reporting thresholds are configured, the reporting thresholds are configured in ascending order.
Proposal 4. If PDU set discard is not configured, delay reporting data associated with i:th reporting threshold is defined as follows:

- For i > 1, PDCP/RLC data whose remaining time till PDCP discard timer expiry is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold.

- For i = 1, PDCP/RLC data whose remaining time till PDCP discard timer expiry is less than i:th reporting threshold and larger than 0.

Proposal 5. If PDU set discard is configured, delay-reporting data associated with i:th reporting threshold is defined based on the shortest remaining time till PDCP discard timer expiry among the PDCP SDUs belonging to the PDU Set:

- For i > 1, PDCP/RLC data belonging to a PDU set, when the shortest remaining time till PDCP discard timer expiry of the PDU Set is less than i:th reporting threshold and larger than or equal to (i-1):th reporting threshold.

- For i = 1, PDCP/RLC data belonging to a PDU set, when the shortest remaining time till PDCP discard timer expiry of the PDU Set is less than i:th reporting threshold and larger than 0.

Proposal 6. When the UE does not include other data ahead of delay-reporting data in Rel-19 DSR, the UE should consider delay-reporting data associated with i:th reporting threshold for delay-reporting data volume calculation associated with i:th reporting threshold.

Proposal 7. In the Rel-19 DSR configuration, the network may configure whether to include other data ahead of delay-reporting data for delay-reporting data volume calculation, based on UE capability signalling.

Proposal 8. When the UE includes other data ahead of delay reporting data in Rel-19 DSR, the UE should consider followings for delay-reporting data volume calculation associated with i:th reporting threshold:

- delay-reporting data associated with i:th reporting threshold, and are not considered as delay-reporting data volume associated with any of the k:th reporting threshold where k < i

- PDCP SDUs associated with a COUNT value smaller than the largest COUNT value of the delay-reporting PDCP SDU associated with i:th reporting threshold, and are not considered as delay-reporting data volume associated with any of the k:th reporting threshold where k < i

Proposal 9. The PDCP layer should consider followings for delay-reporting PDCP data volume calculation associated with first reporting threshold:

-	the PDCP Control PDUs;

-	for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13;

-	for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

Proposal 10. The RLC layer should consider followings for delay-reporting RLC data volume calculation associated with first reporting threshold:

- RLC data PDUs that are pending for retransmission (RLC AM).

- Estimated size of STATUS PDU.

Proposal 11. When the UE needs to report buffer status of at least one LCG configured with reporting thresholds, the UE always transmits Rel-19 DSR MAC CE.

4.	Reference

[1] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[2] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

[3] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024