3GPP TSG-RAN2 Meeting #129	R2- 2501244

# Athens, Greece, Feb 17th – 21st, 2025

Agenda item:		8.11.2 (NR_duplex_evo)

Source:	LG Electronics Inc.

Title: 	Discussion on Random Access procedure in SBFD

Document for:	Discussion and Decision

1.	Introduction

According to WID of Rel-19 SBFD[1], it is agreed to support the Random Access procedure using the SBFD symbols.

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier: - … - Specify SBFD operation to support random access in SBFD symbols by UEs in RRC_CONNECTED mode and RRC_IDLE/INACTIVE mode [RAN1, RAN2] - …

This document further discusses RAN2 aspects on RA procedure for SBFD.

2.	Discussion

2.1 Signaling aspects of SBFD RACH configuration

In previous RAN1 meetings [2-3], it is agreed to support Option 1 with Alt 1-1 and Option 2 for the configuration of RA resource in SBFD symbol:

[RAN1#117 agreement] Agreement Confirm the following working assumption. Working Assumption For SBFD-aware UEs in RRC CONNECTED state, both RACH configuration Option 1 with Alt 1-1 (i.e., use one single RACH configuration, and only based on the existing parameters of the single RACH configuration) and RACH configuration Option 2 (i.e., Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration) are supported. Enabling both options at the same time for a UE is not supported. - For Option 1 with Alt 1-1, FFS whether/how to reinterpret msg1-FrequencyStart in rach-ConfigCommon, RO validation rules and SSB-RO mapping rules, etc. - For Option 2, FFS the RO validation rules, SSB-RO mapping rules, whether all the parameters currently in rach-ConfigCommon are necessary to be included in the additional RACH configuration, etc. UE is not required to support both options. [RAN1#119 agreement] Agreement For RACH configuration Option 1, for support of separate power control parameters for PRACH transmission in additional-ROs and legacy-ROs, support separate preambleReceivedTargetPower for additional-ROs. - FFS: powerRampingStep, preamble maximum output power PCMAX, preambleTransMax, powerRampingStepHighPriority

Based on the RAN1 agreements on RACH configuration Options of SBFD RO, it is agreed in RAN2#127bis meeting [4] to support the RACH configuration Option 1 and Option 2 and to further discuss on the details of RRC signaling.

The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

For RACH configuration Option 1, in RAN1#119 Meeting [3], it is agreed to indicate whether the SBFD RA procedure is available or not using an explicit indication.

Agreement Introduce explicit indications (1-bit SIB1 indication and/or dedicated signalling to convey cell-specific configuration) to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side.

On the other hand, according to the current RAN2 signaling structure for RA resource configuration, there are multiple RACH configurations based on triggering feature

- RACH configuration included in directly within BWP configuration (i.e., RACH-ConfigCommon IE and MsgA-ConfigCommmon IE in BWP-UplinkCommon), which includes RA resource not associated with any feature (blue-colored)

- RACH configurations included in additionalRACH-ConfigList, for separated RACH configurations for feature or feature combinations (purple-colored)

Therefore, it should be discussed whether the explicit indication (i.e., whether the SBFD RACH configuration Option 1 is available) is indicated per RACH configuration or per cell.

In our understanding, additionalRACH-ConfigList is defined in order to configure the separated RACH occasion for feature/feature combination (e.g., SDT/RedCap/…). Therefore, it would cause network restriction to configure the additional RACH occasion for feature or feature combination, if the SBFD RACH configuration Option 1 is available for all ROs configured in the network. In addition, for some feature/feature combinations, SBFD RO may not be needed, i.e., no need to allow SBFD RACH configuration Option 1 for some additional RACH configuration associated with features. In this sense, it would be better to allow the network flexibility to configure Random Access resources, using the explicit indication configured for each RACH configuration.

Proposal 1. The explicit indication to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not is indicated per RACH configuration (e.g., in RACH-ConfigCommon IE).

On signaling details of RACH configuration Option 1, only some extension of existing RACH configuration is needed, since SBFD-aware UE determines SBFD RO based on the additional rule based on legacy RACH configuration. The details of additional parameters (e.g., separate power control parameters, additional conditions to select SBFD RO for initial RA attempt, number of RA attempt for fallback procedure, etc.) can be determined later based on further RAN1/RAN2 discussion.

For RACH configuration Option 2, as agreed in RAN2#127bis meeting, additional RACH configuration for SBFD RO (i.e., ‘SBFD RACH configuration’) should be defined in order to configure SBFD RO and corresponding RA parameters. According to the current RA procedure, following methods were defined in order to configure separated ROs for various purpose:

- Method 1: define a new IE for the separated RACH configuration, e.g., RACH-ConfigCommonIAB defined for IAB

- Method 2: define a feature for RACH partitioning, and use RACH configuration within AdditionalRACH-Config, e.g., RO partitioning configured for SDT, Slicing, Msg1 repetition, etc.

Similarly, one of the above methods can be reused to define SBFD RACH configuration.

Meanwhile, in RAN2#127bis meeting [4], it is agreed that SBFD is not defined as a new feature in RACH partitioning framework for preamble partitioning, since SBFD early indication using preamble partitioning is not needed.

From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Based on the above agreement, it would be better to not define as a new feature for RO partitioning as well. In this sense, defining SBFD RACH configuration using Method 1 (i.e., new IE for SBFD RACH configuration) is slightly preferred.

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, define the new IE of SBFD RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

2.2 SBFD RA procedure

RA resource selection criteria between SBFD RO and legacy RO

For RA resource selection for initial PRACH transmission, it is agreed to select one RO between SBFD RO and legacy RO based on certain prioritization/conditions, for both RACH configuration Option 1 and Option 2 [4]:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

In RAN2#128 meeting [5], some details of selection criteria were discussed, including whether 1) to select the RO type (between SBFD RO and legacy RO) based on network indication first and apply other condition or 2) to select the RO type based on RSRP condition first and apply other condition if RSRP condition is not configured.

- FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

In our understanding, if the network indication is agreed, it should be applied prior to check other conditions (e.g., RSRP), since the intention of the network indication is to indicate the RO type before checking the other condition (e.g., for load balancing). For example, if the SBFD RO can be selected in good channel condition considering CLI impacts, even though the RSRP of the pathloss reference is better than the configured RSRP threshold (i.e., UE is in good channel condition), the network may indicate the UE to perform the RA procedure using the legacy RO, in order to mitigate the RA resource congestion of SBFD RO or to avoid the CLI impact temporarily. Therefore, if the network indication is agreed, it is not reasonable to apply the network indication depending on other conditions.

Proposal 3. If the network indication is agreed and configured, the SBFD-aware UE should select between SBFD RO and legacy RO based on the network indication, regardless of other conditions.

In our view, unless the network indication for RO type selection is agreed, for SBFD-aware UE, SBFD RO should be prioritized over legacy RO, since the legacy UE cannot use the SBFD RO. However, if the SBFD-aware UE perform RA procedure using SBFD RO in bad channel condition, CLI impact would occur as followings:

- When the UE in bad channel condition performs RA procedure using SBFD RO, the transmission of RA preamble may not be successful due to CLI impact caused by DL subband or DL slot on the network side.

- When the UE performs RA procedure using SBFD RO even though the UE is in cell edge, the UE would perform the power ramping procedure for re-attempt of the RA procedure, causing additional CLI impact to other UEs (e.g., to UEs in neighboring cell).

Therefore, in order to minimize the aforementioned CLI impact of using SBFD RO, it is suggested to use SBFD RO only if the channel is good enough to perform the RA procedure using SBFD RO as a baseline. In other words, if the SBFD RACH configuration is configured for SBFD-aware UE,

- SBFD RO is used when the channel condition is better than the threshold (i.e., when the RSRP is larger than the RSRP threshold)

- Otherwise, legacy RO is used for the current RA procedure.

Proposal 4. For the initial attempt of the RA procedure, if the network indication is not configured/agreed, SBFD-aware UE prioritizes SBFD RACH occasion if the channel condition is better than the RSRP threshold as a baseline. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

On the other hand, in RAN2#128 meeting, some companies suggested to use SBFD RO in bad channel condition (e.g., for RACH configuration Option 2), since the RA procedure using SBFD RO can be beneficial for coverage enhancement, e.g., when the long preamble format can be used for coverage enhancement using SBFD RO. However, even though the long preamble format can be used for SBFD RO in some cases, there are still aforementioned CLI impacts to perform uplink transmission using SBFD RO. In this sense, it is hard to estimate whether the RA preamble transmission using long format in SBFD RO is beneficial than RA preamble transmission using another format in legacy RO, in coverage aspects. Therefore, it should be confirmed by RAN1 whether it is really beneficial to perform RA procedure using SBFD RO when the SBFD-aware UE is in bad channel condition.

Proposal 5. It is up to RAN1 on whether SBFD RO can be used in bad channel conditions, e.g., in cell edge areas.

RA resource selection procedure for SBFD RA procedure

According to current MAC procedure, the Random Access resource selection is as follows:

- Step 1) Selection of UL carrier between NUL/SUL: If SUL is configured in addition to NUL, the UL carrier selection is done using downlink RSRP.

- Step 2) Selection of a set of Random Access resources (i.e., RACH partition) associated with feature

  - Determine feature applicability (e.g., Msg1 repetition + repetition number)

  - Determine availability for each set of RA resources

  - Set of RA resource selection

- Step 3) Selection of RA type between 4-step RA and 2-step RA

- Step 4) Selection of SSB

- …

On the other hand, as mentioned above, it is agreed to select between SBFD RO (i.e., additional RO) and legacy RO based on certain criteria in RAN2#127bis meeting. During the discussion, it was agreed to apply the following agreement for both RACH configuration Option 1 and RACH configuration Option 2, in order to have a common procedure for both RACH configuration Options:

Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network.

With this understanding in mind, it should be discussed in which step the SBFD-aware UE selects between SBFD RO and legacy RO. In our understanding, if SBFD RO is selected for RACH configuration Option 2, the RA parameter should be initialized using the separated RA configuration, which should be performed before the selection of RA type between 2-step RA and 4-step RA. Then, there are three options for determining the order of SBFD RO selection within the current RA procedure:

- Alt 1: Before the selection of a set of Random Access resources (i.e., between Step 1 and Step 2)

- Alt 2: Within the selection of a set of Random Access resources (i.e., in Step 2), i.e., selecting RO type (between SBFD RO and legacy RO) and RACH partition together

- Alt 3: After the selection of a set of Random Access resources and before the RA type selection (i.e., between Step 2 and Step 3)

Meanwhile, in RAN1#119 meeting [3], it is agreed to configure the separated RSRP threshold to determine Msg1 repetition number for SBFD RO, for RACH configuration Option 1 and RACH configuration Option 2.

Agreement For RACH configuration Option 2, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 and msg1-RepetitionNum for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - msg1-RepetitionNum is configured in RACH-ConfigCommon in current specification, i.e., for RACH configuration Option 2, msg1-RepetitionNum can already be separately configured for legacy-ROs and additional-ROs. Agreement For RACH configuration Option 1, support separate configuration of rsrp-ThresholdMsg1-RepetitionNum2/4/8 for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs. - Same msg1-RepetitionNum is used for PRACH transmission with preamble repetitions within additional-ROs and PRACH transmission with preamble repetitions within legacy-ROs.

In other words, whether to perform Msg1 repetition for this Random Access procedure and corresponding Msg1 Repetition Number should be determined based on different condition for SBFD RO and legacy RO.

Given that whether to perform the Msg1 repetition for this Random Access procedure and the corresponding Msg1 repetition number is determined during the RACH partition selection (i.e., Step 2), RO type should be selected prior to the RACH partition selection (i.e., Alt 1) or within the RACH partition selection (i.e., Alt 2), in order to apply the appropriate threshold to determine Msg1 repetition number. In this sense, selecting RO type after the RACH partition selection (i.e., Alt 3) is not reasonable.

For Alt 2, in order to determine whether to perform Msg1 repetition and corresponding Msg 1 repetition number should be determined separately for SBFD RO and legacy RO, which would cause too complicated RA procedure. Specifically, when the feature applicability is determined (i.e., Step 2A), the feature applicability of Msg1 repetition and applicable Msg1 repetition number should be separately determined for SBFD RO and legacy RO. Then, when the UE determines availability of each RACH partition (i.e., Step 2B), the availability of RACH partition associated with Msg1 repetition should be determined whether the RACH partition is configured for SBFD RO, legacy RO, or both, based on the feature applicability for SBFD and the feature applicability for legacy RO. In details

- For feature applicability determination (i.e., Step 2A) for Msg1 repetition:

  - Whether the Msg1 repetition is applicable and the applicable Msg1 repetition number for non-SBFD is determined using existing RSRP threshold (i.e., rsrp-ThresholdMsg1-RepetitionNum2/4/8)

  - Whether the Msg1 repetition is applicable and the applicable Msg1 repetition number for SBFD is determined using additional RSRP threshold, (i.e., rsrp-ThresholdMsg1-RepetitionNum2/4/8)

- For availability of RACH partition determination (i.e., Step 2B), if the RACH partition is associated with Msg1 repetition:

  - If the Msg1 repetition partition is configured only for legacy RO, the availability of the RACH partition should be determined based on whether Msg1 repetition is applicable and the applicable Msg1 repetition number for non-SBFD

  - If the Msg1 repetition partition is configured only for SBFD RO, the availability of the RACH partition should be determined based on whether Msg1 repetition is applicable and the applicable Msg1 repetition number for SBFD

  - If the Msg1 repetition partition is configured both for legacy RO and SBFD RO, the availability of the RACH partition should be determined based on whether Msg1 repetition is applicable and the applicable Msg1 repetition number for SBFD or non-SBFD

In addition, it could be discussed whether the RSRP threshold to determine Msg3 repetition (i.e., rsrp-ThresholdMsg3) can be configured separately for SBFD RO, which may cause additional complexity for Alt 2. Therefore, selecting RO type and RACH partition together (i.e., Alt 2) is not preferred.

Therefore, for simplified procedure to determine whether to perform Msg1 repetition and corresponding Msg1 repetition number based on the selected RO type, the RO type selection should be performed prior to RACH partition selection (i.e., Alt 1).

Proposal 6. The SBFD-aware UE selects between SBFD RO and legacy RO before selecting a set of Random Access resources.

Meanwhile, according to the current signaling structure, the RACH partition associated with feature can be differently configured in SBFD RO and legacy RO. Specifically,

- For RACH configuration Option 1, it is possible that SBFD RO may not be configured in some RACH configurations, e.g., in RACH-ConfigCommon included in AdditionalRACH-Config IE.

- For RACH configuration Option 2, due to separated RACH configuration, the preamble partitioning can be different from the legacy RACH configuration.

In other words, it is possible that the RACH partition associated with a feature is configured in legacy RO, while RACH partition associated with the feature is not configured in SBFD RO.

However, based on proposal 6, the selection between the SBFD RO and legacy RO is performed prior to the selection of RACH partition, in order to apply the appropriate RSRP threshold to determine Msg1 repetition number. In other words, the selection of RACH partition is performed within the selected RO type.

In our understanding, RA procedure using the SBFD symbol is to perform the Random Access procedure with better quality, e.g., for latency reduction, collision handling, and coverage enhancement. Therefore, if the follow-up procedure is different for specific feature, it may not be reasonable to always select SBFD RO based on the condition (e.g., RSRP), prior to select the feature/feature combination.

For example, the network may configure RACH partition for RA-SDT as follows, due to RA resource limitation in SBFD RO:

- In legacy RO,

  - RACH partition 1: common RACH partition (i.e., RACH partition not associated with any feature)

  - RACH partition 2: RA-SDT (i.e., RA resource to perform RA-SDT)

- In SBFD RO,

  - RACH partition 3: common RACH partition

Then, if the UE selects the RO type between the SBFD RO and legacy RO prior to the RACH partition selection, it is possible that SBFD-aware UE selects common RACH partition in SBFD RO (i.e., RACH partition 3), even though the SBFD-aware UE may selects the RACH partition for RA-SDT in legacy RO (i.e., RACH partition 2) as well. Then, the SBFD-aware UE would unnecessarily perform legacy RRC resume procedure (i.e., transition to RRC_CONNECTED state) even though the SBFD-aware UE is in good channel condition to perform RA-SDT procedure, causing additional delay to transmit SDT data.

Therefore, it should be discussed whether the RA procedure associated with a specific feature using legacy RO can be prioritized over the legacy RA procedure using the SBFD RO.

Proposal 7. Discuss whether the set of Random Access resource associated with a specific feature within legacy RO can be prioritized over the set of Random Access resource not associated with any feature within SBFD RO, even though the SBFD-aware UE meets the condition to select SBFD RO.

Fallback procedure from one type of RO to another type of RO

For re-attempt in one random access procedure, it is agreed to support the fallback procedure from SBFD RO to legacy RO, after the certain number of RA attempt using SBFD RO:

For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

Meanwhile, when the fallback procedure is defined for Msg1 repetition using the RACH partitioning framework, the fallback procedure is allowed only for increasing the Msg1 repetition number (e.g., from Msg1 repetition number 2 to Msg1 repetition number 4) using the same feature combination, in order to minimize additional complexity of RA procedure with RACH partitioning framework.

Similarly, the fallback procedure for SBFD RA procedure should be defined as simple as possible. Therefore, in order to avoid the re-selection of RACH partition after the fallback from SBFD RO to legacy RO, it is suggested to allow fallback from SBFD RO to legacy RO only if the same RACH partition (i.e., same feature combination and Msg1 repetition number) is configured in the legacy RACH configuration.

Proposal 8. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch from SBFD RACH occasion to legacy RACH occasions within the same feature combination and Msg1 repetition number.

In last RAN2 meeting, it was remained as FFS on whether the SBFD-aware UE is allowed to switch from legacy RO to SBFD RO. However, the fallback procedure from legacy RO to SBFD RO does not ensure successful transmission of RA preamble, since SBFD RO has additional CLI impact and may cause additional RA attempt failure due to signalling quality of RA preamble. That is, the benefits of fallback procedure from legacy RO to SBFD RO is marginal, compared to the fallback procedure from SBFD RO to legacy RO. Note that current RA procedure allows fallback procedure for only one direction (e.g., from 2-step RA to 4-step RA and from low Msg1 repetition number to high Msg1 repetition number), and it is allowed to ensure the robustness of Msg1 transmission.

Rather, if the fallback procedure from legacy RO to SBFD RO is additionally supported, lots of follow-up discussion may be needed including followings:

- Whether the additional condition to switch from legacy RO to SBFD RO is needed (e.g., RSRP Threshold),

- Whether/how to change the power related variables after performing the fallback procedure, and

- Whether the sequential fallback procedure is allowed (e.g., legacy RO  SBFD RO  legacy RO)

Therefore, if the fallback procedure is supported between SBFD RO and legacy RO for both directions, it would be very complicated to determine RO and performing RA parameter re-initialization, without any critical benefits. In this sense, for simplicity, once the SBFD-aware UE selects the legacy RO, the legacy RO should be used for the rest of the random access procedure, i.e., not supporting fallback from legacy RO to SBFD RO.

Proposal 9. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

2.3 Early indication for SBFD aware UE

During the discussion in RAN2#127bis [4], early indication for SBFD-aware UE using the preamble partitioning is not supported, while early indication using SBFD RO is supported without any spec impact:

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework.

Then, if the Msg3-based early indication is not supported, the network cannot identify SBFD aware UE in RRC_IDLE/INACTIVE state for following cases:

- If the SBFD-aware UE performs RA procedure using legacy RO, e.g., based on fallback procedure or additional condition to select SBFD ROs,

- If the SBFD-aware UE does not support RACH configuration option provided by network, or

- If the network does not configure SBFD RO even though the network supports SBFD symbols in the cell.

Then, even though the UE is SBFD-aware UE, the network cannot allocate the uplink grant using the UL subband within SBFD symbols, until the UE transfers its UE capability information with SBFD indication.

Therefore, Msg3-based early indication is slightly preferred, since the UE and the network need to perform message exchanges (e.g., including NAS messages for initiation setup, security activation, and RRC reconfiguration) before UE transfers UE capability information, and it would be better to transmit these initial messages using SBFD symbols for latency reduction. In our understanding, Msg3-based early indication can be easily implemented, as the only works for Msg3-based early indication is to define additional LCIDs for SBFD-aware UE.

Proposal 10. Support Msg3-based early indication for SBFD-aware UE.

3.	Conclusion

In this contribution, it is discussed about RAN2 aspects on Random Access operation for SBFD. This document includes following proposals.

Proposal 1. The explicit indication to indicate whether RACH configuration Option 1 for SBFD random access operation is enabled or not is indicated per RACH configuration (e.g., in RACH-ConfigCommon IE)

Proposal 2. For the details of configuring the SBFD RACH configuration for RACH configuration Option 2, define the new IE of SBFD RACH configuration, e.g., RACH-ConfigCommonSBFD IE.

Proposal 3. If the network indication is agreed and configured, the SBFD-aware UE should select between SBFD RO and legacy RO based on the network indication, regardless of other conditions.

Proposal 4. For the initial attempt of the RA procedure, if the network indication is not configured/agreed, SBFD-aware UE prioritizes SBFD RACH occasion if the channel condition is better than the RSRP threshold as a baseline. Otherwise, SBFD-aware UE performs RA procedure using legacy RACH occasion.

Proposal 5. It is up to RAN1 on whether SBFD RO can be used in bad channel conditions, e.g., in cell edge areas.

Proposal 6. The SBFD-aware UE selects between SBFD RO and legacy RO before selecting a set of Random Access resources.

Proposal 7. Discuss whether the set of Random Access resource associated with a specific feature within legacy RO can be prioritized over the set of Random Access resource not associated with any feature within SBFD RO, even though the SBFD-aware UE meets the condition to select SBFD RO.

Proposal 8. For PRACH transmission re-attempt in one RA procedure, UE is allowed to switch from SBFD RACH occasion to legacy RACH occasions within the same feature combination and Msg1 repetition number.

Proposal 9. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE.

Proposal 10. Support Msg3-based early indication for SBFD-aware UE.

4.	Reference

[1] RP‑241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

[2] Report of 3GPP TSG RAN WG1 #117

[3] Report of 3GPP TSG RAN WG1 #119

[4] Draft Report of 3GPP TSG RAN WG2 meeting #127bis, Hefei, China, 14 - 18 October, 2024

[5] Report of 3GPP TSG RAN WG2 meeting #128, Orlando, USA, 18 - 22 November, 2024