3GPP TSG-RAN WG2 Meeting #129	R2-2501249

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.2.2

Source:		Futurewei

Title:		Discussions on A-IoT Paging

Document for:		Discussion and Decision

# Introduction

In the WID [1] of Rel-19 Ambient IoT (A-IoT) WI, the RAN2-specific scope with respect to A-IoT paging is described as the following:

    - AIoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be     contained in one paging message, for forward compatibility purposes.

In this contribution, we discuss paging message content and format, including subsequent paging for the same service, paging ID details, etc.

# Discussions

Generally speaking, an A-IoT paging message may include the following information:

- Information for identifying the intended device(s)

- Information for preventing duplicated D2R responses

- Information for indicating D2R resource(s) to be used by the responding device(s)

- Information for triggering device(s) to re-access

In the following sub-sections, we discuss each type of information in details, including the content and format.

## Information for identifying the intended device(s)

Information identifying the intended device(s) should be provided by the AIOTF to the reader in the A-IoT Service Request, which information may indicate one Device ID, a group ID, or no Device ID at all. According to the latest TR 23.700-13 [2], operator allocated ID and third-party allocated ID, as shown in Figures 1 and 2, respectively, are supported by AIOTF as the permanent Device ID of an A-IoT device, while whether a temporary ID in the AIoT NAS layer is required for the privacy protection or not is FFS and is pending SA WG3 decision.

Figure 1 (same as Figure 8.2.1-1: The example of Operator allocated ID in [2])

Figure 2 (same as Figure 8.2.1-2: The example third party allocated ID in [2])

Our understanding is that a group ID can be generated as a common mask of the permanent Device IDs of the devices belonging to the same group, unless SA WG(s) decide on a separate procedure for assigning group ID to devices by the A-IoT CN.

Observation 1. In Rel-19, information identifying the intended device(s) may indicate one Device ID, a group ID, or no Device ID at all.

Observation 2. A group ID can be generated as a common mask of the permanent Device IDs of the devices belonging to the same group, unless SA WG(s) decide on a separate procedure for assigning group ID to devices by the A-IoT CN.

Based on the observations above, we propose to use an A-IoT MAC CE, e.g., referred to as the Device ID MAC CE, for carrying the information that identifies the intended devices in the A-IoT paging message. For example, a Device ID MAC CE includes a D/C (data/control) bit, a Type field, a Length field, and optionally an ID field, as shown in Figure 3 below. The D/C field is set to “1” to indicate that the MAC element is an A-IoT MAC CE. (In our companion paper [3] submitted to the same meeting, we also propose to introduce A-IoT MAC data element (DE) for carrying A-IoT MAC SDU (A-IoT NAS PDU) and the D/C bit in an A-IoT MAC DE is set to “0”.)  The Type field is set to a standardized value corresponding to Device ID MAC CE. The Length field indicates the length of the ID field when the Length field contains a non-zero value and indicates that the ID field is not present when the Length field is set to value zero. In our companion paper [3], we also propose an alternative general format for MAC CE, where the Type field and the Length field switch places and the Length field indicates the total length of all remaining field(s) to provide forward compatibility.

Figure 3. Example Device ID MAC CE Format

When an A-IoT Service Request is intended for only one device, the A-IoT paging message includes one Device ID MAC CE and the ID field in the Device ID MAC CE includes the full Device ID (as defined in [2]) of the intended device. When an A-IoT Service Request is intended for a group of devices that share a common mask among their respective Device IDs, the A-IoT paging message can include one Device ID MAC CE and the ID field in the Device ID MAC CE contains the common mask shared by the group of devices. When an A-IoT Service Request is intended for any device within the coverage, the A-IoT paging message includes one Device ID MAC CE that does not include an ID field.

Due to the lack of an agreement on the need for a temporary NAS ID in SA2 and due to the length of a full permanent Device ID, as defined in [3], we suggest that a Device ID MAC CE can include only up to one full Device ID and an A-IoT paging message can include only one such Device ID MAC CE. We also suggest that a Device ID MAC CE can include only up to one Device ID mask but FFS whether an A-IoT paging message can include multiple such Device ID MAC CEs.

Proposal 1. Information identifying the intended devices in an A-IoT paging message is indicated using A-IoT MAC CE, e.g., Device ID MAC CE, in the A-IoT paging message. The Device ID MAC CE includes no Device ID when the A-IoT paging message is intended for any device within the coverage; otherwise, the Device ID MAC CE includes an ID field, which contains the full Device ID of the intended device when the A-IoT paging message is intended for only one device or contains the Device ID mask shared among a group of devices when the A-IoT paging message is intended for the group of devices.

Proposal 2. A Device ID MAC CE can include only up to one full Device ID and an A-IoT paging message can include only one such Device ID MAC CE. A Device ID MAC CE can include only up to one Device ID mask, FFS whether an A-IoT paging message can include multiple such Device ID MAC CEs.

## Information for preventing duplicated D2R responses

Due to clock drifting at the devices, only a small number of devices can be triggered to respond at a time. Therefore, to complete an inventory operation for hundreds or thousands of devices, in addition to the initial paging message, subsequent paging (or similar R2D trigger) messages are usually needed to trigger multiple rounds of paging cycles sequentially for a same A-IoT Service Request so that only a sub-set of the eligible devices are triggered to respond in each paging cycle. In addition, subsequent paging (or similar R2D trigger) messages may also be needed to allow re-access by devices whose pervious response for the same A-IoT Service Request was not received successfully. Finally, subsequent paging (or similar R2D trigger) message may also be needed as a catch-all step to allow devices that have missed all prior paging cycles for the same A-IoT Service Request, e.g., due to a lack of energy, to respond when they become sufficiently charged.

During Rel-19 SI phase, RAN2 made various agreements with respect to preventing duplicated D2R responses from the device, which are captured in [4] as the following:

It is supported that the reader can send multiple (subsequent) A-IoT paging messages that are associated with the same service request from the CN. The duplicated response from devices for the same service request should be avoided. The A-IoT paging message can include information to avoid this duplicated response from the device to a reader. It needs to be further discussed on how to design this information in A-IoT paging message (e.g., including stage-3 details and considering the related aspects from other WGs). Then, based on this information, the device determines whether to skip sending the response to A-IoT paging message or not (if the device had successfully responded the same service before). This information should be short and simple. This information is one ID, while it needs to be further discussed whether the ID is generated by the reader or by the core network. It needs to be further discussed on the size of this information.

It needs to be further discussed for the scenario that different readers may send A-IoT paging messages, which are associated with the same service request from the CN, to the same device for response. If this scenario is in the scope, it needs to be further discussed, by considering the progress from all the WGs.

According to the latest TR 23.700-13 [2], a correlation ID is used between the A-IoT RAN and AIOTF to indicate the association between an A-IoT Service Request and an A-IoT Service Response. However, it is unclear whether this NAS layer information will be sufficient for all scenarios, e.g., to differentiate the case where the same service request received from different readers needs to be responded by the device individually to each reader (e.g., for positioning purpose) vs. the case where the same service request received from different readers needs to be responded by the device only once to any reader. Therefore, we propose that a separate paging ID, which is an AS information, can be included in the A-IoT paging message. Whether the paging ID is same as the correlation ID or a combination of the correlation ID and the reader ID, or is generated by the reader independently from the correlation ID based on the deployment scenario can be FFS. Overall, the size and the content of this paging ID may be under the control of the reader.

Paging ID may not always be required for every A-IoT paging message. For example, if an A-IoT paging message is used for paging only one device and the Msg3 or a subsequent D2R transmission from the device was not received successfully by the reader, the reader can send the same A-IoT paging message again to trigger the device to re-access without a need for sending an explicit NACK to the device first. Therefore, we propose that the paging ID is optionally present in an A-IoT paging message, and if present, is carried in an A-IoT MAC CE, e.g., referred to as the Paging ID MAC CE. If no Paging ID MAC CE is included in an A-IoT paging message, a device shall respond to the A-IoT paging message if it meets all other criteria, e.g., matching the Device ID, having sufficient energy, etc.

The Paging ID MAC CE, if present, includes the D/C (data/control) bit, the Type field, a Length field, and a Paging ID field, as shown in Figure 4 below. The D/C field is set to “1”, as described before. The Type field is set to a standardized value corresponding to Paging ID MAC CE. The Length field indicates the length of the Paging ID field to give the reader the flexibility to choose the size of the paging IDs to be used.  In our companion paper [3], we also propose an alternative general format for MAC CE, where the Type field and the Length field switch places and the Length field indicates the total length of all remaining field(s) to provide forward compatibility.

Figure 4. Example Paging ID MAC CE Format

Proposal 3. A paging ID, which is an AS information, can be included in the A-IoT paging message. FFS whether the paging ID can be same as the correlation ID or a combination of the correlation ID and the reader ID, or is generated by the reader independently from the correlation ID.

Proposal 4. The paging ID is optionally present in an A-IoT paging message, and if present, is carried in an A-IoT MAC CE, e.g., referred to as the Paging ID MAC CE. A device does not send duplicated response to an A-IoT paging message if it has already responded successfully to another A-IoT paging message with the same Paging ID before.

Proposal 5. If no Paging ID MAC CE is included in an A-IoT paging message, a device shall respond to the A-IoT paging message if it meets all other criteria, e.g., matching the Device ID, having sufficient energy.

## Information for indicating D2R resource(s) to be used by the responding device(s)

Because the scheduled D2R resource information may be needed not only in the A-IoT paging message but also possibly in  other R2D messages such as Msg2 (when 16-bit random IDs of multiple devices being echoed at the same time), we propose that the scheduled D2R resource information is also carried in an A-IoT MAC CE so that the same MAC CE design may be used in different R2D messages. However, because the detailed D2R resource information depends on the final design in RAN1, we suggest that discussion on the detailed content in this MAC CE be deferred.

Proposal 6. The scheduled D2R resource information in an A-IoT paging message is carried in an A-IoT MAC CE. FFS the detailed content in this MAC CE.

## Information for triggering device(s) to re-access

RAN2 has decided, during Rel-19 SI phase, that (ACK/NACK-based) RLC-like retransmission is not supported for A-IoT. Meanwhile, after failing to receive an expected D2R response from a recognized device, the reader is allowed to send another R2D trigger message including the same Command Request to cause the device to retransmit the Command Response, as devices always follow the command from the reader. However, if we do not mandate such behaviors (i.e., immediately re-sending the same Command Request after a failure) on all readers, and when there is no further R2D data transmission needed to the device, there is still ambiguity on the device side as to whether its previous D2R data transmission has been received successfully or not (and whether it needs to re-access or not) if the device does not receive anything back from the reader.

To address the ambiguity issue mentioned above, we propose that after all transmissions triggered by a current paging (or similar R2D trigger) message are complete, whether successfully or unsuccessfully, the reader sends a subsequent paging (or similar R2D trigger) message with the same paging ID. This subsequent paging message can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response to seeing their respective 16-bit random IDs being listed, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader. The subsequent paging (or similar R2D trigger) message may include information to trigger additional devices, which were unable to participate in the previous paging cycles of the same paging ID, to participate in the current paging cycle.

Proposal 7. A subsequent paging (or similar R2D trigger) message with a same paging ID can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID.

Proposal 8. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader.

# Conclusions

We propose the following:

Proposal 1. Information identifying the intended devices in an A-IoT paging message is indicated using A-IoT MAC CE, e.g., Device ID MAC CE, in the A-IoT paging message. The Device ID MAC CE includes no Device ID when the A-IoT paging message is intended for any device within the coverage; otherwise, the Device ID MAC CE includes an ID field, which contains the full Device ID of the intended device when the A-IoT paging message is intended for only one device or contains the Device ID mask shared among a group of devices when the A-IoT paging message is intended for the group of devices.

Proposal 2. A Device ID MAC CE can include only up to one full Device ID and an A-IoT paging message can include only one such Device ID MAC CE. A Device ID MAC CE can include only up to one Device ID mask, FFS whether an A-IoT paging message can include multiple such Device ID MAC CEs.

Proposal 3. A paging ID, which is an AS information, can be included in the A-IoT paging message. FFS whether the paging ID can be same as the correlation ID or a combination of the correlation ID and the reader ID, or is generated by the reader independently from the correlation ID.

Proposal 4. The paging ID is optionally present in an A-IoT paging message, and if present, is carried in an A-IoT MAC CE, e.g., referred to as the Paging ID MAC CE. A device does not send duplicated response to an A-IoT paging message if it has already responded successfully to another A-IoT paging message with the same Paging ID before.

Proposal 5. If no Paging ID MAC CE is included in an A-IoT paging message, a device shall respond to the A-IoT paging message if it meets all other criteria, e.g., matching the Device ID, having sufficient energy.

Proposal 6. The scheduled D2R resource information in an A-IoT paging message is carried in an A-IoT MAC CE. FFS the detailed content in this MAC CE.

Proposal 7. A subsequent paging (or similar R2D trigger) message with a same paging ID can include the 16-bit random ID(s) of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging cycle, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging cycle or a later paging cycle triggered with the same paging ID.

Proposal 8. Devices, which had transmitted during the previous paging cycle and the 16-bit random ID of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same paging ID from the reader.

# References

RP-243326, A-IoT WID, RAN1 Vice-chair (Huawei).

TR 23.700-13 V1.3.0.

R2-2501250, Discussions on A-IoT data transmission and other aspects, Futurewei.

TR 38.769 V19.0.0.