# 3GPP TSG-WG2 Meeting #129                                            			R2-2501252

Athens, Greece, Feb. 17th – 21st, 2025

Source:                      Qualcomm Incorporated

Title:                          LP-WUS operation in IDLE and Inactive state

Agenda Item:	8.4.2

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

- For IDLE/INACTIVE modes

  - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4)

# Discussion

## 2.1 Inter-frequency bands support

The intention of supporting inter-frequency bands for LR and MR is UE could only support LP-WUS on specific band, and MR is operating on another frequency band, or reduce LR cell overloading. In RAN plenary 106, RP-242656 proposes some solutions as below, and agreed the way forward:

- For idle/inactive mode, consider a standardized solution for the intra-frequency case (e.g., solutions 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g., solution 4) will not be considered.

- Detailed discussions and final solution selection are left to the working groups (WGs).

In solution 1, MR is on the same band as LR for LP-WUS indication, paging monitor and RACH access, and introduce LP-WUS specific frequency priority in SIB. LP-WUS UE can be prioritized to reselect to the frequency which supports LP-WUS. Since this frequency priority is configured in SIB, then all the LP-WUS UEs will follow the priority and reselect to the same cell. This will increase LP-WUS cell overload when all the LP-WUS UEs camps on the same frequency.

In solution 2, MR is on the same band as LR for LP-WUS indication, paging monitor, but RACH access is on another MR band. UE is distributed to the LP-WUS frequency band for paging monitoring. After UE receives paging on LR band, UE should reselect to another MR frequency for RACH access. This solution requires UE to reselect to another MR frequency after receiving paging message, and UE will perform cell reselection and acquire system information of another MR cell. This will introduce MT access latency. Another issue for solution 2 is that UE needs to camp on LR cell and MR cell simultaneously so that UE needs to maintain two sets of system information and it is unclear whether UE should perform two location update procedure respectively in both LR and MR when the UE moves outside of location area e.g. TA or RAN paging area.

In solution 3, gNB can use dedicated frequency priority to distribute the LP-WUS UE to another frequency according to UE LP-WUS capability. E.g. if the current LP-WUS cell is overload, then the gNB can redirect the UE to another LP-WUS frequency if the UE support LP-WUS on that frequency. It is assumed that UE LP-WUS capability on each frequency band will be reported to gNB in capability, then gNB can distribute different UEs to different LP-WUS band. This could be achieved by existing dedicated frequency priority, and no enhancement is needed.

Considering limited Rel-19 time, existing dedicated frequency priority can be reused to distribute UE to different frequency bands (LP-WUS or legacy frequency band) to resolve the LP-WUS cell overload.

- Reuse existing dedicated frequency priority to distribute UEs to other frequency bands to reducing LP-WUS cell overload.

## 2.1 LP-WUS mode control

There were some discussions in RAN2#127 meeting that whether dedicated LP-WUS configuration can be provided to the UE. It is understood that the LP-WUS configuration here means entry/exit condition, it is unclear what is the motivation for dedicated entry/exit condition to be introduced. And if the dedicated condition is introduced, the UE needs to maintain both the dedicated configuration and common configuration and RAN2 also needs to discuss in what condition the dedicated configuration is invalid.

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

In RAN2#127 meeting, there were proposals and discussion that network can activate or deactivate LP-WUS monitoring using dedicated RRC signalling. If the UE is deactivated LP-WUS monitoring, then the UE will never use LP-WUS to save power consumption as long as the UE stays in IDLE state in this cell. And the motivation is not clear. Then it is proposed not to introduce LP-WUS activation/deactivation in dedicated RRC message.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

Some companies proposed to indicate in SIB whether the UE should skip PEI monitoring when the UE is monitoring LP-WUS. The motivation is to replace PEI with LP-WUS totally, and the network does not need to send indication signalling in both LP-WUS and PEI. But the issue is, when the UE moves outside of LP-WUS coverage, the UE will not indicate to the network as the baseline RAN2 made in the last meeting and the network cannot enable PEI, so the UE cannot use PEI. Another issue is, LP-WUS and PEI can be used together to reduce more false paging. Also, RAN1 has discussed this issue, and the agreement is that it is left to UE implementation to monitor PEI or not.

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Then, it is proposed network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB, and do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

In RAN2#127 meeting, it has been agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

RAN1 also agreed to introduce OFDM overlaid LP-SS measurement, this is a kind of new measurement type, so it is worthy to introduce a separate entry/exit threshold for OFDM overlaid LP-SS measurement.

- Introduce a separate entry/exit threshold for OFDM overlaid LP-SS measurement.

From UE perspective, UE may have different capabilities to support SSB based measurement, OOK based LP-SS measurement, and OFDM overlaid LP-SS measurement. If the supported measurement type has not be configured entry/exit condition, it is up to UE implementation to determine the entry/exit condition.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

One issue is, if the entry/exit conditions for all of three measurement types are configured, which one of should be used by the UE. In this case, it should allow the UE to select the measurement metrics the UE can support.

- It is up to UE implementation to choose which one is used if all of three measurement types (SSB based measurement, OOK based LP-SS measurement, and OFDM overlaid LP-SS measurement) are configured are configured.

## 2.3 LP-WUS subgrouping

In last RAN2 meeting, RAN2 agreed that for UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping. In last RAN2 meeting, RAN2 discuss the following three options

FFS on the following options

- - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping.

- - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed.

- - Option 3: The formula for UE_ID based PEI subgrouping is reused.

After UE detects LP-WUS indication, RAN1 agreed that it is left to UE implementation to monitor PEI or not. If the UE is indicated to trigger MR for paging monitoring, UE may continue to monitor PEI and determine whether to further monitor paging message. one additional issue is whether any enhancement is needed to further distribute the UEs mapped to the same LP-WUS subgroup into different PEI subgroups. The joint subgrouping between LP-WUS and PEI improves the subgrouping granularity by a factor of the number of PEI subgroups with no additional signaling cost. Then the subgroup ID in LP-WUS can be independent with the one in PEI..

- The formula for UE_ID based PEI subgrouping is taken as baseline for LP-WUS UE_ID based PEI subgrouping.

- If enhancement is needed, then option 1 can be acceptable.

## 2.4 Emergency PDU session handling

In last RAN2 meeting, there is open issues on emergency PDU session as follows.

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

In RAN3#126 meeting, there was discussion on whether to disable LP-WUS during an emergency PDU Session, with no agreements made.

In SA2 #166AHE, 20-24 January 2025 meeting there were discussions on whether it is necessary to disable paging mechanisms like PEI, eDRX for a UE with an active emergency PDU session. SA2 did not conclude and is following up with CT1 on whether there are any use cases where a call back is expected during a MO emergency service to decide on disabling of paging mechanisms.

It is proposed to wait for SA2/CT1 progress on emergency PDU session handle to decide whether to disable LP-WUS during emergency PDU session.

- Wait for SA2/CT1 progress on whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

# Conclusion

- LP-WUS load reduction,

- Reuse existing dedicated frequency priority to distribute UEs to other frequency bands to reducing LP-WUS cell overload.

- LP-WUS mode control

- Do not support dedicated LP-WUS monitoring entry/exit condition configuration.

- Do not introduce LP-WUS activation/deactivation in dedicated RRC message.

- Network sends paging indication in both of LP-WUS and PEI if LP-WUS and PEI are supported by both of UE and gNB.

- Do not introduce indication in SIB to indicate whether UE should skip PEI monitoring.

- Introduce a separate entry/exit threshold for OFDM overlaid LP-SS measurement.

- It is up to UE implementation to determine the entry/exit condition if there is no entry/exit condition configured for the supported measurement type.

- It is up to UE implementation to choose which one is used if all of three measurement types (SSB based measurement, OOK based LP-SS measurement, and OFDM overlaid LP-SS measurement) are configured are configured.

- LP-WUS subgrouping

- The formula for UE_ID based PEI subgrouping is taken as baseline for LP-WUS UE_ID based PEI subgrouping.

- If enhancement is needed, then option 1 can be acceptable.

Emergency PDU session handling

- Wait for SA2/CT1 progress on whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)