3GPP TSG-RAN WG2 Meeting #129	R2-2501254

Athens, Greece, Feb. 17th – 21st, 2025

Source:                Qualcomm Incorporated

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Agenda Item:	8.4.3

Document for:	Discussion and Decision

# Introduction & Background

The following objective is approved for IDLE and Inactive mode LP-WUS, this contribution discusses the related issues.

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# Discussion

## 2.0 RAN4 supported cases

RAN4 agreed to support the following cases for measurement offloading to LP-WUS.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

In case #1, serving cell is fully offloaded to LR

In case #3, serving cell can be measured by MR and LR, but combined measurement results from both LR and MR are not supported. The scenario can be shown in the following Figure 1.

Figure 1

## 2.1 For case #1 (i.e. green area)

As RAN4 agreed scenarios, the measurement offloading mode transition can be shown in Figure 2. Following the offloading mode. Then the conditions among the offloading modes should be defined.

Figure 2

For fully offloading case, RAN2 made agreements that MR and LR measurement can be considered for serving cell measurement relaxation with offloading to LP-WUS as follows:

For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

    - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

    - The exit condition is based on the LR measurement results.

However, the UE behavior is still unclear on how to consider the MR/LR measurement when determining whether to offload to LP-WUS. UE can be configured a threshold to determine whether to offload measurement to LR. Considering MR and LR may have different coverage, it is more safe that UE starts offloading measurements to LR. That means when MR measurement is above a threshold and the LR measurement is above a threshold, UE starts offloading measurements to LR. There could be case that MR measurement is above the threshold but the LR measurement is below a threshold, it can be left to UE implementation to determine when to start LR measurement.

- The entry condition for fully offloading UE: When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

As shown in Figure 1, when the UE moves outside of green area, the UE should exist from fully offloading mode, enters partially offloading mode.

- The exit condition for fully offloading UE: When the LR measurement is below a threshold, UE exist from fully offloading mode.

Since in case #1, neighbouring cell measurement is also off, then the condition for offloading serving cell can also be used for neighbouring cell measurement.

- When the UE is in fully offloading mode, UE stops serving cell measurement and neighbouring cell measurement.

## 2.2 For case #3b (i.e. gray area in Figure1)

In case #3, serving cell can be measured by MR and LR, but combined measurement results from both LR and MR are not supported. Then it should be defined in which condition the UE will enter or exit from the partially offloading mode.

Similar with case #1, the MR and LR measurement can be the condition as follows

-	When both MR and LR measurement are above the thresholds defined for partially offloading and,

-	When any of LR and MR measurement is below the threshold which is defined for totally offloading

In last RAN2 meeting, RAN2 had the following agreements:

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

UE may have different capabilities on MR measurement, when the MR measurement is higher than relaxation threshold, some UE may not have reliable LR measurement. Then it is beneficial to have additional LR measurement threshold for the entry condition

When either of MR or LR is below a threshold, UE will leave partially offloading mode and enter no offloading mode.

- The entry condition for partially offloading:

-	When both MR and LR measurement are above the thresholds defined for partially offloading, and,

-	When any of LR and MR measurement is below the threshold which is defined for totally offloading

## 2.2 For no offloading case (i.e. white area in Figure1)

This condition is used for the case that UE switch to legacy MR measurement. As discussed for fully and partially offloading, when any of LR or MR measurement is below a threshold defined for no offloading, UE should leave offloading mode (fully or partially).

- The condition for leaving offloading mode (fully or partially):  when any of LR or MR measurement is below a threshold defined for no offloading, UE should leave offloading mode (fully or partially).

## 2.3 Neighboring measurement relaxation criterion

Currently, for intra-frequency measurement, in order to save UE power, a serving cell quality threshold (SIntraSearchP/SIntraSearchQ) are introduced for trigger intra-frequency measurement. When in partially offloading mode to LP-WUS, UE will measure using both of MR and LR, but the MR measurement is in a relaxed way. That means there are two measurement results. One issue is which measurement result will be used to trigger intra-frequency measurement. One way is using existing serving cell MR measurement, but serving cell measurement is in a relaxed way, the measurement accuracy could not be enough to evaluate the condition; another way is to introduce LP-WUS specific parameters.

Similarly, for inter-frequency measurement relaxation, the criteria to determine ‘not at cell edge’, whether MR measurement or LR measurement is used needs to be determined.

From RAN2 perspective, it should be assumed that existing MR serving cell measurement in a relaxed way criterion can be reused, then not to introduce LP-WUS specific criterion for neighboring  measurement relaxation. It is up to gNB implementation to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

It is proposed to ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurements and measurement relaxation.

- RAN2 assumes to reuse existing MR serving cell measurement criterion for neighboring measurement relaxation. It is up to gNB configuration to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

## 2.4 Cell reselection criteria

Currently, cell reselection criteria is based on MR measurement evaluation. If the serving cell measurement is totally offloaded to LR, then there is no MR serving cell measurement for cell reselection evaluation.  From RAN2 perspective, it is assumed that existing MR serving cell measurement in a relaxed way criterion can be reused for cell reselection evaluation, then no need to introduce LP-WUS specific criteria for neighboring  measurement relaxation. It is up to gNB implementation to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- RAN2 assumes to reuse existing MR serving cell measurement for cell reselection evaluation. It is up to gNB configuration to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- Ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurement relaxation and cell reselection.

# Conclusion

- The entry condition for fully offloading UE: When the MR measurement is above a threshold and the LR measurement is above a threshold, UE starts totally offloading measurement to LR (i.e, stop MR measurements). It is up to UE implementation to determine when to start LR measurement.

- The exit condition for fully offloading UE: When the LR measurement is below a threshold, UE exist from fully offloading mode.

- When the UE is in fully offloading mode, UE stop serving cell measurement and neighbouring cell measurement.

- The entry condition for partially offloading:

-	When both MR and LR measurement are above the thresholds defined for partially offloading, and,

-	When any of LR and MR measurement is below the threshold which is defined for totally offloading

- The condition for leaving offloading mode (fully or partially):  when any of LR or MR measurement is below a threshold defined for no offloading, UE should leave offloading mode (fully or partially).

- RAN2 assumes to reuse existing MR serving cell measurement criterion for neighboring measurement relaxation. It is up to gNB configuration to guarantee neighboring measurement relaxation should be triggered outside of LPWUS fully loading area.

- RAN2 assumes to reuse existing MR serving cell measurement for cell reselection evaluation. It is up to gNB configuration to guarantee cell reselection to be evaluated outside of LPWUS fully loading area.

- Ask RAN4 whether the relaxed MR serving cell measurements can be used to evaluate triggering neighboring cell measurement relaxation and cell reselection.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)