3GPP TSG-RAN WG2 Meeting #129	R2-2501258

Athens, Greece, Feb. 17th – 21st, 2025

Source:                Qualcomm Incorporated

Title:	SON and MDT for SDT

Agenda Item:	8.10.5.1

Document for:	Discussion and Decision

# Introduction & Background

Rel-19 SON/MDT agreed to continue Rel-18 leftovers as following, and this contribution discuss the related issues.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

•	RACH optimization for SDT

# Discussion

## 2.1 RACH optimization for SDT

In the last RAN2 meeting, RAN2 had some discussion on RACH optimization for SDT and achieved the following agreement.

Agreements

- The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report.

- The following failure causes can be logged for failed SDT in RA report:

⁻	upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

⁻	upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

⁻	upon T319a expires;

⁻	upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- FFS: unsuccessfully completed upon cell re-selection.

For the highlighted part, the timers of cg-SDT-TimeAlignmentTimer or the configuredGrantTimer is used in CG-SDT, but RAN2 only focus on RA-SDT in Rel-19. Then UE does not need to include in RA report when these two timers expire.

- Revert the agreement that UE should log the cause value for failed SDT in RA report.

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

For the following FFS part: FFS: unsuccessfully completed upon cell re-selection, this is related to UE mobility and is not related to SDT configuration or coverage, so it is unclear how the gNB to use the cause information. Then it is proposed not to report this failure cause in RA-SDT.

- UE does not report ‘unsuccessfully completed upon cell re-selection’ failure cause in RA-report.

# Conclusion

- Revert the agreement that UE should log the cause value for failed SDT in RA report.

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- UE does not report ‘unsuccessfully completed upon cell re-selection’ failure cause in RA-report.

# Reference