3GPP TSG-RAN WG2 Meeting #129	R2-2501259

Athens, Greece, Feb. 17th – 21st, 2025

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re)selection for multi-hop U2N relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discuss. This contribution discusses the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.3 Discovery

For Model B discovery response transmission, one issue is whether last Relay UE needs to check the PC5 link quality of the receiving link satisfies a PC5 threshold. There are two options to be considered.

Option 1: The last Relay UE checks the PC5 link quality of the receiving link and determines to send discovery response message. If the receiving link is above the PC5 threshold, then the last Relay UE sends discovery response message.

Option 2: The last Relay UE does not check the PC5 link quality of the receiving link as single-hop Relay UE. It is up to the intermediate Relay checks the PC5 link quality of the receiving link for discovery response message and determines whether to forward the discovery response message.

Option 2 will waste the discovery resource if the link between the intermediate Relay UE and the last Relay UE does not satisfy the PC5 threshold. It is better that the last Relay UE check PC5 link quality of the receiving link.

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

For the AS container included in discovery message, it is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

## 2.4 Relay (Re)selection

In existing U2N or U2U relay, the Remote UE (re)selects candidate Relay UE considering the hop link quality (SD-RSRP) between the Remote UE and the Relay UE, similar mechanism can be reused for Remote UE to (re)select the candidate Intermediate Relay UEs. And the Remote UE considers an Intermediate Relay UE for which SD-RSRP exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

For Intermediate Relay UE, it can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the next Relay UE (U2N Relay UE for two-hops relay or Intermediate Relay UE for three-hops relay). The Intermediate Relay UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate U2N Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers an Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

Similar with existing U2N relay, the Remote UE can trigger to reselect a candidate Intermediate Relay UE when PC5 link quality between the Remote UE and the current Intermediate Relay UE is below a threshold; or the current PC5 link between the Remote UE and the Intermediate Relay is released due to e.g. RLF, released by upper layer etc.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

When the intermediate relay UE detects PC5 RLF, how to handle the failure should be discussed. One possible way is the intermediate relay UE notifies the Remote UE via PC5 and the Remote UE reselects the relay path. In this way, any PC5 link failure will trigger the Remote UE to reselect a path, which will consume signalling overhead and increase service interruption time, and if more relay UEs are added into the multi-hop relay path, there could be more signaling overhead introduced. Another way is, the intermediate relay UE can trigger relay reselection and tries to reselect candidate path towards the last relay UE. This way enables local link recovery and does not trigger the whole link recovery from the Remote UE. It is proposed that the intermediate Relay UE can trigger to reselect the next Relay UE when PC5 link quality between the Intermediate Relay UE and the next Relay UE is below a threshold; or the current PC5 link between the Intermediate Relay UE and the next Relay is released due to e.g. RLF, released by upper layer etc.

- When the Intermediate Relay detects PC5 RLF, the Intermediate Relay UE does not need to notify the downstream UE (the Remote UE or Intermediate Relay UE)

- The Intermediate Relay UE can trigger to reselect a candidate last Relay UE when RLF is detected or the current link quality between the Intermediate Relay UE and upstream Relay UE is below a threshold.

When the intermediate Relay UE receives RRCReconfiguration message for HO, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

- For L2 relay, if intermediate Relay UE receives RRCReconfiguration message for HO, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

Similarly, if intermediate Relay UE switches to Uu coverage, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

- For L2 relay, if intermediate Relay UE switches to Uu coverage, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

- Conclusion

Discovery

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

Relay UE (re)selection

- From AS layer, Remote UE (re)selects candidate Intermediate Relay UEs considering the hop link quality (SD-RSRP) between the Remote UE and the Intermediate Relay UE.

- From AS layer, Remote UE considers an Intermediate Relay UE for which SD-RSRP between the Remote UE and the Intermediate Relay UE exceeds a threshold as candidate Intermediate Relay UE.

- From AS layer, Intermediate Relay UE can select the candidate upstream Relay UE considering the hop link quality between the Intermediate Relay UE and the U2N Relay UE.

- From AS layer, the Intermediate Relay UE considers a Relay UE for which SD-RSRP between the Intermediate Relay UE and the U2N Relay UE exceeds a threshold as candidate U2N Relay UE.

- The Remote UE can trigger to reselect a candidate Intermediate Relay UE in the following conditions:

-	PC5 link quality towards the current Intermediate Relay UE is below a threshold

-	The current PC5 link is released due to e.g. RLF, release by upper layer, etc.

- When the Intermediate Relay detects PC5 RLF, the Intermediate Relay UE does not need to notify the downstream UE (the Remote UE or Intermediate Relay UE)

- The Intermediate Relay UE can trigger to reselect a candidate last Relay UE when RLF is detected or the current link quality between the Intermediate Relay UE and upstream Relay UE is below a threshold.

- For L2 relay, if intermediate Relay UE receives RRCReconfiguration message for HO, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

- For L2 relay, if intermediate Relay UE switches to Uu coverage, the intermediate Relay UE should notify the Remote UE or the downstream relay UE.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet