3GPP TSG-RAN WG2 Meeting #129                                                      R2-2501260

Athens, Greece, Feb. 17th – 21st, 2025

Source:	Qualcomm Incorporated

Title:	Open issue for control plane approach 1

Agenda Item:	8.13.3

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay was approved as Rel-19 item, and this contributions discuss the contron plane issues.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

In last RAN2 meeting, RAN2 agreed to support a baseline procedure for the case in which the intermediate relay UE should enter CONNECTED state when the Remote UE goes to CONNECTED state. In this case, the intermediate Relay UE is also acting as a Remote UE, so the intermediate Relay UE cannot support different indirect paths to the gNB.

Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED.

Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell.

If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links.

Taking the following as example, if the intermediate Relay UE already established red relay path for Remote UE(s), then the green relay path should not be established. How to avoid green relay path is the issue to be addressed.

Figure 1

The intention to establish different PC5 connection is for different RSCs, there could be two options to avoid establish different PC5 connection:

Option 1: The intermediate Relay UE only broadcasts one RSC in discovery message(s)

If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE and transmitted in discovery message, the intermediate Relay UE does not transmit another RSC in discovery message. With this option, when the Remote UEs select relay path, the Remote UEs only considers the RSC for which the PC5 link is already established on red relay path.

Option 2: The intermediate Relay UE rejects the new relay establishment request for a different RSC

Once the intermediate relay UE established the red relay path, the intermediate Relay reject the request from the Remote UE to establish a new relay path. This option will consume discovery message transmission and Remote UE relay (re)selection procedure.

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

- Sends LS to SA2 if proposal 1 is agreed.

Another scenario is if the intermediate Relay UE has Uu connection with the different cell with the last Relay UE cell, e.g. in the following figure, the intermediate Relay UE established RRC connection with on green path for its own service towards same or different cell of the last relay UE, but the intermediate Relay UE receives relay request from the Remote UE to establish relay path, what the intermediate Relay UE should do.

Figure 2

Considering the existing mechanisms, there could be the following options to be considered

Option 1: The intermediate Relay UE performs path switch from direct path to indirect path.

Since the intermediate Relay UE is acting a Remote UE, when the intermediate Relay UE receives relay establishment request from Remote UE, the intermediate Relay UE performs path switch from cell2 to cell 1 using existing direct to indirect path switching.

Option 2: The UE does not act as an intermediate Relay UE if already established direct Uu connection

If the UE already established direct Uu connection, the UE does not transmit discovery message which includes other relay UE information. Then the UE will not act as an intermediate Relay UE.

Option 2 restricts the multi-hop relay applied scenario. Then it is proposed to adopt option 1.

- If a UE in CONNECTED state via direct path receives relay establishment request from the Remote UE or downstream intermediate Relay UE, the UE can perform direct path to indirect path switch to act as an intermediate Relay UE for the relay traffic and Remote UE for its own traffic

- gNB can configures the UE whether to perform direct path to indirect path switch.

# Conclusion

- If the intermediate Relay UE has established PC5 connection for one RSC associated with one last Relay UE, the intermediate Relay UE does not transmit other RSC(s) in any discovery message.

- Sends LS to SA2 if proposal 1 is agreed.

- If a UE in CONNECTED state via direct path receives relay establishment request from the Remote UE or downstream intermediate Relay UE, the UE can perform direct path to indirect path switch to act as an intermediate Relay UE for the relay traffic and Remote UE for its own traffic

- gNB can configures the UE whether to perform direct path to indirect path switch.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet