3GPP TSG-RAN WG2 #129	R2-2501262

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.1.2.2

Source:	Ericsson

Title:	LCM for UE-side models for beam management

Document for:	Discussion

# 1	Introduction

In this contribution we share our views on FFSs and further aspects on the following LCM topics for UE-sided models:

- Applicability reporting and inference configuration

- AI/ML functionality activation/deactivation

- Triggering of data collection for training and resource configuration for training.

# 2	Discussion

## 2.1	Applicability reporting and inference configuration

According to the RAN1 LS reply in [1], RAN1#119 reached the following agreement:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction [...]

Regarding Step 3, RAN1 distinguishes between two cases where different information is sent by the network:

- Case A: One or more of CSI-ReportConfig for full inference configuration; and

- Case B: One set or multiple sets of inference related parameters for applicability report only.

For case A, it is rather straightforward to implement the signalling and RAN1 already provided most of the solution by agreeing to use the CSI-ReportConfig to signal the inference configuration. In this case the UE can reply by simply including the applicability report in RRCReconfigurationComplete and indicating whether the received inference configurations are applicable.

Upon receiving one or more CSI-ReportConfig for inference configuration (case A), the UE responds by sending the applicability report via RRCReconfigurationComplete.

Furthermore, in our view the UE should behave the same way after receiving a full inference configuration, regardless of when the inference configuration is sent from the network to the UE (in Step 3 or Step 5). Namely, the UE should in any case check the applicability status when receiving an inference configuration, since it is still possible that an inference configuration sent in Step 5 is not applicable, e.g. due to changing UE conditions between transmission of Step 4 (transmission of initial applicability report), and reception of Step 5 (reception of full inference configuration). We note that sending an applicability report after Step 5 is easy from both specification and implementation perspectives, since the applicability report can simply be included in the RRCReconfigurationComplete message that is anyway sent by the UE in response to receiving the inference configuration, the same way as after Step 3.

The UE reports the applicability of an inference configuration (in CSI-ReportConfig) when it receives it, regardless of whether the inference configuration is sent by the network in Step 3 or Step 5.

For case B, the network does not configure the UE for inference, but instead sends set(s) of inference related parameters for applicability report. Given that RAN2 has already agreed that applicability reports can be sent also via UAI, it is natural for the network to send these set(s) of inference related parameters in otherConfig, so that the UE can reply by sending the applicability report as assistance information via UAI, to help the network decide how to configure the UE for inference.

The one or more set(s) of inference related parameters for applicability report (case B) are sent to the UE in otherConfig and the UE responds by sending the applicability report via UAI.

Furthermore, RAN2 agreed that a UE can also send updates in the applicability status of the functionalities, from applicable to non-applicable and vice versa. Such updates in the applicability status should also be sent via UAI, since the UAI framework already allows reporting UE preference updates, as for other legacy features.

Upon applicability changes (from applicable to non-applicable and vice versa), the UE can send the applicability report via UAI for both inference configurations and sets of inference related parameters.

Let us now discuss the following FFS from RAN2#128:

3.	FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

In our view, there are two possible solutions regarding this FFS:

- Solution 1: The UE always includes in the applicability report (initial and upon changes) the full list of applicable configured functionalities. In this case, it is not needed to report explicitly non-applicable functionalities, since all the configurations that are not included in the applicability report are implicitly non-applicable.

- Solution 2: Upon changes in the applicability, the UE includes in the applicability report only configured functionalities whose applicability status has changed from applicable to non-applicable or vice versa. In this case, the configurations need to be explicitly marked as non-applicable or applicable, to avoid ambiguities.

Both solutions above can work, where the purpose of Solution 2 is to reduce the content of signalled applicability reports compared to Solution 1, by including only configurations with changed applicability. However, it is questionable how much gain Solution 2 would bring in practice, given that we do not expect the network to configure the UE with many inference configurations simultaneously, so we expect that the list of applicable configurations is anyway not long. Furthermore, with Solution 2, if one applicability report with changes is missed by the network, there will be a mismatch between the network and UE, and the network cannot recover from this when it receives the next applicability report with changes. Thus, always sending a full list of applicable configurations (Solution 1) is more robust to errors than Solution 2, since the network does not need the previous report with changes to understand the full list of applicable/non-applicable configurations. Finally, it is easier to implement Solution 1 in the specifications, compared to Solution 2.

Upon changes in the applicability status (from applicable to non-applicable and vice versa), the UE includes the full list of applicable configurations in the applicability report. The non-applicable configurations are implicitly indicated by being omitted.

Finally, RAN2 should discuss if the UE should be allowed (upon configuration from the network) to send a preferred set of inference related parameters, other than what it may have received from the network in the form of full inference configurations (case A) or sets of inference related parameters (case B). In our view, there are situations in which this can be useful. For instance, the UE may decide that all the received inference configurations or sets of inference related parameters from the network are non-applicable. However, the UE may have stored from earlier training phases (e.g. earlier times when the UE was present in that cell) some parameter sets that would make an inference configuration applicable, so it would be relevant to report them to the network. Also, it could be that the gNB is directly interested in the UE’s preference and could thus ask the UE without sending inference configurations or sets of inference related parameters, if it does not have good configurations/parameters stored for that UE.

The UE can be allowed to send its own preference for a set of inference related parameters, other than the set of inference related parameters configured by the gNB (if any).

## 2.2	Functionality activation/deactivation

RAN2#127bis agreed that:

- UE can report to the network when an applicable AI functionality becomes non-applicable.  FFS how this is signaled (e.g. explicitly/implicitly).  Consider different scenarios, whether it is regarding an active functionality)

Further, RAN2#128 agreed that:

2. When a functionality becomes non-applicable the UE doesn’t autonomously deactivate. NW is expected to deactivate active functionality when it receives report from UE that it is non-applicable.

Given the two agreements above, it is critical for the network to be informed about functionalities/configurations that become non-applicable as soon as possible, so that the network can deactivate those functionalities/configurations. Thus, it should be mandated (and not optional) that the UE reports when an applicable functionality/configuration becomes non-applicable. This aspect was also discussed in RAN2#128 and it was suggested that we revise the agreement from RAN2#127bis as follows: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

## 2.3	UE-side Data Collection for Training

RAN2#128 agreed the following regarding UE-side data collection for training:

[…]

7.	For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

8.	The network can provide the data collection configuration (at any point in time), with or without UE request.

9.	The following methods for network control of the initiation and configuration for data collection:

- The network can decide when to start/stop the data collection and send configuration.

- The network can configure whether UE is allowed to initiate request for data collection.

10.	FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

Thus, according to agreement 9, the network can configure whether the UE is allowed to initiate a request for radio resources for data collection. The next step is to discuss how the network can configure the UE for this. In our view, the network can use RRCReconfiguration, i.e. OtherConfig, to configure the UE to send the request via UAI, since this request is in fact a UE’s preference to be configured with radio resources for data collection.

The gNB indicates to the UE whether the UE is allowed to request radio resources for data collection for UE-side model training via OtherConfig in RRCReconfiguration.

It is still FFS what the UE’s request for radio resources for data collection contains, but it is expected to contain parameters related to the inference configuration/the set of inference related parameters which are used for applicability reporting. Moreover, this request represents a preference of the UE, so overall it is natural that the UE sends the data collection request via UAI. For the content of this request, the UE can include in the request a set of preferred parameters for the data collection configuration, including associated ID(s), Set A, Set B, etc., based on a configuration from the network and on RAN1 discussions on which parameters are needed to decide the applicability of an inference configuration/set of inference related parameters.

Furthermore, it is useful if the UE can also report its preference to stop data collection, after it collects enough data. Thus, the network can be informed immediately if the dedicated radio resources for data collection are not needed by the UE anymore, so the network can de-configure them to avoid wasting those resources. These steps are shown in Figure 1. The stop preference from the UE can be signalled in a similar way as the request for data collection resources and since both represent UE preferences, it is natural to send them via UAI. Moreover, allowing the UE to send a stop preference is useful not only after the UE is finished with collecting data, but also for the case where the gNB configures the UE with resources for data collection without receiving a UE request (according to agreement 8 above). In this latter case, the UE can send the same stop preference signaling to indicate to the network that it does not intend to use those data collection resources.

If allowed, the UE can send via UAI a request for UE data collection (i.e. its preference to be configured with radio resources for UE-side training), or a preference to stop it.

The gNB can configure a set of parameters (e.g. Set A/B, associated ID, etc.) in which the UE can perform radio measurements for the purpose of UE-side data collection, and the UE can indicate its preferred parameters in the data collection request.

Figure 1 Signalling between gNB and UE, for UE-side data collection for training.

Another important aspect is that the radio resources for training need a configuration, which should coexist with the legacy radio measurement configuration (used for conventional “real time” measurement reports). For example, the gNB may need to provide a specific measurement configuration (CSI-RS/SSBs) for the UE to perform data collection for training and for the related reporting, including the Set A/B, and the associated NW-side additional conditions. Drawing inspiration from the RAN1 agreements on the inference and monitoring configurations, it seems natural to assume that the configuration of radio measurement resources for the UE-side data collection should be included in the CSI reporting configuration. In this way, all the three fundamental configurations (inference, monitoring, data collection) needed to enable LCM can be provided within the same signalling which simplifies the signalling and procedures implementation.

The configuration of the radio measurement resources (including Set A/B, associated IDs, etc.) in which the UE should perform radio measurements for the purpose of UE-side data collection are provide via the CSI report configuration (as the inference and monitoring configuration as per RAN1 agreements).

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Upon receiving one or more CSI-ReportConfig for inference configuration (case A), the UE responds by sending the applicability report via RRCReconfigurationComplete.

Proposal 2	The UE reports the applicability of an inference configuration (in CSI-ReportConfig) when it receives it, regardless of whether the inference configuration is sent by the network in Step 3 or Step 5.

Proposal 3	The one or more set(s) of inference related parameters for applicability report (case B) are sent to the UE in otherConfig and the UE responds by sending the applicability report via UAI.

Proposal 4	Upon applicability changes (from applicable to non-applicable and vice versa), the UE can send the applicability report via UAI for both inference configurations and sets of inference related parameters.

Proposal 5	Upon changes in the applicability status (from applicable to non-applicable and vice versa), the UE includes the full list of applicable configurations in the applicability report. The non-applicable configurations are implicitly indicated by being omitted.

Proposal 6	The UE can be allowed to send its own preference for a set of inference related parameters, other than the set of inference related parameters configured by the gNB (if any).

Proposal 7	Revise agreement 3 from RAN2#127bis as: “UE shall report to the network when an applicable AI functionality becomes non-applicable.”

Proposal 8	The gNB indicates to the UE whether the UE is allowed to request radio resources for data collection for UE-side model training via OtherConfig in RRCReconfiguration.

Proposal 9	If allowed, the UE can send via UAI a request for UE data collection (i.e. its preference to be configured with radio resources for UE-side training), or a preference to stop it.

Proposal 10	The gNB can configure a set of parameters (e.g. Set A/B, associated ID, etc.) in which the UE can perform radio measurements for the purpose of UE-side data collection, and the UE can indicate its preferred parameters in the data collection request.

Proposal 11	The configuration of the radio measurement resources (including Set A/B, associated IDs, etc.) in which the UE should perform radio measurements for the purpose of UE-side data collection are provide via the CSI report configuration (as the inference and monitoring configuration as per RAN1 agreements).

# References

R1-2410898 / R2-2500011, “Reply LS on applicable functionality reporting for beam management UE-sided model”, 3GPP TSG RAN WG1#119, Orlando, US, November 18th – 22nd, 2024.