3GPP TSG-RAN WG2 Meeting #129	                                R2-2501264

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.2

Source: 	TCL

Title: 	Ambient IoT device paging

Document for:	Discussion and decision

Introduction

- In the last RAN2 meeting, RAN2 discussed paging issue of Ambient IoT devices and came to some agreement as below[1]:

Agreements on paging

- An ID in paging message is beneficial to avoid duplicated response for same service request. FFS whether ID is generated by the reader or CN. FFS on the ID size.

- Capture the scenario that different readers may page the same device for same service request.  We will discuss this in WI phase (if in scope of WI) and consider progress from all the WGs

- Whether we include further information in paging message to indicate service type/command type or to indicate that further subsequent messages are coming can be discussed in WI phase once we understand more the paging design and AS ID.

In this contribution, we will further discuss the paging issue of ambient IoT devices and give some proposals on the top of the agreement of last meeting.

Discussion

Delta paging

It came to common understanding as RAN1 agreed that ALOHA mechanism will be used by Ambient IoT devices to initial the random access to the network. The ALOHA is a random selection based mechanism to select the access occasion, in result that a large portion of Ambient IoT devices can’t access the network successfully in the first round of random access. This requires the second round of random access, which the devices who have already accessed to the network successfully in the first round of random access should not access to the network again. Thus delta paging in Ambient IoT is necessary.

Obseravation 1: It is possible that the reader cannot reach all A-IoT devices in a coverage area in one paging round.

Proposal 1: delta paging should be introduced to avoid duplicated access procedure in the same transaction of paging.

On the other hand, RAN2 should introduce a mechanism to avoid duplicated random access in the same batch of paging. As paging likely message should allocate the resources and occasion for random access, the the devices who have already accessed to the network successfully in the first round of random access should know that the next round of paging is triggered for the same transaction of paging.

In order to implement delta paging, the paging message should add a transaction ID, thus all the rounds of paging message should use the same transaction ID, with the same transaction ID in the paging message compared to the previous paging message, the devices who have already accessed to the network successfully in the previous round of random access will not access to the network again.

Proposal 4: introduce a transaction ID in the paging message to identify the same request from CN paging, the devices who have already accessed to the network successfully in the previous round of random access should not access to the network again, if the transaction ID is the same compared to the previous paging message after which it has already accessed to the network.

Paging area and reader ID

In the last meeting discussion, RAN2 assume that the device will not support tracking/RAN area updated procedure. But at least that the paging area should be introduced. Especially in topology 2, as illustrated in the picture below, paging area 1 may be covered by two UEs, thus both of these two UEs should covered the same paging area, in order to provide full coverage, as one UE may have limited power to cover the whole area.

Observation 2: paging area is beneficial for providing full paging coverage in topology 2.

Proposal 5: it is proposed to introduce paging area for paging optimization.

In RAN2#126 meeting, it was agreed as the following:

AIoT paging message indicate information from which the device can determine resources to be used for response (D2R message).  FFS how (e.g. implicit/explicit/configured/preconfigured) and what resources (dedicated and/or shared) are provided to the device taking into account RAN1 discussion.

If a device is covered by multiple readers and it has already received the resources for random access from multiple readers. In this case, the device should know which reader allocated the resources, otherwise the other readers may be confused if the device uses the other reader’s resource for random access. For example, for slotted-Aloha access, where devices determine the slot for the uplink message transmission, an A-IoT device under coverage of multiple readers may get confused if subsequent paging messages are received. An identifier provided in the paging message can help an A-IoT device to determine when/how to transmit.

Proposal 6: RAN2 is proposed to introduce a reader ID in the paging message.

Conclusion

In this contribution, we discussed the Ambient IoT device paging issue. Then we have the following observations and proposals:

Observation 1: mask/filter based Ambient IoT grouping solution makes the Ambient IoT devices never be able to change the group.

Observation 2: paging area is beneficial for providing full paging coverage in topology 2.

Proposal 1: RAN2 should agree a more flexible Ambient IoT grouping mechanism.

Proposal 2: a Ambient IoT device should be able to be allocated a group ID by the network any time required.

Proposal 3: delta paging should be introduced to avoid duplicated access procedure in the same transaction of paging.

Proposal 4: introduce a transaction ID in the paging message to identify the same transaction of paging, the devices who have already accessed to the network successfully in the previous round of random access should not access to the network again, if the transaction ID is the same compared to the previous paging message after which it has already accessed to the network.

Proposal 5: it is proposed to introduce paging area for paging optimization.

Proposal 6: RAN2 should agree the UE reader should support the paging area/tracking area update procedure in order to update the paging area/tracker area that the UE reader provides.

Reference

RAN2#125bis chairman notes