3GPP TSG-RAN WG2 Meeting #129                                    	R2- 2501265

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.9.3

Source:	German Aerospace Center (DLR), European Space Agency (ESA), Toyota ITC, Inmarsat, Viasat, Thales, Aalyria Technologies, Echostar, Eutelsat Group, Sateliot, Novamint

Title:	Implicit pointer for locating CB-Msg3 DSA replicas

Document for:	Discussion and Decision

- Introduction

RAN WID [1] has the following objective for the EDT enhancements.

Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signalling to complete an Early Data Transmission (EDT) transaction [RAN2]:

- Msg3 transmission without msg1/ Random Access Response (RAR)

In RAN2#127-bis, it was agreed that

RAN2 will introduce support for DSA (i.e. the possibility to transmit more than one replica of CB-msg3, if
configured by the NW). RAN2 does not intend to work on CRDSA in Rel-19

In RAN2#128, it was agreed that the use of an implicit pointer for DSA replicas can be further discussed.

In this document, we provide additional discussion on the procedures that allow a UE to decide on the placement of the replicas of a CB-msg3 within the available resources, and the use of an implicit pointer that – without cost in terms of overhead – allows the network to infer the positions of the replicas upon decoding one of them in order to avoid unnecessary usage of downlink resources.

- Discussion

Following the decision to support diversity slotted ALOHA (DSA) [2-9] for the transmission of CB-Msg3 for NB-IoT via NTN taken in RAN2#127-bis, the definition of several protocol implementation aspects is still ongoing. The fundamental operation of the scheme is though clear, and foresees a UE to transmit up to 4 replicas of its CB-Msg3 during a configurable time window. The position of such replicas over the set of available uplink PRACH resources shall be selected randomly, as agreed during RAN2#128.

In this perspective, the specification of a mechanism that UEs shall follow to perform the random replicas placement can be convenient. On the one hand, this can enable a testable behavior of different UEs. On the other hand, it can ensure a proper and efficient operation of the DSA protocol, which is obtained by having a uniform placement of CB-Msg3 copies over the pool of resources.

A simple mechanism to achieve this can be summarized as follows, whereas a more detailed example is provided in Sec. 2.1:

- The UE applies a hash function to (a portion of) the CB-msg3 it wants to send via DSA, and obtains an unsigned integer;

- The obtained value is used as seed for a pseudo-random number generator (RNG);

- The RNG outputs a list of ordered positions, indicating the time-frequency resource that the UE will use to send its CB-Msg3 replicas.

Observation 1: The specification of a mechanism that UEs shall follow to place DSA CB-Msg3 replicas in time and frequency over the set of available PRACH resources can ensure a testable behavior and a proper operation of the DSA protocol with a uniform placement of CB-Msg3 copies over available resources.

It was also initially observed in [9] that making the eNB aware of where the CB-Msg3 replicas sent by a UE are placed can help improve resource efficiency, for instance by avoiding transmission of unnecessary CB-Msg4 on the PDCCH. We discuss more details on this in Sec. 2.2.

Observation 2: Making the eNB aware of the time-frequency placement of all CB-Msg3 replicas sent by UE as soon as one of them has been decoded can be beneficial at the network side, e.g., by avoiding transmission of unnecessary multiple CB-Msg4 over the PDCCH.

Combining these observations, three different solutions can be envisioned and compared:

Solution A: The UE generates the time-frequency positions of its CB-Msg3 replicas without following a predefined algorithm. Then, the UE transmits them.

Advantages:

- Solution A does not entail any uplink overhead or additional complexity and it is easy to standardize.

Disadvantages:

- The eNB has no way of inferring the position of other replicas after receiving any of them, without resorting to computationally complex operations, e.g., correlation over other time-frequency resources to identify other message copies. Consequently, the eNB may send up to 4 CB-Msg4 to confirm the correct reception of the transmitted uplink replicas which is undesirable from the overall resource consumption and UE point of view.

- The randomization is based on a local UE implementation-specific random number generator which may create inconsistent UE behavior.

Solution B: The UE generates the time-frequency positions of its CB-Msg3 replicas following a predefined algorithm, e.g., the one outlined in Sec. 2.1. The UE prepends/appends to each replica explicit pointers to the resources it will use, i.e., some bits are used to identify the time-frequency resources where the UE will send the replicas. The replicas are then transmitted.

Advantages:

- The eNB can infer the position of all the replicas sent by the UE upon decoding any of them by simply looking at the explicit pointers contained in the messages. Thus, the eNB could simply reply with a single CB-Msg4, regardless of the presence of multiple replicas.

Disadvantages:

- Solution B requires a modification of the CB-Msg3 format to convey replica positions

- It entails a cost in terms of uplink overhead and requires more standardization efforts than Solution A.

- Like Solution A, the randomization is based on a local UE implementation-specific random number generator which may create inconsistent UE behavior.

Solution C: The UE generates the time-frequency positions of its CB-Msg3 replicas following a predefined algorithm, e.g., the one outlined in Sec. 2.1. Then, the UE transmits the replicas.

Advantages:

- Solution C does not require any modification of the CB-Msg3 formats.

- It does not entail a cost in terms of uplink overhead, as opposed to Solution B.

- Like solution B, the eNB can infer the position of all the replicas sent by the UE. Indeed, upon decoding a CB-Msg3, the eNB can apply the same algorithm used by the UE to decide its replica placements, and infer the time-frequency resources used by the UE. This is possible since the RNG can be initialized with the same seed. We will refer to this mechanism to locate the replicas as implicit pointer [9].

- Thus, the eNB could simply reply with a single CB-Msg4, regardless of the presence of multiple replicas.

- The randomization is based on a standardized rule and hence the UE behavior is consistent and testable.

Disadvantages:

- Solution C requires more standardization efforts than Solution A, consisting in the specification of the algorithm employed by the UE to decide the replica placements, e.g., as outlined in Sec. 2.1

Following this discussion, Solution C appears to be the most promising, as i) it entails no additional complexity at the UE side; ii) it requires no modification to the CB-Msg3 structure; iii) it requires no additional uplink overhead; iv) it ensures a consistent behavior among UEs and a testable behavior by specifying a procedure to randomize replicas placements; and v) it can trigger positive effects at the receiver side, among which by providing a way for the eNB to transmit a single CB-Msg4, regardless the number of replicas transmitted by the same user.

Observation 3: Based on the analysis of advantages and disadvantages, Solution C (use of implicit pointers) is to be preferred.  Indeed, the use of a predefined mechanism to randomize replica placement, combined with the notion of implicit pointer i) does not require any modification to the CB-Msg3 message, ii) entails no costs in terms of uplink overhead, iii) ensures consistent behavior among UEs and a testable behavior; iv) can trigger positive effects at the network side by providing a way for the eNB to detect the positions of  all replicas of a UE’s CB-Msg3 and to transmit a single CB-Msg4, regardless the number of replicas transmitted by the same user.

Based on these considerations, the use of Solution C is proposed when supporting DSA for CB-Msg 3 transmission.

Proposal 1: RAN2 to specify a procedure to be followed by UEs to randomize placement of their replicas, enabling the use of an implicit pointer to infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

  - Example of algorithm for generating replica positions

An example of an algorithm that could be used for generating the replica positions is provided in the following. It is assumed for simplicity of discussion that the set of time-frequency resources that can be used by the UE to place its replicas are described as an ordered list of num_resources elements. The algorithm implements a linear congruential generator (LCG) with modulus chosen as a Mersenne prime and increment set to 0.

INPUTS:

num_replicas: 		integer, number of DSA Msg-3 replicas that the UE shall send

num_resources: 		integer, number of available time-frequency resources

seed: 			24-bit unsigned integer. this can be obtained as a 24-bit linear hash function

taking as input the content of the CB-Msg3.

OUTPUT:

positions: 		array of num_replicas integers, each between 1 and num_resources,				corresponding to a time-frequency occasion in the ordered list of EDT available

resource elements

ALGORITHM:

const m = 2^31 - 1           // modulus of the LCG
const A = 7^5                  // multiplier of the LCG
int x = seed                     	// initialize the LCG with the seed
positions = []             	// initialize an empty int array for positions

FOR k = 1 TO num_replicas DO

REPEAT
          x = (A * x) MOD m                     		// generate the next pseudo-random number
          position = (x MOD num_resources) + 1  	// map to [1, num_resources]
      UNTIL position NOT IN positions
   positions.APPEND(position)                		// add the valid position to the array

END FOR

RETURN sorted positions

The outlined algorithm allows the UE to generate pseudo-random positions for all the replicas that have to be sent. In this respect, it is noted that the algorithm also ensures that unique positions are generated for all replicas that have to be sent by the UE. Moreover, it is observed that if one or more UEs contending over the same set of resources pick the same seed (e.g., hash function collision), they will transmit their replicas in the same positions and collide. This event is in general not avoidable when resorting to pseudo-random number generations. Should this happen, and should the eNB not be able to decode any of the messages, the collision can be avoided at the next attempt by the presence of a random backoff applied independently by the UEs.

The seed value can be derived from CB-Msg3 using unique information about the UE. One solution is to use transport blocks at the MAC layer, which convey the transfer syntax of the RRC message encoding. Below is an example breakdown of rrEarlyDataRequest-NB-r15, Rel-15 branch,

-- ASN1START

UL-CCCH-Message-NB ::= SEQUENCE {

message					UL-CCCH-MessageType-NB

}

UL-CCCH-MessageType-NB ::= CHOICE {

c1						CHOICE {

rrcConnectionReestablishmentRequest-r13	RRCConnectionReestablishmentRequest-NB,

rrcConnectionRequest-r13				RRCConnectionRequest-NB,

rrcConnectionResumeRequest-r13			RRCConnectionResumeRequest-NB,

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-NB-r15

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

-- ASN1START

RRCEarlyDataRequest-NB-r15 ::=		SEQUENCE {

criticalExtensions					CHOICE {

rrcEarlyDataRequest-r15				RRCEarlyDataRequest-NB-r15-IEs,

later								CHOICE {

rrcEarlyDataRequest-r16				RRCEarlyDataRequest-5GC-NB-r16-IEs,

criticalExtensionsFuture			SEQUENCE {}

}

}

}

RRCEarlyDataRequest-NB-r15-IEs ::=	SEQUENCE {

s-TMSI-r15							S-TMSI,

establishmentCause-r15				ENUMERATED {mo-Data, mo-ExceptionData, delayTolerantAccess, mt-Access-v1610},

cqi-NPDCCH-r15						CQI-NPDCCH-NB-r14						OPTIONAL,

dedicatedInfoNAS-r15				DedicatedInfoNAS,

nonCriticalExtension				RRCEarlyDataRequest-NB-v1590-IEs		OPTIONAL

}

RRCEarlyDataRequest-NB-v1590-IEs ::=	SEQUENCE {

lateNonCriticalExtension				OCTET STRING				OPTIONAL,

nonCriticalExtension					SEQUENCE {}					OPTIONAL

}

RRCEarlyDataRequest-5GC-NB-r16-IEs ::=	SEQUENCE {

ng-5G-S-TMSI-r16					NG-5G-S-TMSI-r15,

establishmentCause-r16				ENUMERATED {mo-Data, mo-ExceptionData, mt-Access, spare1},

cqi-NPDCCH-r16						CQI-NPDCCH-NB-r14			OPTIONAL,

dedicatedInfoNAS-r16				DedicatedInfoNAS,

lateNonCriticalExtension			OCTET STRING				OPTIONAL,

nonCriticalExtension				SEQUENCE {}					OPTIONAL

}

-- ASN1STOP

-- ASN1START

S-TMSI ::=							SEQUENCE {

mmec								MMEC,

m-TMSI								BIT STRING (SIZE (32))

}

-- ASN1STOP

-- ASN1START

MMEC ::=							BIT STRING (SIZE (8))

-- ASN1STOP

The transfer syntax, which includes encoded octets of mandatory fields, is represented with octet counts and bit positions as follows:

| Octet/Bit | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Dispatching of the message branches |  |  |  |  |  | Optionality bitmap |  |
| 2 | mmec |  |  |  |  |  | m-TMSI |  |
| 3 | m-TMSI |  |  |  |  |  |  |  |
| 4 | m-MTSI |  |  |  |  |  |  |  |
| 5 | m-TMSI |  |  |  |  |  |  |  |
| 6 | m-TMSI |  |  |  |  |  | establishmentCause |  |
| 7 | Length indicator of dedicatedInfoNAS octet string |  |  |  |  |  |  |  |
| 8 | dedicatedInfoNAS |  |  |  |  |  |  |  |
| 9 | dedicatedInfoNAS |  |  |  |  |  |  |  |
| 10 | dedicatedInfoNAS |  |  |  |  |  |  |  |

It should be noted that the example contains only mandatory present content. In case the optionally present cqi-NPDCCH-r15 is included, the length indicator is shifted four bits which should not be an issue.

A possible solution is to skip the beginning of the transport block, include at least parts of m-TMSI, include establishment cause value, and include some octets from the NAS container. It would be useful to skip the length indicator because in case the container is short the indicator is populated with many zeros. Likewise, it may be useful to skip the last octet of the message because it may contain padding bits. In this case, it suggests using, e.g., the fifth, sixth and eighth or more octets from the NAS container.

  - Benefits of using implicit replica pointers

The use of a predefined algorithm for replica placement as the one described in Sec. 2.1, enables the use of implicit replica pointers as foreseen by Solution C. Indeed, upon decoding any replica of a CB-Msg3 sent by a UE, the eNB may infer the positions of all the other replicas by running the same algorithm implemented at the UE. The algorithm will provide exactly the same outputs used by the UE, as the eNB can employ the same initialization parameters (namely, num_replicas which is configured by the network, and seed obtained by the payload which was just decoded). After running the algorithm, the eNB is provided with the list of positions used by the UE.

It shall be noted that the use of an implicit pointer can trigger some advantages at network level and contribute to overall resource efficiency [9].

First, the use of Solution C may be useful in handling reception procedures for the CB-Msg4 at the UE. When using DSA, one aspect which is still under discussion is how to handle potential multiple CB-Msg4. In fact, if the time-frequency position chosen by the UE to transmit a CB-Msg3 is used to determine via a unique mapping the downlink resource where the acknowledgement is sent, sending more replicas may require the UE to listen to multiple resources, possibly overlapping in time, and possibly on different frequencies, introducing increased complexity.

A possible solution to this would be to send only one CB-Msg4, for instance on the earliest time-frequency resource among the ones mapped by the replicas sent by the UE. This approach has multiple advantages: i) it reduces the downlink overhead; ii) it simplifies operation at the UE as only one window has to be monitored for the CB-Msg4; iii) it still relies on the existing RNTI-based mapping of downlink resources to send the CB-Msg4. The use of implicit pointers as presented in Solution C would ease the implementation of this approach, as discussed in the following example.

Example 1: a UE sends 3 replicas of its CB-Msg3, over an ordered list of possible time-frequency resources. When running the replica placement algorithm, it selects three positions, leading to the configuration shown in the top part of Fig. 1. In turn, the EDT occasions chosen by the UE to send its replicas are mapped to three time-frequency resources where it should to listen for a CB-Msg4, as illustrated in the lower part of Fig. 1.

Assume now that the eNB decodes the second replica sent by the UE (highlighted in blue).  If no implicit pointer is used, i.e., with Solution A, the eNB does not know that this is the second replica, nor does it know where the others will be placed. Thus, the eNB may send a CB-Msg4 over the time-frequency resource corresponding to the sent replica, and the UE would have to listen to all possible locations to make sure it receives it. possibly having to monitor at the same time different frequencies.

Instead, if Solution C is implemented, it can be agreed that only one CB-Msg4 for all the replicas sent by the UE is sent in the earliest mapped occasion (highlighted in red in Fig. 1). The UE thus already knows that it only has to check this resource. In turn, upon decoding this message, the eNB can run the same RNG used by the UE and with the same seed, becoming aware of where the other replicas are placed. This also allows the eNB to understand where to send the unique CB-Msg4 as expected by the UE.

Figure 1: Reference for example 1

The example shows that the use of implicit pointers can lead to PDCCH resource saving, transmitting only one CB-Msg4, and can allow the UE to listen to a single reception window, reducing complexity.

Finally, as noted in [9], the use of implicit pointers may also have other positive effects. Consider for instance the following example:

Example 2: DSA-2 is used, and the eNB has decoded one DSA message from UE1 and one from UE2. Using Solution C, the eNB is able to understand that both of them have also picked the same time-frequency resource, denoted by X, to send another replica. By knowing this, the eNB can already expect that a collision among at least UE1 and UE2 takes place over that resource. Thus, it could prioritize decoding over other EDT occasions, or smartly handle a retransmission request by correctly interpreting the decoding failure over X as due to collision of at least two already retrieved users and not to, e.g., poor channel conditions. For two users this applies especially to the case without OCC, but it can be relevant also with OCC if more than two users are involved in the collision set.

Observation 4: The use of an implicit pointer can trigger benefits at the network side, such as avoiding multiple transmissions of CB-Msg4, and providing, without overhead or complexity cost, a way for the UE to know which time-frequency resources to monitor for a sole CB-Msg4 even when multiple replicas are sent.

Observation 5: The specification for the use of implicit pointers can be done without expected impact of RAN1. In fact, the seed for the RNG can be obtained only using information available at MAC.

- Conclusion

Following observations and proposals are made:

Observation 1: The specification of a mechanism that UEs shall follow to place DSA CB-Msg3 replicas in time and frequency over the set of available PRACH resources can ensure a testable behavior and a proper operation of the DSA protocol with a uniform placement of CB-Msg3 copies over available resources.

Observation 2: Making the eNB aware of the time-frequency placement of all CB-Msg3 replicas sent by UE as soon as one of them has been decoded can be beneficial at the network side by avoiding transmission of unnecessary multiple CB-Msg4 over the PDCCH.

Observation 3: Based on the analysis of advantages and disadvantages, Solution C (use of implicit pointers) is to be preferred.  Indeed, the use of a predefined mechanism to randomize replica placement, combined with the notion of implicit pointer i) does not require any modification to the CB-Msg3 message, ii) entails no costs in terms of uplink overhead, iii) ensures consistent behavior among UEs and a testable behavior; iv) can trigger positive effects at the network side by providing a way for the eNB to detect the positions of  all replicas of a UE’s CB-Msg3 and to transmit a single CB-Msg4, regardless the number of replicas transmitted by the same user.

Observation 4: The use of an implicit pointer can trigger benefits at the network side, such as avoiding multiple transmissions of CB-Msg4, and providing, without overhead or complexity cost, a way for the UE to know which time-frequency resources to monitor for a sole CB-Msg4 even when multiple replicas are sent.

Observation 5: The specification for the use of implicit pointers can be done without expected impact of RAN1. In fact, the seed for the RNG can be obtained only using information available at MAC.

Proposal 1: RAN2 to specify a procedure to be followed by UEs to randomize placement of their replicas, enabling the use of an implicit pointer to infer the positions of the DSA replicas sent by a UE over the available set of EDT occasions.

- References

[1] RP-240776, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, March 18-21, 2024.

[2] G. Choudhury, S. Rappaport, “Diversity ALOHA—A Random Access Scheme for Satellite Communications,” in IEEE Transactions on Communications, Vol. COM-31, No. 3, pp. 450-457, Mar. 1983.

[3] R2-2405202, “Discussion on EDT enhancements”, ESA, 3GPP TSG-RAN WG2 Meeting #126, Fukuoka, Japan, May 2024.

[4] R2-2406593, “Performance of Advanced Random Access Protocols,” DLR, 3GPP TSG-RAN WG2 Meeting #127, Maastricht, the Netherlands, Aug. 2024.

[5] R2-2408547, “Repetitions and Delay Considerations about SA and DSA,” ESA,  3GPP TSG-RAN WG2 Meeting#127-bis, Hefei, China, Oct. 2024.

[6] R2-2408863, “Discussion on throughput and delay performance of slotted ALOHA and diversity slotted ALOHA”, DLR, 3GPP TSG-RAN WG2 Meeting#127-bis, Hefei, China, Oct. 2024.

[7] R2-2410347, “Discussion on Uplink Capacity Enhancement”, Toyota, 3GPP TSG-RAN WG2 Meeting#128, Orlando, US, Nov. 2024.

[8] R2-2410875, “CB-Msg3 and Msg4 enhancements”, Qualcomm Inc., 3GPP TSG-RAN WG2 Meeting#128, Orlando, US, Nov. 2024.

[9] R2-2410875, “Implicit pointer for locating DSA replicas for EDT of CB-Msg3”, DLR, 3GPP TSG-RAN WG2 Meeting#128, Orlando, US, Nov. 2024.