3GPP TSG-RAN WG2 Meeting #129	R2-2501266

Hefei, China, Oct. 14th – 18th, 2024

Agenda item:		8.2.3

Source:		Samsung

Title:			Discussion on unified CBRA procedures for Ambient IoT

Document for:	Discussion & Decision

- 1 Introduction

The work item on Ambient IoT (A-IoT) has been approved in RAN#106 with the following objectives related to RAN2 [1]:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we discuss whether/how to design 2-step and 3-step CBRA for A-IoT.

- 2 Discussion

A-IoT random access procedure is always triggered by the reader including which device(s) need to respond with corresponding access occasion (e.g. time-frequency resource) respectively. Thus each triggered device determines how much data to be transmitted from its memory based on reader instruction. For instance, the device sends 16-bit random ID generated by itself to the reader if 3-step CBRA is triggered; while the device sends its 16-bit random ID together with upper layer data (e.g. device ID) to the reader if 2-step CBRA is triggered. From the perspective of device complexity, there is no much difference between 2-step CBRA and 3-step CBRA for Msg1 transmission.

Observation 1: For Msg1 transmission, there is no much difference between 2-step CBRA and 3-step CBRA from the perspective of device complexity.

Since the slotted-ALOHA is the baseline for A-IoT random access procedure, it is unavoidable that more than one device sends the same random ID at the same access occasion. One may argue that 2-step CBRA does not bring much gain than 3-step CBRA if such collision probability is not controlled efficiently. Based on the SA2 reply LS [2], it is possible for reader to manage collision problem based on the assistance informantion (e.g. whether the service is targeted for a single or multiple devices and the approximate number of target devices (if available) provided by CN).

Observation 2: It is possible for reader to trigger 2-step CBRA with sufficiently low collision probability based on assistance information provided by CN.

Based on above we think that there is no critical argument to support 3-step CBRA only.

Proposal 1: Suppport 2-step CBRA and 3-step CBRA in a unified design.

Proposal 2: A 16-bit random ID generated by device is included in Msg 1 for 2-step CBRA and 3-step CBRA. For 2-step CBRA, device ID and/or any other upper layer data is also included in Msg1.

Proposal 3: For 2-step CBRA and 3-step CBRA, the device considers the contention resolution as successful if the Msg2 including the same random ID in Msg1 is received.

In RAN2#127bis meeting, RAN2 agreed that it is up to reader whether to transmit Msg2.

- Agreements on new lean/counting scenario

- We will not capture a new use case in the TR.   RAN2 confirms that this use case can be done via reader implementation, as per previous agreement that it is up to the reader whether MSG2 is transmitted.  FFS if the device needs to be aware whether msg2 will be transmitted.

One remaining issue is wheher the device needs to be aware of optional Msg2 transmission. If the device knows that the reader skips Msg2 transmission, the device can also skip monitoring of Msg2 in a certain time period. Besides, both reader and some already inventorized devices do not need to perform A-IoT random access procedure for inventory service from the scratch if the triggering of A-IoT random access procedure is simply for lean/counting scenario.

Proposal 4: For 2-step CBRA and 3-step CBRA, the device knows whether Msg2 will be transmitted based on R2D message.

- 3	Conclusion

Based on above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Suppport 2-step CBRA and 3-step CBRA in a unified design.

Proposal 2: A 16-bit random ID generated by device is included in Msg 1 for 2-step CBRA and 3-step CBRA. For 2-step CBRA, device ID and/or any other upper layer data is also included in Msg1.

Proposal 3: For 2-step CBRA and 3-step CBRA, the device considers the contention resolution as successful if the Msg2 including the same random ID in Msg1 is received.

Proposal 4: For 2-step CBRA and 3-step CBRA, the device knows whether Msg2 will be transmitted based on R2D message.

- 4	Reference

[1] RP-243326	New Work Item: Solutions for Ambient IoT (Internet of Things) in NR

[2] S2-250141	Reply LS on Assistance Information from CN to Reader