3GPP TSG-RAN WG2 Meeting #129	R2-2501267

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.7.4.1

Source:	Apple, Lenovo, Xiaomi

Title:	Views on Delay-based Logical Channel Priority Adjustment

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

In previous meetings, RAN2 has made the following agreement for LCH priority adjustment [1]:

Introduce an independent per-LCH remaining time threshold for applying delay-critical priority.

We think there may be some complexity issues if the remaining time threshold is configured per LCH, considering that some DRBs have more than one LCHs (e.g. split bearer or duplication). Hence in this paper we will discuss some of the potential issues relating to this agreement.

# Discussions

## Issues Relating to Per-LCH Remaining Time Threshold

First of all, we would like to point out that UE complexity has always been a factor that needs to be taken into account from the beginning of this study. RAN2 has made the following agreement in RAN2 #126 [2] to confirm the importance of considering UE complexity in any solution to be developed for delay-aware LCP enhancement:

The solution should consider impact on UE complexity (as already indicated in SI objective description)

Observation 1: RAN2 has agreed that impacts to UE complexity should be considered for any delay-aware LCP enhancement.

With this in mind, we think it is important to re-consider whether the remaining time threshold for LCH priority adjustment is configured per LCH, or instead it should be configured per DRB. If the remaining time threshold is configured per LCH, we think the following issues may arise:

- PDCP discard timer is maintained by PDCP layer instead of MAC, and hence it is much easier for the PDCP to detect the remaining time-based triggering conditions. Apparently, this is more logical for the PDCP to track the remaining time status of each PDCP SDU. On the other hand, if it is configured per LCH, this means the MAC may need to have the knowledge of PDCP discard timer value and the exact arrival time of every PDCP SDU, in order to detect the triggering conditions by continuously tracking the remaining time status of every PDCP SDU. Alternatively, it requires the MAC entity to check persistently with PDCP about the remaining time status of each PDCP SDU. In either way, it increases the complexity of MAC entity operations significantly.

- In addition to the remaining time till PDCP discard timer, whether LCH priority should be adjusted may further depend on cross-packet dependency, such as PDU Set (especially when pdu-SetDiscarding is configured). For example, as long as there is a packet that belongs to a same PDU Set as at least one other packet that is considered urgent (i.e. the remaining time of which is smaller than a threshold), the priority of the LCH should be adjusted. However, MAC layer does not have the knowledge about how different PDCP SDUs are related in terms of PDU Set, as currently all PDU Set information is identified and utilized only by the PDCP layer. Thus, configuring remaining time threshold per LCH also brings additional complexity for the MAC entity to exploit PDU Set information that is only possessed by PDCP.

- In cases there are multiple LCHs associating to the PDCP entity in one radio bearer, such as Split Bearer and PDCP Duplication, it is questionable how the UE is going to deal with the situations where these LCHs are configured with different remaining time thresholds. In particular, with PDCP duplication where the PDCP PDU is submitted to multiple RLC entities (and hence LCHs), it becomes ambiguous about which remaining time threshold should be used for the LCH priority adjustment. Also, with split bearer, effectively every PDCP SDU would have a different remaining time threshold as it depends on which RLC entity this PDCP SDU is submitted to. Of course, we could further define rules to handle these situations, but this increases specification complexity unnecessarily. One could also argue that we can mandate the gNB to always configure the same remaining time threshold for all LCHs of the same radio bearer, but we do not see why we should have such constraints on gNB implementation flexibility.

- With Split Bearer, the packets of a PDU Set may be submitted to different RLC legs. When pdu-SetDiscarding is configured for the DRB, we think all packets of the same PDU Set should be considered as “LCH priority-adjusted data” if at least one packet has the remaining time smaller than the threshold. However, if the remaining time threshold is configured per LCH, and if all packets with remaining time smaller than the threshold are submitted to the primary RLC leg, while some other packets with remaining time larger than the threshold are submitted to the secondary RLC leg, it seems some interaction between different MAC entities is needed to enable priority adjustment for both LCHs. This may result in a lot of complications of UE implementation.  The problematic scenario is depicted in Figure 1, wherein different LCHs (in different MAC entities) need some coordination to make sure LCH priority can be adjusted for both of them for joint handling of the PDU Set, in cases the packets of the same PDU Set are buffered in either leg.

Figure 1 A problematic scenario with per-LCH remaining time threshold configuration in split bearer.

Observation 2: If the remaining time threshold for LCH priority adjustment is configured per LCH, there may be several UE complexity issues, including:

- Cross-layer checking (between MAC and PDCP) of remaining time status of each PDCP SDU,

- Cross-layer checking (between MAC and PDCP) of PDU Set identification and packet dependency,

- Handling of potentially different remaining time thresholds across multiple LCHs on one DRB, and

- Cross-MAC entity interaction for LCH priority adjustment in split bearer.

As mentioned earlier, we have agreed that the impacts to UE complexity must be considered, and therefore it is desirable to eliminate these potential complexity issues discussed above. In our views, all these complexity issues can be resolved simply if the remaining time threshold is configured per DRB rather than per LCH. That is, the PDCP can check if the conditions for LCH priority adjustment is satisfied (based on remaining time and PDU Set correlation), and then send an indication to MAC layer to trigger LCH priority adjustment if needed. There is no need for MAC layer to get involved in the decision process. Furthermore, it is also more straightforward for cases involving multiple LCHs in a DRB such as Split Bearer or Duplication scenarios, since PDCP can also send the indication to the lower layer to trigger priority adjustment for either LCH, and there is no need to coordinate the two LCHs.

As compared to per-DRB remaining time threshold configuration, we do not see any advantage or any reason of having per-LCH remaining time threshold. Instead, per-LCH only configuration only increases UE complexity unnecessarily. Thus, we think RAN2 should revisit the agreement and make the remaining time threshold per DRB rather than per LCH, as this is much more logical.

Proposal 1: RAN2 should change the agreement of “per-LCH remaining time threshold” to “per-DRB remaining time threshold” (e.g. configuration in pdcp-Config), as it would avoid a lot of complexity issues if the triggering condition of LCH priority adjustment is checked by PDCP rather than MAC.

Proposal 2: PDCP can detect the triggering conditions for LCH priority adjustment (based on PDCP discard timer remaining time and/or PDU Set information) and send the indication to the lower layer to enable LCH priority adjustment if needed.

# Conclusions

This paper provides our views on configurations of remaining time threshold for delay-based LCH priority adjustment. We have the following observations and proposals.

Observation 1: RAN2 has agreed that impacts to UE complexity should be considered for any delay-aware LCP enhancement.

Observation 2: If the remaining time threshold for LCH priority adjustment is configured per LCH, there may be several UE complexity issues, including:

- Cross-layer checking (between MAC and PDCP) of remaining time status of each PDCP SDU,

- Cross-layer checking (between MAC and PDCP) of PDU Set identification and packet dependency,

- Handling of potentially different remaining time thresholds across multiple LCHs on one DRB, and

- Cross-MAC entity interaction for LCH priority adjustment in split bearer.

Proposal 1: RAN2 should change the agreement of “per-LCH remaining time threshold” to “per-DRB remaining time threshold” (e.g. configuration in pdcp-Config), as it would avoid a lot of complexity issues if the triggering condition of LCH priority adjustment is checked by PDCP rather than MAC.

Proposal 2: PDCP can detect the triggering conditions for LCH priority adjustment (based on PDCP discard timer remaining time and/or PDU Set information) and send the indication to the lower layer to enable LCH priority adjustment if needed.

# References

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #126 Chair’s Notes, Fukuoka, Japan, May 2024.