3GPP TSG-RAN2 Meeting #129-RAN2#129	R2-2501269

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 5101 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on GNSS procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In RAN2#125bis, it was agreed that UE fails to conduct the GNSS position fix in an aperiodic GNSS measurement gap, the UE can still remain in RRC_CONNECTED until [X + GNSS validity duration] expires. Here is a possible case that the network trigger the GNSS measurement (shortly) before current GNSS validity duration expiry, and UE fails to conduct the GNSS position fix. We understand the above agreement can cover this case. And according to the agreement, in this case, UE can remain in RRC_CONNECTED state and await another network trigger before the expiry of current GNSS validity duration or even perform the autonomous GNSS measurement at expiry of the current GNSS validity duration. However, according to current description in section 5.5.9 in TS 36.331, in this case, if UE performs GNSS measurement, UE stop timer T390. And if later UE receives indication that GNSS measurement has failed, due to the stopped timer T390, UE will leave RRC_CONNECTED. So we think such description is not aligned with the above agreement. A correction way is to add the condition of stopping T390 after receving indication that a new GNSS position becomes valid, thereby it can ensure the condition of leaving RRC_CONNECTED isn’t met in this case. - In current TS 36.331, there are description of “GNSS validity duration expiry” in two places, one is in section 5.5.9 and another is in the 7.3.1 Timers Table for start of T390. However, RAN2 has agreement that there is no timer for GNSS validity duration in RRC and so there is no concept of GNSS validity duration expiry. Instead, RRC can only receive indication of out-of-date GNSS position from outside GNSS module. Therefore, it’s suggested to align “GNSS validity duration expiry” to “reception of indication of out-of-date GNSS position”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - To add a condition of stopping T390 in section 5.5.9, e.g., upon indication that a new GNSS position becomes valid, stop timer T390 - To align the “GNSS validity duration expire” with “receiving indication that the GNSS position has become out-of-date”. Impact Analysis Impacted functionality: GNSS procedure Inter-operability: No inter-operability issue can be seen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - If first change is not adopted, UE leaves RRC_CONNECTED upexpectedly in some cases. - If second change is not adopted, it may cause confusion on having a GNSS validity duration timer in RRC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.9, 5.3.3.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF #1 CHANGE

5.5.9	GNSS measurement triggering and reporting

For BL UEs or UEs in CE or NB-IoT UEs that are connected to NTN, GNSS measurement can be triggered aperiodically by the GNSS Measurement Command MAC CE (see TS 36.321 [6]), or triggered by the UE autonomously if enabled by the network, or triggered by the UE using available idle periods.

The UE shall:

1>	if an indication to perform GNSS measurement is received from lower layers:

2>	perform GNSS measurement using the measurement gap with a gap length indicated by lower layers, as specified in TS 36.213 [23];

2>	stop timer T390 upon indication that a new GNSS position becomes valid, if running;

1>	if gnss-AutonomousEnabled is configured:

2>	if the gap length is indicated by lower layers:

3>	set the autonomous gap length to the gap length indicated by lower layers;

2>	else:

3>	set the autonomous gap length to the latest reported time duration required for the UE to acquire a GNSS position;

2>	perform GNSS measurement using the autonomous gap starting from T390 expiry if ul-TransmissionExtensionEnabled is configured, otherwise starting from GNSS validity duration expiry;

NOTE:	UE can autonomously start GNSS measurements during available idle periods in RRC_CONNECTED to keep GNSS valid and stop T390 upon indication that a new GNSS position becomes valid. The exact time of starting GNSS measurements during available idle periods is left to UE implementation.

1>	upon starting GNSS measurement:

2>	stop timer T318, if running;

1>	upon indication that a new GNSS position becomes valid:

2>	instruct lower layers to report the remaining GNSS validity duration (see TS 36.321 [6]);

1>	upon indication that GNSS measurement has failed:

2>	if GNSS position is out-of-date; and

2>	if ul-TransmissionExtensionEnabled is not configured or T390 is not running:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

END OF #1 CHANGE

START OF #2 CHANGE

5.3.3.21	UE actions upon indication of out-of-date GNSS position

Upon indication that the GNSS position has become out-of-date while in RRC_CONNECTED, the UE considers GNSS validity duration expiry, and the UE shall:

1>	if the UE does not support performing GNSS position fix in RRC_CONNECTED and ul-TransmissionExtensionEnabled is not configured:

2>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other';

1>	else if ul-TransmissionExtensionEnabled is configured:

2>	if timeAlignmentTimer is configured to be infinity:

3>	start timer T390 with the timer value set to ul-TransmissionExtensionValue;

2>	else:

3>	start timer T390 with the timer value set to the remaining time of timeAlignmentTimer;

1>	else if ul-TransmissionExtensionEnabled is not configured and no indication of network triggered GNSS measurement is received from lower layers:

2>	if gnss-AutonomousEnabled is configured:

3>	perform GNSS measurement using autonomous gaps as specified in clause 5.5.9;

2>	else:

3>	perform the actions upon leaving RRC_CONNECTED as specified in 5.3.12, with release cause 'other'.

END OF #2 CHANGE