3GPP TSG-RAN WG2 Meeting #129	R2-2501272

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.8.6

Source:	Samsung

Title:	Re-direction from E-UTRAN TN to NR NTN

WID/SID:	LTE_TN_NR_NTN_mob-Core

Document for:	Discussion and Decision

# Introduction

In RAN#103 the LTE-part of the WID NR NTN Phase in Rel-19 was agreed [1]:

The work item aims at specifying support for the following objectives: - idle mode mobility based on cell reselection from E-UTRA TN to NR NTN, where E-UTRA TN provides satellite information for NR NTN neighbor cells in a System Information Block. [RAN2]

In this contribution, we provide our views on one of the last remaining issues on E-UTRAN to NR NTN mobility.

# Discussion

## Redirection

Re-direction is a key feature of 3GPP cellular networks that allows a network to release a UE to another frequency for the UE to perform idle mode cell selection on that frequency. It is often used to get a UE to connect to another RAT without the need for connected mode handovers, which is often challenging to implement.

For NTN, connected mode handovers from TN to NTN are supported both for NR NTN and eMTC since Rel-17, although the connected mode measurements does not have any requirements. However, connected mode handovers face difficulties as it relies on either Xn or NG-based handovers. This may be difficult and costly to implement. This means that at least for early mobility and integration between TN and NTN, re-direction is much less costly yet important type of mobility.

Observation 1: Re-direction is a key type of mobility, especially for mobility between RATs.

Observation 2: Re-direction to NTN is an important type of mobility for early TN-NTN networks.

The procedure for re-direction from NR TN to NR NTN as well as E-UTRAN to NR NTN in our view would be as follows for NTN as seen in Figure 1.

Figure 1. Intra-RAT and inter-RAT redirection to NR NTN.

1. eNB/gNB makes a decision to re-direct the UE to NTN. This can for instance be determined based on knowledge of satellite coverage, for instance in a rural network. The network may for instance based on measurement reports determine that both serving and neighbouring TN cells all offer poor coverage, and based upon this the eNB/gNB re-directs the UE.

2. eNB/gNB sends the RRCRelease/RRCConnectionRelease including the redirectionCarrierInfo along with some assistance information specific to NR.

3. The UE performs cell selection and attempts to find a suitable cell according to the redirectedCarrierInfo. During this process, the UE has to tune to find the NR NTN cells without any NTN assistance information, which involves scanning the band to detect large frequency offsets as seen in NTN.

4. Once the UE has successfully identified a cell and acquired all of the necessary info, the UE camps on the NR NTN cell.

From the above, since the step of performing cell selection on an NTN frequency shall already be supported from Rel-17, the re-direction from both E-UTRAN TN and NR TN to NR NTN may in principle supported, and this view was expressed in the last meeting [3]. However, some issues have come to light since then:

Issue 1 – Supported bands:

For inter-RAT mobility to NR, the UE indicates the bands that it support in field supportedBandListNR-SA-r15 in IRAT-ParametersNR, where the individual information element is FreqBandIndicatorNR whose elements currently has the following field description:

36.331:

[85] 3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone ". ... – FreqBandIndicatorNR The IE FreqBandIndicatorNR indicates the NR operating band as defined in TS 38.101 [85].

36.306:

4.3.34.3 supportedBandListNR-SA-r15 This field indicates whether UE supports standalone NR, as specified in TS 38.331 [35], and includes the supported NR bands as defined in TS 38.101-1 [33] and TS 38.101-2 [34]. The presence of this field also indicates that the UE can perform both NR SS-RSRP and SS-RSRQ measurement in the included NR band(s) as specified in TS 38.215 [36].

This does not include any reference to NR NTN bands, which are specified in 38.101-5. However, it should be noted that the frequency band indicator encoding can already be used to point towards bands in 38.101-5, as the information element in 38.331 is identical, but also includes 38.101-5 in the field description.

Observation 1: Current supported inter-RAT bands do not clearly include NR NTN bands in 38.101-5.

Issue 2 – Lack of band indication:

When a UE in non-terrestrial network receives an RRC Connection Release message to re-direct a UE, there is only an ARFCN (and NTN and TN have overlapping frequencies) provided, which does not indicate the band. This means that, the UE would have to assume that the band is an NTN band. Similarly, if a UE in a terrestrial network receives an RRC Release message to re-direct a UE, the UE would similarly have to assume that the band is a terrestrial band. If the UE does not know that the band is NTN and the UE only searches for TN cells, then it is not unlikely that UE will be unable to find any NTN cells.

Observation 2: Re-direction message only indicates frequency, not the band. This means that UE cannot always know whether to search for terrestrial cells or non-terrestrial cells.

Issue 3 – Lack of assistance information:

The re-direction is likely to be inefficient, as there is currently no support for providing any ephemeris information, which should be used by a UE to make the cell selection procedure more efficient. The current assistance information for instance includes the SCS of the SSBs as well as SMTC configuration. These are not fully necessary for cell selection but provides faster detection of the cells. Providing the satellite ephemeris would be an enhancement that is arguably more useful than these information elements.

Observation 3: Lack of ephemeris info would make the re-direction from TN to NTN very slow.

To allow for re-direction from a terrestrial E-UTRAN to an NR NTN frequency, we believe that in order to support it, the following is needed:

- Clarify in 36.306 and 36.331 that supportedBandListNR-SA includes NR NTN bands from 38.101-5.

- Re-direction via RRC Connection Release is enhanced to include either ephemeris, the satellite ID or a NTN band indicator

The support of RRC Connection Release with ephemeris or NTN/TN indicator is either indicated via a new capability in IRAT-Parameters, or indicating support of the NTN band in supportedBandListNR-SA.

Proposal 1: Re-direction from E-UTRAN to NR NTN is supported via the following:

- supportedBandListNR-SA is clarified to include NR NTN bands from 38.101-5.

- Re-direction via RRC Connection Release to include either ephemeris, satellite ID or if absent, NTN band indicator.

Proposal 2: Support of re-direction from E-UTRAN to NR NTN is indicated via either:

- New capability in IRAT-ParametersNR

- Indicating support of NR NTN bands.

# Conclusion

In this contribution we discussed issues related to E-UTRAN TN to NR NTN.

Observation 1: Current supported inter-RAT bands do not clearly include NR NTN bands in 38.101-5.

Observation 2: Re-direction message only indicates frequency, not the band. This means that UE cannot always know whether to search for terrestrial cells or non-terrestrial cells.

Observation 3: Lack of ephemeris info would make the re-direction from TN to NTN very slow.

Proposal 1: Re-direction from E-UTRAN to NR NTN is supported via the following:

- supportedBandListNR-SA is clarified to include NR NTN bands from 38.101-5.

- Re-direction via RRC Connection Release to include either ephemeris, satellite ID or if absent, NTN band indicator.

Proposal 2: Support of re-direction from E-UTRAN to NR NTN is indicated via either:

- New capability in IRAT-ParametersNR

- Indicating support of NR NTN bands.

# Reference

RP-240846, New WID: Inter RAT mobility support from E-UTRAN TN to NR-NTN, Thales, CATT, RAN#103, Maastrict, Netherlands, March 2024

R2-2410485, Further issues on E-UTRAN to NR NTN idle mode mobility, Samsung, RAN2#128, Orlando, US.