3GPP TSG-RAN WG2 Meeting #129	R2-2501274

Athens, Greece, 17th – 21st February, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	Further issues on contention-based Msg3

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#106 the latest WID for IoT NTN Phase 3 is provided [1]:

The aim of this WI is enhancement of IoT-NTN with the following objectives: - Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4] - Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2] Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI. Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution, we provide some discussions on the objective of Support of Capacity enhancements for the uplink.

# Discussion

- Agreements from RAN2#128 meeting:

Agreements:

1.	Only system Information is used to provide cell-specific CB-Msg3 PUSCH resources (FFS if anything is needed in dedicated signalling for the TA validation parameters, if needed, for the case of 15kHz SCS NB-IoT and eMTC CE mode B)

2.	Reuse the existing CE level selection procedure for CB-Msg3, at least for the initial selection. FFS whether we can reuse the same thresholds. FFS on the number of levels

3.	The UE triggers CB-Msg3 only if the size of pending UL data is less than the configured maximum TBS (FFS if the maximum TBS is same or different for different CE levels)

4.	There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC)

5.	The number of replicas for DSA will be configured by the NW: 1 (SA), 2, 3, 4. The configuration of the number of replicas is CE-level specific

6.	There will be a configurable time window for DSA CB-Msg3 occasion selection. FFS on the details (e.g. when the time window starts)

7.	For SA case (single replica), after the end of all repetition of CB-Msg3 PUSCH transmission, UE starts a window for response reception taking UE-eNB RTT into account. FFS if we need to consider additional delay e.g. for the processing time

8.	For DSA case, FFS if we only have one or multiple PDCCH monitoring window(s) (i.e. one window per each replica) for response reception. FFS when the window(s) is/are started (or restarted) and stopped. FFS on the window length. FFS if the UE needs to monitor only one RNTI or multiple RNTIs)

9.	The UE stops the PDCCH monitoring window(s) once it receives a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

10.	Assuming that there will be scenarios where it’s possible to receive a CB-msg4 before the UE transmits some replicas, a UE stops transmitting the remaining replicas if it has received a CB-msg4 containing a matching Contention Resolution Identity (FFS if there is no RRC message together with the CB-msg4)

11.	Within the configured time window, the UE shall select randomly different time domain occasions for transmitting different replicas. And for each time domain occasion, the UE shall select randomly a frequency domain resource.

12.	RAN2 understands that, for DSA, once the eNB successfully decodes one of the multiple replicas, it may respond without waiting for the remaining replica(s) (FFS when the response window(s) is/are started)

## RNTI issues and Msg4 monitoring

In RAN2#127bis the following was agreed:

7.	The RNTI used at least to schedule Msg4 transmission is derived based on the resource associated to the PUSCH occasion used for contention based Msg3 EDT transmission (FFS on the details. FFS how this is impacted by DSA)

In the case of selecting a single subframe to perform contention-based Msg3, deriving the RNTI based on the PUSCH occasion be done through a formula, similar to how the RA-RNTI for monitoring for RAR is calculated. Such formula may for instance include the subframe of the PUSCH occasion, frequency layer, if the frame can be TDMed etc.

Figure 2. A) selecting multiple PUSCH occasions, each with separate RNTI. B) selecting a single RNTI based on the first selected PUSCH occasion.

However, in DSA, the UE will randomly select a set of PUSCH occasions in the time window for DSA CB-Msg3 occasion selection. In this case, it needs to be carefully studied how the RNTI is determined. It should be a design decision that neither UE nor network should need to monitor multiple RNTIs.

Observation 1: UE or the network should not need to monitor multiple RNTIs during CB-Msg3 and Msg4 reception.

For instance, if it is decided that the UE selects a different RNTI for each selected PUSCH occasion in a DSA CB-Msg3 window, then the UE may need to monitor for multiple RNTIs for Msg4, as seen in Figure 1A.

If on the other hand the UE uses only a single RNTI, for instance based on the first selected PUSCH Occasion, or last selected PUSCH Occasion and the contention-based Msg3 resource window is a “sliding window”, the network may needs to monitor for all possible RNTIs in every PUSCH Occasion. This is because the network cannot know which PUSCH Occasion that the UE has selected, as it is randomly selected. This can be seen in Figure 2B.

Observation 2: For DSA, if RNTI is based on each selected PUSCH occasion, the UE may have to monitor for multiple RNTIs.

Observation 3: For DSA, if the CB-Msg3 resource window is sliding, the network may have monitor for all combinations of RNTIs.

Another issue that RAN2 may have to consider is that of RNTI collisions. Under bad design, RNTI collisions may occur if there are ambiguities in which random access resource that was used. This can for instance occur if the monitoring window is long or the resources being too tightly configured, as seen in Figure 3. The effect of RNTI collision is that the wrong UE may be addressed in response message.

Figure 3. Principle of RNTI collision (not necessarily representative of CB-Msg3).

Observation 4: RNTI collisions may occur unless DSA is carefully designed.

One way to resolve the potential issues of UE or eNB needing to monitor multiple RNTIs and RNTI-collisions, is that a single RNTI is used for each CB-Msg3 resource window and that the CB-Msg3 resource windows do not overlap.

Proposal 1: Single RNTI is used for all replicas in a CB-Msg3 resource window.

Proposal 2: CB-Msg3 resource windows does not overlap in time.

In simplified terms, for RACH-based access in eMTC and NB-IoT, the UE starts monitoring for Msg4 in the last subframe of the last repetitions of Msg3. For normal Msg3 and Msg4, this is sufficient to ensure that both UE and the network is synchronized on when a response may be transmitted.

In DSA however, and for Msg4 based enhancements this would be problematic. First of all, starting the response window according to the specific resource of the replica may cause longer response windows, leading to RNTI-collision issues. Secondly, in order to allow for Msg4 enhancements where multiple UEs are addressed in each message, then it would be useful if all UEs that use one CB-Msg3 resource window share the same Msg4 monitoring window.

Proposal 3: For DSA the UE starts the response timer after the end of the CB-Msg3 resource window plus UE-gNB RTT.

## Contention-based Msg3 transmit power

One issue that has not been discussed extensively is the transmit power that a UE performing contention-based Msg3 applies. For PRACH, the UE will ramp up the power. Such behaviour may have pros and cons for CB-Msg3 as PUSCH decoding will be different in comparison to PRACH decoding.

For instance, as indicated in the received LS from RAN1 [2], if UEs use OCC, then the received power cannot have a large difference:

The OCC solution that RAN1 is working on may be applied to CB-Msg3 NPUSCH format 1 single tone at least for OCC length 2, for the subcarrier spacing(s) for which CB-Msg3 is supported, when the power imbalance is small. The design details for the OCC solution are ongoing.

However, for non-OCC based CB-Msg3 it may be different. For instance, power ramping may allow for UEs that have attempted CB-Msg3 transmissions more times. Differences in received power may in some cases help in decoding if there are collisions.

Proposal 4: RAN2 to discuss CB-Msg3 transmit power when applying OCC and when not applying OCC.

## Fallback procedures

In some circumstances, depending on the configuration, it may be expected that contention-based Msg3 transmission would be less reliable compared to legacy random access. Failure rate can be dependent on the synchronization of the network and the UE, the radio characteristics and the load on the contention-based Msg3 resources. Given that the legacy random access should be more reliable and able to take a higher load, a UE should be able to fall back towards legacy random access. There are two ways in which a UE could fallback to random access with RACH; 1) the UE detects the failure and initiates the fallback and 2) the network detects a failure and signals to a UE to fallback to random access with RACH. We think that it would make sense to have both options available.

Proposal 5: If contention-based Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 6: RAN2 introduces UE-initiated Msg3 fallback and network-signalled Msg3 fallback.

If the contention-based Msg3 attempts fails, it may also be argued that the UE may be scheduled directly, similar to the case of NR 2-step random access via the fallbackRAR. However, we believe that this complicates the scheme and unlike 2-step random access where there is both a preamble and a PUSCH which enables the network to distinguish a UE even if the MsgA PUSCH fails, contention-based Msg3 only has a PUSCH transmissions. Therefore, we do not think that fallback to scheduled Msg3 PUSCH is needed for contention-based Msg3.

Proposal 7: Fallback to scheduled Msg3 is not introduced for contention-based Msg3 procedures.

# Conclusion

In this contribution we discussion initial issues on uplink capacity enhancements:

Observation 1: UE or the network should not need to monitor multiple RNTIs during CB-Msg3 and Msg4 reception.

Observation 2: For DSA, if RNTI is based on each selected PUSCH occasion, the UE may have to monitor for multiple RNTIs.

Observation 3: For DSA, if the CB-Msg3 resource window is sliding, the network may have monitor for all combinations of RNTIs.

Observation 4: RNTI collisions may occur unless DSA is carefully designed.

Proposal 1: Single RNTI is used for all replicas in a CB-Msg3 resource window.

Proposal 2: CB-Msg3 resource windows does not overlap in time.

Proposal 3: For DSA the UE starts the response timer after the end of the CB-Msg3 resource window plus UE-gNB RTT.

Proposal 4: RAN2 to discuss CB-Msg3 transmit power when applying OCC and when not applying OCC.

Proposal 5: If contention-based Msg3 fails, it should be possible to fallback to random access with RACH.

Proposal 6: RAN2 introduces UE-initiated Msg3 fallback and network-signalled Msg3 fallback.

Proposal 7: Fallback to scheduled Msg3 is not introduced for contention-based Msg3 procedures.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#106, Madrid, Spain, December 2024.

R2-2500010, Reply LS on OCC for CB-Msg3 NPUSCH, RAN1, RAN2#129, Athens, Greece, February 2025.