3GPP TSG-RAN2 Meeting #129-RAN2#129	R2-2501279

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0428 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on eDRX for UE in RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_redcap_enh-Core |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In MAC spec section 7.4, it is described as below that for RAN paging, when the RAN eDRX cycle is not configured or RAN eDRX is not allowed, UE in RRC_INACTIVE state does not operate in eDRX. - It means in this case the T termination would be same as the case when UE is not configured for eDRX by upper layers or eDRX-AllowedIdle is not configured, e.g., the below description in section 7.1 for that UE does not operate in eDRX: ## 7.4 Paging in extended DRX ………………………….... For CN paging, the UE operates in eDRX in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1; otherwise, the UE does not operate in eDRX. For RAN paging, the UE in RRC_INACTIVE state: - if the UE is configured for eDRX by ran-ExtendedPagingCycleConfig-r18 and eDRX-AllowedInactive-r18 is signalled in SIB1: - operates in eDRX with an eDRX cycle TeDRX, RAN configured by extendedPagingCycle-r18; - else if the UE is configured for eDRX by ran-ExtendedPagingCycle-r17 and eDRX-AllowedInactive-r17 is signalled in SIB1: - operates in eDRX with an eDRX cycle TeDRX, RAN configured by ran-ExtendedPagingCycle-r17; - else: - does not operate in eDRX. …………………………….. ## 7.1 Discontinuous Reception for paging ………………………….... If the UE does not operate in eDRX as defined in clause 7.4: - T is determined by the shortest of the UE specific DRX value configured by RRC (if any), the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. For L2 U2N Relay UE, T for a L2 U2N Remote UE is determined by the shortest of the UE specific DRX value provided in PC5-RRC signalling and a default DRX value broadcast in system information.. - However, this is not consistent with the below highlight yellow description in section 7.1 for T termination when TeDRX, RAN is NOT configured or used which is legacy R17 behaviour (refer to discussion in RAN2#115e and hereafter) and different from that UE does not operate in eDRX: ## 7.1 Discontinuous Reception for paging ……….. In RRC_INACTIVE state, if the UE operates in eDRX and eDRX is configured by RRC, i.e., TeDRX, RAN (if any), and upper layers, i.e., TeDRX, CN, as defined in clause 7.4: - If both TeDRX, CN and used TeDRX, RAN are no longer than 1024 radio frames: - T = min{TeDRX, RAN, TeDRX, CN}. - If TeDRX, CN is no longer than 1024 radio frames and no TeDRX, RAN is configured or used: - T is determined by the shortest of UE specific DRX value configured by RRC and TeDRX, CN. - If TeDRX, CN is longer than 1024 radio frames: - If TeDRX, RAN is not configured or used: - During CN configured PTW, T is determined by the shortest of the UE specific DRX value configured by RRC, the UE specific DRX value configured by upper layers (if any), and a default DRX value broadcast in system information. Outside the CN configured PTW, T is determined by the UE specific DRX value configured by RRC; - else if used TeDRX, RAN is no longer than 1024 radio frames: <skip> - else if used TeDRX, RAN is longer than 1024 radio frames: <skip>. - In a summary, the main issues are that, in the case that RRC eDRX is not configured while eDRX is configured by CN: - The T termination in either UE in RRC_INACTIVE or NW are unclear or incorrect due to the different description in section 7.1 and section 7.4. In other word, the mentioned description in section 7.4 is not completely aligned with the legacy R17 behaviour. - Moreover, if UE specific DRX value configured by upper layers is shorter than the UE specific DRX value configured by RRC, if NW follows the mentioned description in section 7.4 (e.g., assume UE doesn’t operate in eDRX) while UE follows the mentioned description in section 7.1, UE may miss RAN paging outside the PTW. Or if UE follows the mentioned description in section 7.4 while NW follows the mentioned description in section 7.1, it may cause unnecessary UE power consumption for unnecessary RAN paging monitoring. |
| Summary of change: |  | In section 7.4, to differentiate the UE behavior in RRC_INACTIVE state for the case that RRC eDRX is not configured while eDRX is configured by CN and make it correspond to that operating in eDRX without an eDRX cycle TeDRX, RAN configured. And then T termination for this case can follow the description in section 7.1. Impact Analysis Impacted functionality: Paging monitoring for UE in RRC_INACTIVE Inter-operability: - If UE implements this change but NW does not and NW follows the description in section 7.1, no issue; - If UE implements this change but NW does not and NW follows the description in section 7.4, the UE may miss the RAN paging in some cases; - If NW implements this change but UE does not and UE follows the description in section 7.1, no issue; If NW implements this change but UE does not and UE follows the description in section 7.4, it may cause unnecessary UE power consumption in some cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The T termination in RRC_INACTIVE state may be inconsistent between UE and NW due to the different description in section 7.1 and section 7.4 for the case that RRC eDRX is not configured while eDRX is configured by CN. This will cause either RAN paging missing or unnecessary UE power consumption. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of the change

7.4	Paging in extended DRX

The UE may be configured by upper layers and/or RRC with an extended DRX (eDRX) cycle TeDRX, CN and/or TeDRX, RAN.

For CN paging, the UE operates in eDRX in RRC_IDLE or RRC_INACTIVE states if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1; otherwise, the UE does not operate in eDRX.

For RAN paging, the UE in RRC_INACTIVE state:

-	if the UE is configured for eDRX by ran-ExtendedPagingCycleConfig-r18 and eDRX-AllowedInactive-r18 is signalled in SIB1:

-	operates in eDRX with an eDRX cycle TeDRX, RAN configured by extendedPagingCycle-r18;

-	else if the UE is configured for eDRX by ran-ExtendedPagingCycle-r17 and eDRX-AllowedInactive-r17 is signalled in SIB1:

-	operates in eDRX with an eDRX cycle TeDRX, RAN configured by ran-ExtendedPagingCycle-r17;

-	else if the UE is configured for eDRX by upper layers and eDRX-AllowedIdle is signalled in SIB1:

-	operates in eDRX without an eDRX cycle TeDRX, RAN configured;

-	else:

-	does not operate in eDRX.

If the UE operates in eDRX with an eDRX cycle no longer than 1024 radio frames, it monitors POs as defined in 7.1 with configured eDRX cycle. Otherwise, a UE operating in eDRX monitors POs as defined in 7.1 during a periodic Paging Time Window (PTW) configured for the UE. The PTW is UE-specific and is determined by a Paging Hyperframe (PH), a starting position within the PH (PTW_start) and an ending position (PTW_end). PH, PTW_start and PTW_end are given by the following formula:

The PH for CN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX, CN= (UE_ID_H mod TeDRX, CN), where

-	TeDRX, CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX, CN = 2, …, 1024 Hyper-frames) configured by upper layers.

The PH for RAN is the H-SFN satisfying the following equations:

H-SFN mod TeDRX_RAN= (UE_ID_H mod TeDRX_RAN), where

-	TeDRX_RAN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_RAN = 2, …, 1024 Hyper-frames) configured by RRC.

For CN configured PTW:

PTW_start denotes the first radio frame of the PH for CN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX, CN, where

-	ieDRX, CN = floor(UE_ID_H /TeDRX, CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by upper layers

For RAN configured PTW:

PTW_start denotes the first radio frame of the PH for RAN that is part of the PTW and has SFN satisfying the following equation:

SFN = 128 * ieDRX_CN, where

-	ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8

PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation:

SFN = (PTW_start + L*100 - 1) mod 1024, where

-	L = Paging Time Window (PTW) length (in seconds) configured by RRC

UE_ID_H is defined as follows:

UE_ID_H: 13 most significant bits of the Hashed ID.

Hashed ID is defined as follows:

Hashed_ID is Frame Check Sequence (FCS) for the bits b31, b30…, b0 of 5G-S-TMSI.

5G-S-TMSI = <b47, b46, …, b0> as defined in TS 23.003 [23].

The 32-bit FCS shall be the ones complement of the sum (modulo 2) of Y1 and Y2, where

-	Y1 is the remainder of xk (x31 + x30 + x29 + x28 + x27 + x26 + x25 + x24 + x23 + x22 + x21 + x20 + x19 + x18 + x17 + x16 + x15 + x14 + x13 + x12 + x11 + x10 + x9 + x8 + x7 + x6 + x5 + x4 + x3 + x2 + x1 + 1) divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where k is 32; and

-	Y2 is the remainder of Y3 divided (modulo 2) by the generator polynomial x32 + x26 + x23 + x22 + x16 + x12 + x11 + x10 + x8 + x7 + x5 + x4 + x2 + x + 1, where Y3 is the product of x32 by "b31, b30…, b0 of S-TMSI or 5G-S-TMSI", i.e., Y3 is the generator polynomial x32 (b31*x31 + b30*x30 + … + b0*1).

NOTE:	The Y1 is 0xC704DD7B for any 5G-S-TMSI value. An example of hashed ID calculation is in Annex A.

End of the change