3GPP TSG RAN WG2 #129

Athens, Greece, 17 – 21 Feb, 2025                                                     R2-2501281

Agenda Item:	8.8.3

Source:	Rakuten Mobile

Title:	RAN2 related proposals for Uplink Capacity & Throughput Enhancements

Document for:	Discussion

# Introduction

With the evolution of OCC-based uplink multiplexing in NR-NTN, RAN1 has reached agreements on various aspects of this technique, As per Rakuten Mobile’s understanding, its integration into RAN2 protocols requires enhancements in key areas such as capability signaling, scheduling, HARQ retransmissions, and control information handling to ensure seamless deployment in NTN-specific conditions.

Summary of RAN1 Agreements Relevant to RAN2, RAN1 has confirmed the following OCC techniques​

Agreement RAN1 to confirm the working assumption of RAN1#118bis with revisions as follows: - Support OCC length 2 with inter-slot OCC to multiplex up to 2 UEs. - Support Option 1: inter-slot OCC with OCC length 4 to multiplex up to 4 UEs using Hadamard sequences - RAN1 does not pursue Option 2: Intra-symbol pre-DFT OCC with OCC length 4 to multiplex up to 4 UEs. - RAN1 does not pursue Option 3: Combination of Inter-slot OCC with OCC length 2 and intra-symbol pre-DFT OCC with OCC length 2 to multiplex up to 4 UEs. Note 1: there will be separate UE capabilities for OCC length 2 and OCC length 4, where UE capability for OCC length 2 is a prerequisite for UE capability for OCC length 4. Note 2: gNB can ensure the performance of Option 1 by UE grouping with similar CFO (e.g. maximum differential CFO of 50 or 100 Hz or 200 Hz). Without CFO grouping (e.g. maximum differential CFO of 400 Hz), the performance of option 1 is degraded by at least 1 dB in several cases. For CFO grouping, several companies in RAN1 state that CFO grouping is feasible based on network implementation without any new specification impact. - RAN1 assumes no specification impact for CFO grouping - RAN1 does not pursue closed-loop frequency adjustment commands. - RAN1 assumes that RAN4 does not define new UE requirements for CFO.

While these agreements establish the feasibility of OCC-based multiplexing, RAN2 must address protocol-level implications, particularly in:

- UE capability signaling and OCC configuration updates for dynamic scheduling.

- Handling of UCI transmission when PUCCH and OCC-PUSCH overlap.

- HARQ and scheduling optimizations for OCC retransmissions.

- Inter-beam OCC scheduling in NTN scenarios.

- This contribution analyzes open RAN2 aspects and proposes enhancements aimed at ensuring efficient operation of OCC-based uplink multiplexing in NTN environments.

# Discussion

### 2.1 UE Capability Signaling for OCC-Based Uplink Multiplexing

The deployment of OCC-based uplink multiplexing requires UEs to indicate their support for OCC length 2 and OCC length 4. However, existing NR NTN specifications do not define explicit capability signaling for OCC, making it challenging for networks to dynamically allocate PRBs to OCC-capable UEs. Without proper signaling, networks may inefficiently allocate non-OCC resources to OCC-capable UEs, reducing system efficiency.

Furthermore, CFO grouping is a critical requirement for OCC performance in NTN. Since CFO effects are more pronounced in NTN due to Doppler shifts, a signaling mechanism is required to enable networks to group UEs based on CFO characteristics. Without such a mechanism, OCC multiplexing may fail to achieve its expected gains in NTN deployments​.

### Proposal 1

RAN2 to  consider defining new UE capability indicators in RRC signaling .

- Support for OCC length 2 and OCC length 4, allowing networks to allocate PRBs efficiently.

- A dynamic OCC configuration update procedure, allowing networks to modify OCC assignments based on real-time link conditions.

- Optional CFO-based grouping capability indicators, enabling more efficient UE clustering for OCC scheduling.

### 2.2 Handling of Uplink Control Information (UCI) in OCC-Based Transmission

UCI transmission handling is a critical issue in OCC-based multiplexing. Given that uplink PRB availability is limited in NTN, PUCCH and OCC-PUSCH may overlap, requiring a mechanism to determine whether UCI should be prioritized or multiplexed onto PUSCH.

RAN1 has explored multiple options for handling UCI-PUSCH conflicts, but no definitive approach has been adopted. The primary options under consideration include:

- Dropping UCI when it overlaps with OCC-PUSCH, which may lead to excessive retransmissions and HARQ inefficiencies.

- Prioritizing PUCCH transmission over OCC-PUSCH, which ensures reliable UCI transmission but reduces the spectral efficiency of OCC-based PUSCH.

- Multiplexing UCI onto OCC-PUSCH, allowing simultaneous control and data transmission but increasing scheduling complexity​.

Given that NTN uplink has limited retransmission opportunities due to long propagation delays, dropping UCI is not a viable solution. However, simple PUCCH prioritization also reduces efficiency, necessitating an optimized multiplexing approach.

- The handling of UCI in OCC-based uplink transmission remains undefined, leading to potential inefficiencies in scheduling and control signaling.

Proposal 2: UCI Multiplexing Mechanism for OCC-PUSCH

- Define UCI repetition across OCC transmissions for robustness.

- Specify a priority framework for HARQ-ACK, SR, and CSI at the MAC level

- Introduce RRC signaling to indicate UE support for UCI multiplexing on PUSCH.

Proposal 3: Hybrid UCI Multiplexing Scheme

- Define conditions for multiplexing CSI feedback onto PUSCH.

- Introduce MAC signaling to configure UCI handling behavior.

### 2.3 HARQ and Inter-Beam OCC Scheduling for NTN

The introduction of OCC-based multiplexing introduces complexities in scheduling and HARQ retransmission handling. A key concern is whether HARQ retransmissions should maintain the same OCC index assignment or whether adaptive OCC group reassignment should be permitted. Given that NTN link conditions fluctuate due to Doppler effects and beam switching, maintaining fixed OCC assignments for retransmissions may not always be optimal.

Additionally, there is no existing MAC scheduling mechanism that dynamically optimizes OCC group assignment based on real-time link conditions. This gap in standardization means that NTN deployments may struggle to achieve the full potential of OCC-based multiplexing unless RAN2 explicitly defines scheduling frameworks that incorporate dynamic OCC reassignment​.

- OCC-based uplink transmission lacks clear HARQ retransmission and MAC scheduling guidelines. Additionally, inter-beam OCC scheduling remains undefined, which may limit the spectral efficiency benefits of OCC multiplexing in NTN.

Proposal 4: Dynamic OCC Scheduling Framework

- Enable adaptive OCC group reassignment for HARQ retransmissions based on channel conditions.

- Define MAC scheduling policies to optimize OCC-based multiplexing for NTN uplink.

- Introduce MAC control signaling for real-time OCC adjustments based on traffic demands.

# Conclusion

In this contribution, we discussed about the aspects related to RAN2 for RAN1 agreed Uplink Capacity enhancements.

### Proposal 1

RAN2 to  consider defining new UE capability indicators in RRC signaling .

- Support for OCC length 2 and OCC length 4, allowing networks to allocate PRBs efficiently.

- A dynamic OCC configuration update procedure, allowing networks to modify OCC assignments based on real-time link conditions.

- Optional CFO-based grouping capability indicators, enabling more efficient UE clustering for OCC scheduling.

Proposal 2: UCI Multiplexing Mechanism for OCC-PUSCH

- Define UCI repetition across OCC transmissions for robustness.

- Specify a priority framework for HARQ-ACK, SR, and CSI at the MAC level

- Introduce RRC signaling to indicate UE support for UCI multiplexing on PUSCH.

Proposal 3: Hybrid UCI Multiplexing Scheme

- Define conditions for multiplexing CSI feedback onto PUSCH.

- Introduce MAC signaling to configure UCI handling behavior.

Proposal 4: Dynamic OCC Scheduling Framework

- Enable adaptive OCC group reassignment for HARQ retransmissions based on channel conditions.

- Define MAC scheduling policies to optimize OCC-based multiplexing for NTN uplink.

- Introduce MAC control signaling for real-time OCC adjustments based on traffic demands.

Proposal 4: Dynamic OCC Scheduling Framework

- Enable adaptive OCC group reassignment for HARQ retransmissions based on channel conditions.

- Define MAC scheduling policies to optimize OCC-based multiplexing for NTN uplink.

- Introduce MAC control signaling for real-time OCC adjustments based on traffic demands.

# References

RP-234078, RANP#102, “Non-Terrestrial Networks (NTN) for NR Phase 3”

RP-241659, RANP#104, “Non-Terrestrial Networks (NTN) for NR Phase 3”

3GPP RAN1#117 chairman notes