3GPP TSG-RAN WG2 Meeting #129

Athens, Greece, 17th - 21st February 2025

R2-2501282

Agenda Item:	8.7.4.2

Source:	Rakuten Mobile

Title:	Further discussion on Rel-19 DSR enhancements

Document for:	Discussion and Decision

# Introduction

In RAN2 some agreements of enhanced DSR had been reached as below.

As Rel-19 introduces enhanced DSR mechanisms, including multiple reporting thresholds per LCG, several technical gaps remain in ensuring efficient scheduling and optimal resource allocation. Previous agreements in RAN2#127bis and RAN2#128 have clarified aspects such as buffer segmentation, threshold configurations, and interworking with Rel-18 DSR, but certain issues require further refinements to:

- Optimize DSR for UEs transmitting both delay-critical and non-delay-critical traffic, ensuring that non-urgent data does not interfere with time-sensitive scheduling decisions.

- Introduce explicit DSR cancellation mechanisms, allowing both network and UE-triggered cancellation to reduce redundant signaling and improve scheduling efficiency.

- Ensure compatibility between Rel-18 and Rel-19 DSR implementations, minimizing disruptions when legacy and enhanced UEs coexist in the same scheduling environment.

- Improve the coordination between DSR triggering and retransmission mechanisms, ensuring that buffer status updates remain relevant and dynamic scheduling decisions are properly aligned.

This contribution provides a refined framework for further DSR enhancements, addressing the remaining technical gaps while ensuring that the proposed refinements align with recent agreements in RAN2.

# Discussion

## Optimizing DSR MAC CE Design for Mixed Delay-Critical and Non-Delay-Critical Traffic

The current DSR MAC CE format in Rel-18 is designed to report aggregate buffer size per LCG, but it lacks the ability to differentiate between delay-sensitive and non-delay-sensitive data. This leads to inefficiencies in scheduling, particularly in cases where:

- A UE has a mix of time-sensitive and background traffic in its buffer but cannot explicitly inform the network which portion requires immediate scheduling.

- The gNB assumes that all reported buffer data is urgent, leading to inefficient UL resource allocation that may delay truly urgent traffic from other UEs.

- Multiple UEs compete for limited uplink grants, and the scheduler prioritizes buffer size rather than true urgency, leading to potential misallocations of scheduling resources.

For example, in an XR application, a UE may have:

- Real-time motion-tracking data (latency-critical) that must be scheduled immediately.

- Background texture updates (non-delay-critical) that can be scheduled opportunistically.

However, since both traffic types are aggregated in the same DSR buffer size report, the scheduler may assume all data requires urgent scheduling, potentially delaying other UEs that have genuinely high-priority traffic.

Furthermore, with Rel-19 introducing multiple reporting thresholds per LCG, the MAC CE format must evolve to support more granular, yet efficient, delay reporting. The challenge is ensuring that the additional threshold information does not lead to excessive signaling overhead while still providing the scheduler with the necessary granularity for intelligent uplink resource allocation.

Several MAC CE designs have been proposed in RAN2, including:

- Bitmap-Based Reporting – A bitmap per LCG that indicates which reporting pairs are present. This approach provides structured reporting but can increase MAC CE size when multiple LCGs are reported.

- Extension Bit Per Pair – One additional bit per reporting pair, which reuses existing reserved bits in Rel-18 DSR. This ensures compact signaling but requires dynamic parsing at the gNB.

An additional concern is how the MAC CE should be parsed when multiple LCGs are reported simultaneously. If each LCG has multiple reporting thresholds, the MAC CE size may increase significantly, affecting control channel efficiency. A structured approach that dynamically scales the number of reported pairs based on network requirements is required.

A potential refinement to address both challenges—the lack of delay-critical differentiation and the MAC CE size inefficiency—is to introduce a dual-buffer structure in the MAC CE, allowing UEs to:

- Report delay-sensitive and non-delay-sensitive buffer sizes separately, ensuring that the gNB correctly prioritizes scheduling.

- Dynamically adjust reporting format based on available control signaling resources, ensuring that only the most critical buffer updates are transmitted when control overhead is constrained.

By combining these refinements, the DSR MAC CE format can be optimized to provide granular traffic reporting while keeping signaling overhead minimal.

- Current DSR MAC CE formats do not fully support scalable, priority-aware reporting, leading to potential inefficiencies when handling both delay-critical and non-delay-critical traffic.

#### Introduce a Delay-Critical Data Flag within the DSR MAC CE, allowing the UE to explicitly indicate which portion of the reported buffer requires immediate scheduling.

#### Define a dual-buffer reporting structure in the MAC CE, allowing UEs to report delay-sensitive and non-delay-sensitive buffer sizes separately, ensuring that scheduling prioritization remains efficient.

#### Implement a DSR Field Priority Indicator, allowing the UE to signal only the most urgent delay status updates, dynamically adjusting the reporting format based on available control signaling resources.

- Define whether the priority field should be reported per PDU Set or per LCG to avoid excessive MAC CE signaling overhead.

## Adaptive DSR Retransmission to Prevent Unnecessary Control Signaling

One of the persistent inefficiencies in current DSR retransmission logic is that UEs continue sending updated DSRs even when uplink grants are constrained, leading to unnecessary control signaling. This is particularly problematic in scenarios where:

- A gNB has already deprioritized a UE’s buffer report due to congestion but the UE keeps triggering DSR updates, consuming PUCCH resources that could be used for other control signaling.

- Multiple UEs experience scheduling delays, causing a build-up of redundant DSR retransmissions, ultimately leading to PUCCH congestion and inefficient resource utilization.

Currently, DSR retransmissions do not account for scheduling feedback from the gNB. Even if a gNB does not immediately schedule an uplink grant, the UE keeps retransmitting DSRs at regular intervals, assuming that the buffer status remains unchanged. Instead, a smarter retransmission strategy is required—one that allows UEs to suppress redundant reports if network feedback indicates resource constraints.

A possible solution is to introduce a Dynamic DSR Retransmission Suppression Mechanism, where:

- If the gNB does not schedule UL resources after receiving a DSR, the UE delays retransmission until a specified hold-off period has passed.

- The UE only re-triggers a DSR if the buffer status changes significantly, rather than relying on periodic retransmissions.

- If a DSR has been ignored multiple times, the UE should switch to an extended retransmission interval, preventing excessive PUCCH usage.

- Current DSR retransmission behavior leads to excessive control signaling, particularly in congestion scenarios where the gNB cannot schedule UL grants immediately.

- Introduce a congestion-aware DSR suppression threshold, ensuring that high-priority data can still be retransmitted even if general suppression rules are active.

- Implement an Extended Retransmission Interval Mode, where UEs progressively extend the time between retransmitted DSRs if scheduling opportunities remain unavailable.

## Configurable DSR Reporting for Different Traffic Types

Another aspect discussed in previous RAN2 meetings is whether DSR reporting behavior should vary based on traffic type, rather than applying a uniform reporting mechanism across all services. Some companies have raised concerns that treating all traffic the same may lead to inefficient scheduling, particularly for applications like XR and streaming services, which require continuous buffer updates, versus burst-based traffic that only requires occasional reporting.

For instance, an XR application with strict latency constraints may require higher reporting frequency, while a non-interactive background process may not need frequent DSR updates. However, current DSR procedures do not provide a way for networks to differentiate reporting rules based on service type, leading to potential inefficiencies in how different traffic classes compete for uplink scheduling.

One possible enhancement is to introduce Configurable DSR Reporting Classes, where:

- DSR reporting intervals and thresholds can be dynamically configured per traffic type, ensuring that different applications receive scheduling priority based on service requirements.

- The network can signal adaptive DSR configurations, allowing gNBs to adjust reporting rules dynamically for different QoS profiles.

- UEs can apply different DSR transmission logic for interactive vs. background data, ensuring more efficient uplink buffer management.

- Current DSR procedures do not differentiate reporting frequency or triggering logic based on traffic type, leading to potential inefficiencies in scheduling priority between interactive and non-interactive services.

- Introduce Configurable DSR Reporting Classes, where DSR reporting intervals and triggering rules are dynamically adjusted based on traffic type.

- 1 Allow network-controlled DSR adaptation, where gNBs can configure different DSR behavior based on QoS profiles, ensuring that interactive applications receive prioritized scheduling.

- Define gNB-configurable DSR reporting adaptation, where the network can adjust UE reporting behavior dynamically based on congestion levels and resource availability.

## Introducing Explicit DSR Cancellation Mechanisms

#### Background and Problem Statement

While DSR plays a crucial role in uplink scheduling efficiency, one of its key inefficiencies is that there is no mechanism for the gNB to cancel a previously received DSR report, even if the network determines that uplink resources cannot be allocated. In such cases, UEs may continue to attempt retransmitting their buffer status, leading to:

- Redundant control signaling, particularly when UL grants remain unavailable.

- Increased PUCCH congestion, as multiple UEs repeatedly report unschedulable DSR information.

- Potential scheduling inefficiencies, as repeated DSR retransmissions may interfere with more urgent control signals.

Rel-18 DSR already relies on triggering and cancellation based on predefined UE conditions, but in cases where a network-wide congestion event prevents scheduling, there is no way for the gNB to explicitly instruct UEs to stop sending further DSR updates.

Rel-19, with its multi-threshold reporting enhancements, increases the complexity as UEs may report more frequent and granular delay status updates, further exacerbating congestion-related inefficiencies when scheduling resources are unavailable.

An effective way to resolve this issue is to introduce a network-triggered DSR cancellation mechanism, allowing the gNB to signal UEs when their previously reported DSR information is no longer needed, thus:

- Eliminating unnecessary DSR retransmissions, reducing control signaling overhead.

- Ensuring that UEs do not repeatedly send buffer status updates when uplink scheduling constraints prevent resource allocation.

- Allowing UEs to dynamically adjust retransmission behavior, ensuring that new scheduling attempts are only made when necessary.

Additionally, UEs should be able to autonomously cancel a pending DSR report if:

- The UE buffer state changes significantly, rendering the previous DSR report outdated.

- Multiple scheduling attempts have failed, and the UE determines that re-reporting is inefficient.

This dual-triggered approach—allowing both network-initiated and UE-initiated DSR cancellations—ensures that Rel-19 DSR remains efficient and responsive to network scheduling conditions.

- Current DSR mechanisms do not provide an explicit cancellation method, leading to redundant reporting when uplink resources are unavailable.

- Introduce a DSR Cancellation MAC CE, allowing the gNB to signal UEs when previously reported buffer status information is no longer required, reducing unnecessary retransmissions.

- Introduce a DSR Cancellation Confirmation Mechanism, where the gNB confirms the cancellation to the UE, preventing unintended retransmissions of previously reported data.

- Define HARQ interaction rules, ensuring that a DSR is not canceled if related data is still in the HARQ retransmission process.

- Define a network-controlled DSR rejection mechanism, ensuring that UEs do not continue retransmitting reports that cannot be scheduled due to congestion.

- Allow UE-initiated DSR cancellations in cases where the application layer determines that certain delay-sensitive data is no longer relevant (e.g., expired XR frames).

# Conclusion

In this contribution, we provide our views on the enhanced DSR for XR use case, and the observation and proposals are summarized as follows:

Optimizing DSR MAC CE Design for Mixed Delay-Critical and Non-Delay-Critical Traffic

- Current DSR MAC CE formats do not fully support scalable, priority-aware reporting, leading to potential inefficiencies when handling both delay-critical and non-delay-critical traffic.

- Introduce a Delay-Critical Data Flag within the DSR MAC CE, allowing the UE to explicitly indicate which portion of the reported buffer requires immediate scheduling.

- Define a dual-buffer reporting structure in the MAC CE, allowing UEs to report delay-sensitive and non-delay-sensitive buffer sizes separately, ensuring that scheduling prioritization remains efficient.

- Implement a DSR Field Priority Indicator, allowing the UE to signal only the most urgent delay status updates, dynamically adjusting the reporting format based on available control signaling resources.

- Define whether the priority field should be reported per PDU Set or per LCG to avoid excessive MAC CE signaling overhead.

Adaptive DSR Retransmission to Prevent Unnecessary Control Signaling

- Current DSR retransmission behavior leads to excessive control signaling, particularly in congestion scenarios where the gNB cannot schedule UL grants immediately.

- Introduce a congestion-aware DSR suppression threshold, ensuring that high-priority data can still be retransmitted even if general suppression rules are active.

- Implement an Extended Retransmission Interval Mode, where UEs progressively extend the time between retransmitted DSRs if scheduling opportunities remain unavailable.

Configurable DSR Reporting for Different Traffic Types

- Current DSR procedures do not differentiate reporting frequency or triggering logic based on traffic type, leading to potential inefficiencies in scheduling priority between interactive and non-interactive services.

- Introduce Configurable DSR Reporting Classes, where DSR reporting intervals and triggering rules are dynamically adjusted based on traffic type.

- Allow network-controlled DSR adaptation, where gNBs can configure different DSR behavior based on QoS profiles, ensuring that interactive applications receive prioritized scheduling.

- Define gNB-configurable DSR reporting adaptation, where the network can adjust UE reporting behavior dynamically based on congestion levels and resource availability.

Introducing Explicit DSR Cancellation Mechanisms

- Current DSR mechanisms do not provide an explicit cancellation method, leading to redundant reporting when uplink resources are unavailable.

- Introduce a DSR Cancellation MAC CE, allowing the gNB to signal UEs when previously reported buffer status information is no longer required, reducing unnecessary retransmissions.

- Introduce a DSR Cancellation Confirmation Mechanism, where the gNB confirms the cancellation to the UE, preventing unintended retransmissions of previously reported data.

- Define HARQ interaction rules, ensuring that a DSR is not canceled if related data is still in the HARQ retransmission process.

- Define a network-controlled DSR rejection mechanism, ensuring that UEs do not continue retransmitting reports that cannot be scheduled due to congestion.

- Allow UE-initiated DSR cancellations in cases where the application layer determines that certain delay-sensitive data is no longer relevant (e.g., expired XR frames).

# References

- R2_128 R18 MBS, R18 QoE and R19 XR session notes (Dawid)_EOM.docx