3GPP TSG-RAN WG2 #129	R2-2501285

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.10.2.2

Source:	Ericsson

Title:	SON support for CHO with Candidate SCG

Document for:	Discussion/Decision

# Introduction

- MRO enhancement for R18 mobility mechanisms, including, Lower layer triggered mobility (LTM), CHO with candidate SCGs, subsequent CPAC [RAN3, RAN2]:

•	Specification of the inter-node information exchange, including possible enhancements to interfaces [RAN3]

•	Identify and specify necessary UE reporting to enhance the mobility parameter tuning [RAN2]

One of the main objectives of the SONMDT Rel-19 WID [RP-234038] is to enhance the MRO features to optimize the LTM cell switch procedure.

This contribution discusses considerations regarding SON enhancements for LTM and CHO with candidate SCGs.

# Discussion

## SON features for CHO with candidate SCGs

### Time information when execution conditions are not fulfilled

In RAN2#127-bis meeting, the following agreements were taken-

- UE reports the time gap between the first met condition (CHO or CPAC) and the second met condition (CPAC or CHO), and the first met execution condition (as agreed by RAN3), for a failed CHO with candidate SCGs. Details FFS.

- Include the elapsed time between the point in time of the first fulfilled condition and RLF in RLF report. Details FFS.

However, if none of the execution condition are fulfilled and UE declares RLF, it remains unclear what UE should log in the RLF report. In legacy, UE uses timeSinceCHO-Reconfig IE to log time information related to the reception of the CHO configuration and RLF.

2> if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure: 3> if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover; 3> else: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig; 3> set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResultNeighCells;

We first note that the timeSinceCHO-Reconfig will be logged also for the case of CHO with candidate SCGs because, as it is specified currently, this field is “set to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message”.

- No matter if the HO is a pure Rel.16 CHO or the Rel.18 CHO with candidate SCG, the UE will always log the timeSinceCHO-Reconfiguration as the time gap since receiving the condRRCReconfig, according to the existing specification.

Hence, to avoid ambiguity and confusion with the legacy timeSinceCHO-Reconfig, and erroneous interpretation at the gNB, it should be introduced a similar field just capturing the time gap between the CHO with candidate SCG and the RLF. Otherwise, the gNB would interpret this as the time gap since the last CHO configuration, whereas in fact this should be the gap between the last CHO with candidate SCG and RLF.

UE reports the time gap between the reception of the last configuration of the CHO with candidate SCG configuration and the RLF/HOF.

Furthermore, if the UE declares RLF after successfully performing CHO with candidate SCG, it will generate an RLF report. Since CHO with candidate SCG execution depends on fulfilling execution conditions for both CHO and CPC, a RLF afterwards requires analysis of both CHO and CPC conditions whereas legacy CHO would require optimization of CHO execution condition only. Thus, it is important for the network to understand from the RLF report that the last successfully executed mobility procedure was a CHO with candidate SCG. Hence, the IE for the type of the last executed mobility procedure, i.e. the lastHO-Type IE, should be extended with a new value indicating CHO with candidate SCG in the RLF report.

RAN2 agree to define a new value for the lastHO-Type IE in the RLF report for CHO with Candidate SCG.

### Impacts on SHR and SPR

In RAN2#128 meeting, the following were agreed:

1) RAN2 understands that current agreements is that the UE shall logs (in RLF report, SHR report and SCGFailureInformation) time from the last triggered event for the PCell (or PSCell) to the time to the last triggered event for the PSCell (or PCell). We don’t intend to do further or more granular enhancements. 2) Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

As can be seen from the agreements, although SHR is considered to log the agreed information, but SPR is left out of the agreements. In our view, there is no strong reason for omitting SPR. Hence, we propose to include all agreed information in SPR also.

RAN2 agrees to include the agreed information, i.e., measurement results of PCell and PSCells and timer information in the SPR.

Furthermore, since it is possible that SHR and SPR are both generated while UE executes CHO with candidate SCG configuration, if configured by the network. In our view, the SHR and SPR are correlated, and the network needs to consider this correlation while analyzing the reports.

RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.

# Conclusion

In the previous sections we made the following observations:

Observation 1	No matter if the HO is a pure Rel.16 CHO or the Rel.18 CHO with candidate SCG, the UE will always log the timeSinceCHO-Reconfiguration as the time gap since receiving the condRRCReconfig, according to the existing specification.

Based on the discussion in the previous sections we propose the following:

Proposal 1	UE reports the time gap between the reception of the last configuration of the CHO with candidate SCG configuration and the RLF/HOF.

Proposal 2	RAN2 agree to define a new value for the lastHO-Type IE in the RLF report for CHO with Candidate SCG.

Proposal 3	RAN2 agrees to include the agreed information, i.e., measurement results of PCell and PSCells and timer information in the SPR.

Proposal 4	RAN2 to discuss how to indicate correlation between SHR and SPR, if the SHR and SPR are generated because the UE performed simultaneous PCell handover and PSCell change due to CHO with candidate SCG.