3GPP TSG-RAN WG2 #129	R2-2501288

Athens, Greece, 17th – 21st February, 2025

Agenda Item:	8.1.5

Source:	Ericsson

Title:	On signalling feasibility of dataset and parameter sharing

Document for:	Discussion

# 1	Introduction

In R1-2410922 [1], RAN1 asked RAN2’s feedback on the feasibility of standardized signaling (over-the-air and/or other approaches) for NW-side sharing model parameters and/or dataset to the UE or UE-side for the following options:

- Dataset sharing consisting of {(Target CSI, CSI feedback)}

- Encoder parameter sharing

- Encoder parameter sharing + dataset sharing consisting of {target CSI}

In the context of two-side models, the dataset/parameter sharing across UE-side and NW-side turns out to be an essential feature to enable training for a two-sided model. This paper focuses on the feedback that RAN2 could provide to RAN1 related to this matter, taking into account in particular the overhead and other feasibility aspects related to the various mechanisms that can be envisaged for the dataset/parameter sharing.

# 2	Discussion

Regarding the procedure for over-the-air delivery of dataset/model parameters from the NW to the UE, we first consider the case in which the transfer would occur over the control plane (CP) using RRC signaling from the gNB to the UE. For this case, two scenarios can be considered: dataset/model parameter transferred via RRC without information partitioning, or with information partitioning before sending it in RRC.

In the case of transferring dataset/model parameters without splitting the information (i.e., a single RRC message), certain limitations arise. The dataset/model parameter size, as indicated by RAN1, exceeds the capacity of an RRC message. RAN1 in [1] indicates that the median dataset size may be 11.6MB in option 3a-1, and 225MB for option 4-1.

In the LS [1], RAN1 indicates to consider a dataset size between 11.6MB and 225MB for the dataset/parameter transfer.

The maximum allowable size for a dedicated RRC message conveyed in PDCP SDU is 9000 bytes, whereas for a SI message is 2976 bits. For an RRC message the DL RRC segmentation considers the support of up to five segments, whereas the maximum number of segments considered at least for some SIBs (e.g. SIB12, 17, 23) is 64. Hence, both in case the dataset is transferred via RRC dedicated signalling and via SIB signalling, the amount of data that can be conveyed given the current specification is very far away from the amount of data requested by RAN1 for the delivery of the dataset/model parameters.

The maximum RRC and SIB message size cannot carry the large amount of data expected for dataset/model parameter delivery over-the-air

Regardless of the size limitations, RRC is not designed to handle large packets, even if DL RRC segmentation is extended beyond 5 segments. Additionally, RRC procedures require that the NW needs to transmit all the  segments of an RRC message before initiating the transmission of a new one. Hence, all segments of an RRC message carrying dataset/model parameters must be delivered to the UE before initiating a new RRC transmission procedure. The large amount of segments can hence impact normal network services, potentially causing interruptions and posing a significant risk to overall network performance.

RRC protocol requires that all segments of an RRC message are transmitted before sending another RRC message. Therefore, a large dataset/model parameters would require many RRC segments which in turn will have an impact on other network services and may degrade the performance.

Similar concerns arise also for the transmission via SIB, which is also not designed to carry large amount of data. Additionally, the overhead issue becomes more significant in the case of broadcasting, as the network must continuously transmit the information to ensure it remains accessible to any UE. This occurs without prior knowledge of UEs' preferences, such as which partition is missing or whether the UE can store the data at the time of broadcast.

Dataset/model parameter sharing via SIB of large amount of data would cause significant overhead in broadcasting signalling,

Some of the limitations outlined above can be addressed by partitioning the data into multiple subsets before transfer over the RRC protocol or SIB. This solution would imply that different gNBs (of the same vendor) would be in charge of delivering different portions of the dataset. However, this solution also introduces significant specification impacts and complexity:

- Over-the-Air Identification: A mechanism is needed to uniquely identify a dataset/model parameter and its corresponding partition IDs across different vendors. Without this, it would be unclear how the UE could correctly merge partitions to reconstruct the data. How these IDs are assigned at various network nodes of different vendors should be addressed in 3GPP, as well as in a real-world deployment. Related to this issue, we would need mechanism to identify UEs that have already received certain partitions in order to reduce the overhead.

- Increased Complexity: Both the NW and UE would face additional complexity in managing which parts of the dataset/parameters should be transmitted through specific gNBs and delivered to specific UEs in the network.

- Specification Changes: Updates would be required in the specifications to enable UEs to request specific partitions of datasets/model parameters, or to at least know which are the partition(s) that a certain network node can provide, so that the UE does not need to acquire again partition(s) already acquired. Further, how to signal any updates related to a specific partition is something that should be considered in the spec.

- Data Redundancy: In addition to the large Uu signaling overhead in the DL, as outlined in the previous observations, there would be the further unnecessary redundancy in sending datasets/model parameters back in the UL, from the UE to the UE-side training entity. This latter step is obviously needed because the training cannot happen locally in the UE, and the data must eventually be routed from the UE device to the training entity (e.g. an over-the-top (OTT) server). If we consider that in order to generate the dataset/model parameters, the network may also need to collect data from the UE in the UL (e.g. based on NW-side data collection solutions that we will specify in Rel.19), then the overall burden over the air interface in generating the two-sided model will become non-negligible. 
While alternative means such as Wi-Fi could be used for sending data from the UE to the training server, this cannot always be guaranteed.

- Service Assurance: Additional mechanisms are required to ensure that the information delivery will not impact normal services running at the network.

- Scalability: In order to bring benefits from the overhead point of view, the partitioning solution would require that many gNBs/cells share the burden of sending the overall dataset. However, this requires that many gNBs/cells share the same NW-side dataset/parameters (i.e. the same NW-side model). Assuming that one cell can deliver one or few dataset portions in RRC segments (each segment of 9000 bytes), to average out the overhead per cell, this would imply that the network would still need to rely on dozens of cells to deliver the 11.6MB dataset (which is the lightest dataset size according to RAN1), and hundreds of cells for the 225MB case (worst case). The issue would be even worse if SIB is used. Whenever such massive amount of sharing is not possible, e.g. the NW-model is shared by few cells/gNBs due to deployment reasons, then the benefits of this approach cannot be any longer exploited.

Partitioning the data into multiple subsets, and delivering the different partitions via different gNBs/cells poses the concerns of:

  - Over the air identification of the partitions

  - Increase complexity at UE/NW in handling the sharing of the transmission and reception of the various partitions in various gNBs/cells

  - Specification complexity

  - Data Redundancy

  - Scalability

For the reasons mentioned above, we believe that it is worth evaluating the feasibility of alternative approaches not based on over-the-air signalling.

We first note that the case of dataset/model parameter transfer from the NW to the UE-side training entity exhibits notable similarities with the topic of data transfer for UE-side collected data studied by RAN2 during the Rel.19 WI. This is because both scenarios share the same AI/ML-specific data transfer path, which involves delivering data to the entity responsible for training the UE-side model. In our view, such similarity should be exploited in order to build a framework in which the procedures of UE-side data collection, and the procedures of dataset/model parameters transfer can naturally coexist without unnecessary complications and signalling overhead.

RAN2 introduced the term "first termination entity" to refer to the network entity that initially receives collected data from the UE. Across various options, there may also be a "final termination entity" for the collected data, such as an OTT server in charge of performing the UE-side model training. The primary distinction in the case of dataset/model parameters transfer is that the data originates from the gNB rather than from the UE.

Hence, it is logical to assume that the options that RAN2 defined in the CR to TR38.843 [2] for collecting UE-side data can be evaluated in the context of dataset/model parameter transfer from the NW to the training entity in charge of training a two-sided model (which can be the same entity also training a single UE-side model). Accordingly, the following options can be further explored for facilitating dataset/model parameter transfer from the NW to the training entity for the two-side model training:

1a.	NW vendor collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for two-sided model training. No 3GPP specification impact for the transfer of training data is expected.

1b.	gNB collects training data and transfers it to the server for data collection for two-sided model training (inside the MNO), and then optionally from the server for data collection for two-side model training to the OTT server (outside the MNO).

2.	gNB collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for two-side model training/or to the OTT server.

3.	gNB collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for two-side model training/OTT server.

The data collection entity for the two-side model training can be obviously the same as the UE-side data collection entity, since the two-sided training entity needs to combine the dataset received from the NW with the UE-side data.

In our view, the above alternative approaches, based on the options studied for the UE-side data collection, do not have any impact in the air interface and they are significantly more straightforward than over-the-air data delivery. Reusing the UE-side data collection options would also reduce the effort required within 3GPP.

Hence, RAN2 should conclude that standardized signalling over the air interface should be avoided, and alternative approaches should be explored. RAN2 concludes that standardized signalling over the air interface should be avoided.

RAN2 concludes that standardized signalling over the air interface should be avoided.

The options defined in the CR to TR38.843 [2] in the context of UE-side data collection can be reused as baseline approaches for the transferring of dataset/model parameters without impacting the over-the-air signalling:

  - gNB collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for two-sided model training.

  - gNB collects training data and transfers it to the server for data collection for two-sided model training (inside the MNO), and then optionally from the server for data collection for two-side model training to the OTT server (outside the MNO).

  - gNB collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for two-side model training/OTT server.

  - gNB collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

To this end, we propose RAN2 informing SA groups about the above alternative approaches and assess their feasibility for transferring dataset/model parameters from the NW to the UE.

RAN2 respectfully asks SA2/SA3/SA5’s feedback on the feasibility of non-over-the-air options outlined in Proposal 1 for the transfer of dataset/model parameters from the gNB to the UE-side for the purpose of two-side model training.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In the LS [1], RAN1 indicates to consider a dataset size between 11.6MB and 225MB for the dataset/parameter transfer.

Observation 2	The maximum RRC and SIB message size cannot carry the large amount of data expected for dataset/model parameter delivery over-the-air

Observation 3	RRC protocol requires that all segments of an RRC message are transmitted before sending another RRC message. Therefore, a large dataset/model parameters would require many RRC segments which in turn will have an impact on other network services and may degrade the performance.

Observation 4	Dataset/model parameter sharing via SIB of large amount of data would cause significant overhead in broadcasting signalling,

Observation 5	Partitioning the data into multiple subsets, and delivering the different partitions via different gNBs/cells poses the concerns of:

a.	Over the air identification of the partitions

b.	Increase complexity at UE/NW in handling the sharing of the transmission and reception of the various partitions in various gNBs/cells

c.	Specification complexity

d.	Data Redundancy

e.	Scalability

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 concludes that standardized signalling over the air interface should be avoided.

Proposal 2	The options defined in the CR to TR38.843 [2] in the context of UE-side data collection can be reused as baseline approaches for the transferring of dataset/model parameters without impacting the over-the-air signalling:

a.	gNB collects and directly transfers training data to the data collection entity outside the MNO (e.g. Over-The-Top (OTT) server) for two-sided model training.

b.	gNB collects training data and transfers it to the server for data collection for two-sided model training (inside the MNO), and then optionally from the server for data collection for two-side model training to the OTT server (outside the MNO).

c.	gNB collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for two-side model training/OTT server.

d.	gNB collects training data and transfers it to OAM. OAM transfers the training data to the server for data collection for UE-side model training/OTT server.

Proposal 3	RAN2 respectfully asks SA2/SA3/SA5’s feedback on the feasibility of non-over-the-air options outlined in Proposal 1 for the transfer of dataset/model parameters from the gNB to the UE-side for the purpose of two-side model training.

# References

R1-2410922, LS on signalling feasibility of dataset and parameter sharing,” RAN1#119.

R2-2407807, CR on RAN2 inputs to TR 38.843,” RAN2#127