3GPP TSG-RAN WG2 #129	R2-2501289

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.10.5

Source:	Ericsson

Title:	On Rel.18 leftovers

Document for:	Discussion

# 1	Introduction

Rel-19 WID supports the following leftover topics in RAN2.

- Support of the leftovers in Rel-18 SON/MDT [RAN3, RAN2]:

- MHI Enhancement for SCG Deactivation/Activation

- RACH optimization for SDT

In MHI Enhancements for SCG Deactivation/Activation, RAN2 asked RAN3 if a NW-based solution is sufficient to analyse the time UEs spend with the SCG activated vs. deactivated. In response, RAN3 replied they will specify a NW-based solution, but that a UE-based solution is beneficial in addition.

We therefore propose here that the UE shall log the accumulated time with SCG state set to activated during the stay in the PSCell.

Furthermore, we address some FFSs regarding RACH optimization for SDT.

# 2	Discussion

## 2.1	MHI Enhancement for SCG Deactivation/Activation

In Rel-16, Mobility History Information (MHI) was introduced, logging up to 16 PCells visited by the UE. In Rel-17, PSCell MHI was introduced, enabling the UE to log up to 16 visited PSCells, or time with no PSCell, across all reported PCells. In Rel-17, SCG activation and deactivation was also introduced, which enables power saving while still allowing fast access to the SCG when needed. In the Rel-19 SON/MDT Work Item [1], MHI enhancement for SCG deactivation/activation is listed as one of the objectives for the Work Item.

The logging of Mobility History Information is performed when the stay in the cell to be logged is ending, typically upon handover or cell change. The cell the UE is leaving is then logged together with information on the time spent in the cell. Similarly, the logging of PSCell Mobility History Information is done upon addition of a PSCell, release of a PSCell, or upon change of PCell (since the PSCell entries are stored in a substructure to the PCell entries). Just as the PCell history, the PSCell history includes the cell ID and the time spent in the cell, or the time spent with no PSCell. The logged information will, after reporting to the network, help the network to understand the movement of the UE and identifying ping-pong or coverage issues in the network.

From the current MHI framework, there is no possibility to see from the Mobility History Information whether the SCG state has been set to activated or deactivated during the time the UE spent in the PSCell.

SCG activation and deactivation was introduced in Rel-17.

There is no possibility to see from the Mobility History Information whether the SCG state was set to activated or deactivated.

With the introduction of SCG activation and deactivation, a PSCell addition or change could take place with the SCG state set to either activated or deactivated. Information indicating the SCG state during the stay in the PSCell could give additional valuable insights and enable optimization of the configuration of dual connectivity.

In RAN2#127, RAN2 agreed that it is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Two options for collecting the time with SCG state set to activated and deactivated was discussed, namely a network-based solution or a UE based solution. An LS [2] was written and sent to RAN3, asking RAN3 to analyse whether a network-based solution is sufficient enough for network optimization, or whether a UE based solution is needed. In RAN3#126, this was discussed, and a reply LS was agreed [3], stating that RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation, and also that RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

Based on this, to increase the understanding of the secondary cell usage, information on the accumulated time with SCG state set to activated for each PSCell entry could be included in the Mobility History Information. Since the time spent with the PSCell configured is already logged by the UE since Rel.17, this new information will give an indication of how large part of the time spent in the PSCell was spent in activated or deactivated state. We therefore propose the following:

For each PSCell entry of the MHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

## 2.2	RA report logging in case of SDT

During the last 3GPP meetings, the following was agreed for the topic of SDT:

From RAN2#128:

Agreements

- The UE logs the DL RSRP and UL data volume for the failed to initiate of RA-SDT in RA report.

- The following failure causes can be logged for failed SDT in RA report:

- upon receiving indication from the MCG RLC that the maximum number of retransmissions has been reached while SDT procedure is ongoing;

- upon random access problem indication is received from MCG MAC while SDT procedure is ongoing;

- upon T319a expires;

- upon the lower layers indicate that cg-SDT-TimeAlignmentTimer or the configuredGrantTimer expired before receiving network response for the UL CG-SDT transmission with CCCH message while SDT procedure is ongoing;

- FFS: unsuccessfully completed upon cell re-selection.

As captured above, in RAN2#128 meeting, there is an FFS on whether to include SDT failure due to cell re-selection. Adding this information in the RA report aids network to analyse the reason of SDT failure. However, cell reselection is triggered by the UE and not under control of the network; if the network is unaware of the SDT failure reason and optimizes SDT parameters such as data volume threshold and RSRP threshold, it would lead to further potential failures. Reporting such information enables the network to avoid unnecessary optimization of the SDT related parameters.

UE includes failure cause upon SDT being unsuccessfully completed on cell-reselection.

Furthermore, it was agreed to include a failure cause if the CG SDT fails due to expiry of cg-SDT-TimeAlignmentTimer or the configuredGrantTimer.

RAN2 has agreed to include CG-SDT related failure cause in the RA report.

However, there are other causes of CG-SDT related failures, e.g., not meeting the cg-SDT-RSRP-ThresholdSSB conditions. Including these other conditions aids the network to perform root-cause analysis of the SDT failure.

RAN2 to include other causes of CG-SDT evaluation failure prior to the random-access procedure in the RA report, i.e. conditions on cg-SDT-RSRP-ThresholdSSB not met.

# Conclusion

In the previous sections we made the following observations:

Observation 1	SCG activation and deactivation was introduced in Rel-17.

Observation 2	There is no possibility to see from the Mobility History Information whether the SCG state was set to activated or deactivated.

Observation 3	RAN2 has agreed to include CG-SDT related failure cause in the RA report.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For each PSCell entry of the MHI, log the accumulated time with SCG state set to activated during the stay in the PSCell.

Proposal 2	UE includes failure cause upon SDT being unsuccessfully completed on cell-reselection.

Proposal 3	RAN2 to include other causes of CG-SDT evaluation failure prior to the random-access procedure in the RA report, i.e. conditions on cg-SDT-RSRP-ThresholdSSB not met.

# References

RP-234038, New WID: Data collection for SON (Self-Organising Networks)/MDT (Minimization of Drive Tests) in NR standalone and MR-DC (Multi-Radio Dual Connectivity) Phase 4, CMCC, RAN #102, Dec. 11th – 15th, 2023.

R3-245009, LS on MHI enhancement solution for SCG deactivation/activation, RAN2, 3GPP TSG RAN WG2#127, Maastricht, Netherlands, 19th - 23rd August 2024.

R3-247906, Reply LS on MHI enhancement solution for SCG deactivation/activation, RAN2, 3GPP TSG RAN WG3#126, Orlando, US, 18 - 22 Nov, 2024.